3GPP TSG-SA WG2 Meeting #143-e	S2-2100586

# E-Meeting, 24th February – 09th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2612 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE On-boarding registration for NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the TR 23700-07, On-boarding registration and following remote provisioning for SNPN case should be supported in R17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support on-boarding registration and remote provisioning after successful on-boarding |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing new features as per conclusion of TR 23.700.07 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.X (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of changes *****

5.30.X	Onboarding and Remote Provisioning of UEs in Non-Public Networks

5.30.X.1	General

Editor’s Note: 	This clause should have an overview or general description of onboarding both SNPNs and PNI-NPNs. The applicability scenarios, and a general architecture. Where differences appear between SNPN and PNI-NPNs, the description should be moved to the following sections.

5.30.X.2	Onboarding and Remote Provisioning of UEs in SNPNs

5.30.X.2.4	Initial Connectivity

5.30.X.2.4.1	General

SNPN or PLMN can act as an on-boarding network to support On-boarding registration for UEs that need provisioning of NPN credentials (i.e. for primary authentication) and other information to enable SNPN access.

When the on-boarding network is an SNPN, in order to register to an SNPN for On-boarding, the UE sets the registration type as On-boarding Registration and provides an RRC indication to indicate that the RRC connection is for on-boarding. This RRC indication allows NG-RAN to select an appropriate AMF that supports on-boarding procedures. This is defined in clause 5.30.X.2.4.3.

When the on-boarding network is an PLMN, in order to register to an PLMN for On-boarding with PLMN credentials it follows the exsiting mechanism as decribed in 5.3.2 with conditions defined in clause 5.30.X.2.4.4.

5.30.X.2.4.3	Initial registration when ON is SNPN

In order to register to an SNPN for On-boarding, the UE sets the registration type as On-boarding Registration and provides an RRC indication to indicate that the RRC connection is for on-boarding. This RRC indication allows NG-RAN to select an appropriate AMF that supports on-boarding procedures. The On-boarding Registration type allows AMF to, e.g., select an appropriate SMF and perform other on-boarding-related configuration, e.g., start the timer configured for on-boarding service, restrict the UP connectively only for purpose of remote provisioning with allowed PS(s).

On-boarding SNPN should support functionality to restrict usage to only on-boarding service. The information required to restrict the usage to only on-boarding service is locally configured in the AMF and SMF, and the AMF and SMF restrict the usage when the UE indicates that the registration type is On-boarding Registration or NG-RAN indicates that the RRC connection is for On-boarding.

The UE shall initiate de-registration from the on-boarding network after finishing the remote provisioning or the on-boarding network shall initiate the de-registration after successful completion of on-boarding or based on timer configured for on-boarding service.

5.30.X.2.4.4	Initial Connectivity and Initial Registration when ON is a PLMN

When the ON is a PLMN for which the UE has got credentials, regular network selection and regular initial registration procedures apply.

When On-boarding network is a PLMN, the AMF restricts its functionality only for on-boarding usage based on received operator subscription from the UDM. Based on operator’s configuration, the subscription data shall contain a DNN, S-NSSAI or information (e.g., explicit indication) dedicated for on-boarding.

The UE shall initiate de-registration from the on-boarding network after finishing the remote provisioning or the on-boarding network shall initiate the de-registration after successful completion of on-boarding or based on timer configured for on-boarding service.

***** End of changes *****