SA WG2 Meeting #143E (e-meeting)	S2-2100588

February 24 – March 09, 2020, Elbonia

Source:	CATT

Title:	QoS Handling for Multicast and Broadcast services

Document for:	Approval

Agenda Item:	8.9.2

Work Item / Release:	5MBS / Rel-17

Abstract of the contribution: This contribution proposes the general description of QoS handling for multicast and broadcast services.

- Introduction

Based on TR 23.757 conclusions, this contribution proposes general description of QoS handling for multicast and broadcast services to the TS.

# 2. Proposal

It is proposed to include the following changes in TS 23.247.

******************** 1st Change (all new text)********************

6.5	QoS Handling for Multicast and Broadcast services

For MBS services, the network shall support QoS control per MBS session.

The 5G QoS model and parameters as defined in TS 23.501 [X] clause 5.7 also apply to MBS services with the following differences:

-	Reflective QoS is not applicable;

-	Wireline access network specific 5G QoS parameters do not apply to MBS services;

-	Alternative QoS Profile is not applicable;

-	QoS Notification Control is not applicable;

-	UE AMBR is not applicable;

-	Session-AMBR if provided is enforced at MB-UPF but not communicated to NG-RAN.

NOTE:	Whether Session-AMBR is required in addition to the MBS service data flow bit rate is determined by operator policy and/or agreement with the service provider.

-	For broadcast services, the QoS rule and associated QoS Flow level QoS parameters are not provided to UE.

The network shall support one or multiple QoS flows, which can be either GBR or non-GBR, for an MBS session.

If 5GC Individual MBS traffic delivery method is used to deliver multicast data packets, the network may use dedicated QoS flows for multicast data packets in a PDU session.

During MBS session establishment procedure, the AF (directly or via the NEF) provides the MBS session information description including QoS requirements to the PCF. The MB-SMF, based on PCC rules from the PCF for the MBS session (if dynamic PCC is deployed) or local policy (if dynamic PCC is not deployed), determines the QoS profiles and N4 rules for the MBS session with QoS parameters of the MBS QoS flows, and provides to the RAN and the MB-UPF respectively.

Optionally, for multicast services, the AF may set specific QoS requirement (i.e. priority) of a specific UE within an MBS Session to ensure that this MBS Session to which this UE participates can get priority when needed.

Editor's note:	Whether and how priority upgrade for MBS Session from MCPTT AS is support is FFS.

******************** End of Change ********************