3GPP TSG-WG SA2 Meeting #143E e-meeting  	S2-2100711

# Elbonia, 24 Feb – 09 Mar, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2551 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Event Exposure service Operation of Network Slice Status |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | eNS-Ph2(TR23.700-40) concluded the additional feature in 5GC to support Event Exposure of registered UEs or established PDU Sessions of Network Slice. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add a new service operation to support event exposure of Network Slice status by Network Slice Admission Control Function (NSACF). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC cannot support Event Exposure of Network Slice status by Network Slice Admission Control Function (NSACF) to the AF or other consumer NFs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.1, 4.15.3.1, 4.15.3.2.X (new), 5.2.xx (new), 5.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.15	Network Exposure

4.15.1	General

The network capability exposure comprises

-	Exposure of network events externally as well as internally towards core network NFs;

-	Exposure of provisioning capability towards external functions;

-	Exposure of policy and charging capabilities towards external functions;

-	Exposure of core network internal capabilities for analytics.

-	Exposure of analytics to external party.

-	Retrieval of data from external party by NWDAF.

-	Exposure of slice SLA parameters of the maximum number of registered UEs and/or established PDU Sessions to external party.

When subscribing to event reporting the NF consumer(s) provide:

-	One or multiple Event ID(s). An Event ID identifies the type of event being subscribed to (e.g. PDU Session Release, UE mobility out of an Area of Interest, etc.).

-	Event Filter Information: Provides Event Parameter Types and Event Parameter Value(s) to be matched against, in order to meet the condition for notifying the subscribed Event ID e.g. the Event Parameter Type could be “Area of interest” and Event Parameter Value list could be list of Tas; The Event Filter depends on the Event ID. The Event Filter Information is provided per Event ID(s) being subscribed to: within a subscription different Event ID(s) may be associated with different Event Filter Information.

-	Event Reporting Information described in the Table 4.15.1-1 below. Within a subscription all Event ID(s) are associated with a unique Event Reporting Information.

-	Target of Event Reporting: this may indicate a specific UE or PDU Session, a group of UE(s) or any UE (i.e. all Ues), Within a subscription all Event ID (s) are associated with the same Target of Event Reporting (possibly corresponding to multiple UE or multiple PDU Sessions).

-	A Notification Target Address (+ Notification Correlation ID) allowing the Event Receving NF to correlate notifications received from the Event provider with this subscription. A subscription is associated with an unique Notification Target Address (+ Notification Correlation ID). In the case that the NF consumer subscribes to the NF producer on behalf of other NF, the NF consumer includes the Notification Target Address(+Notification Correlation ID) of other NF for the Event ID which is to be notified to other NF directly, and the Notification Target Address(+Notification Correlation ID) of itself for the Subscription change related event notification. Each Notification Target Address(+ Notification Correlation ID) is associated with related (set of) Event ID(s).

-	An Expiry time represents the time upto which the subscription is desired to be kept as active. The NF service consumer may suggest an Expiry time and provide to the NF service producer. Based on the operator’s policy, the NF service producer decides whether the subscription can be expired. If the subscripton can be expired, the NF service producer determines the Expiry time and provide it in the response to the NF service consumer. If the event subscription is about to expire based on the received Expiry time and the NF service consumer wants to keep receiving notifications, the NF service consumer update the subscription with the NF service producer in order to extend the Expiry time. Once the Expiry time associated with the subscription is reached, the subscription becomes invalid at the NF service producer. If the NF service consumer wants to keep receiving notifications, it shall create a new subscription with the NF service producer.

When the subscription is accepted by the Event provider NF, the consumer NF receives from the event provider NF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

NOTE 1:	The Notification Correlation ID is allocated by the consumer NF that subscribes to event reporting and the Subscription Correlation ID is allocated by the NF that notifies when the event is met. Both correlation identifiers can be assigned the same value, although in principle they are supposed to be different, as they are optimized for finding the subscription related context within each NF.

The consumer NF may use an operation dedicated to subscription modification to add or remove Event ID(s) to this subscription or to modify Event Filter Information.

Events are subscribed by the consumer NF(s) by providing Event Filters. The contents of the Event Reporting Information along with the presence requirement of each information element is described in Table 4.15.1-1.

Table 4.15.1-1: Event Reporting Information

| Event Reporting Information Parameter | Description | Presence requirement |
| --- | --- | --- |
| Event reporting mode | Mode of reporting - e.g reporting up to a maximum number of reports, periodic reporting along with periodicity, reporting up to a maximum duration | mandatory |
| Maximum number of reports | Maximum number of reports after which the event subscription ceases to exist | (see NOTE 2) |
| Maximum duration of reporting | Maximum duration after which the event subscription ceases to exist | (see NOTE 2) |
| Immediate reporting flag | The Event provider NF notifies the current status of the subscribed event, if available, immediately to the consumer NF. |  |
| Sampling ratio | Percentage of sampling (1%..100%) among impacted Ues. | Optional (see NOTE 3) |
| Group Reporting Guard Time | Parameter for group-based monitoring configuration to indicate the time for which the Monitoring Event Reporting(s) related with the Ues in a group can be aggregated before sending them to the consumer NF. | Optional |
| NOTE 2: The requester shall include 2) Maximum number of reports or 3) Maximum duration of reporting, or both, depending on 1) Event reporting mode. NOTE 3: Parameter only applicable to certain event IDs reporting metrics (e.g. Number of Ues present in a geographical area) used and used e.g. by the NWDAF for data collection. |  |  |

NOTE 4:	Explicit unsubscribe by the NF consumer is still possible.

Maximum number of reports is applicable to the subscription to one UE or a group of UE(s). When the subscription is applied to a group of UE(s), the parameter is applied to each individual member UE. The count of number of reports is per Event Type granularity.

Maximum duration of reporting is applicable to the subscription to one UE, a group of UE(s) or any UE. When the subscription is applied to a group of UE(s), this parameter applies to each group member UE. When the subscription is applied to any UE, this parameter applies to all the impacted Ues.

If for a given subscription both Maximum Number of reports and Maximum duration of reporting are included then the subscription is considered to expire as soon as one of the conditions is met.

Sampling ratio is applicable to subscription targeting a group of Ues or any UE. When a sampling ratio is provided, a random subset is selected among the target Ues according to the sampling ratio and only the events related to this subset are reported. A UE remains selected until it is not managed by the event provider NF any more. A UE newly managed by the NF may become selected.

Group Reporting Guard Time is an optional parameter for group-based monitoring configuration to indicate the time for which the Monitoring Event Reporting(s) related with the Ues in a group can be aggregated before sending them to the consumer NF. The value of the Group Reporting Guard time should be set less than the Maximum duration of reporting. For the continuous monitoring reporting, unless the Maximum duration of reporting has been reached, the Group Reporting Guard timer is restarted when it expires. If the time left until the Maximum duration of reporting is less than the Group Reporting Guard Time, then the Group Reporting Guard timer shall be set to expire when the Maximum duration of reporting expires. If the Maximum duration of reporting is expired, the Group Reporting Guard Time, if running, shall be considered to expire and aggregated Monitoring Event Reporting(s) is sent to destination immediately.

Table 4.15.1-1 indicates the presence requirements for the Event Reporting Information.

Corresponding notifications contain at least the Notification Correlation ID together with the Event ID and the individual target (e.g. UE or PDU Session ID) associated with the notification.

If the NF service consumer decides to terminate the event subscription, it unsubscribes the event subscription by sending unsubscription request to the event provider NF. After receiving unsubscription request from the NF service consumer, the event provider NF terminates the event subscription.

The following clauses describe the external exposure of network capabilities and core network internal event and capability exposure.

When the immediate reporting flag is set, the first corresponding event report is included in the output message, if corresponding information is available at the reception of the subscription request of the event.

The optional parameter MTC Provider Information as used e.g. in clause 4.25.3, is a reference parameter that may be provided by AF or determined by NEF based on which AF it communicates with. The MTC Provider Information identifies the MTC Service Provider and/or MTC Application.

NOTE 5:	The MTC Provider Information can be used by Service Providers for, e.g. to distinguish their different customers.

# * * * * Second change * * * *

4.15.3	Event Exposure using NEF

4.15.3.1	Monitoring Events

The Monitoring Events feature is intended for monitoring of specific events in 3GPP system and making such monitoring events information reported via the NEF. It is comprised of means that allow NFs in 5GS for configuring the specific events, the event detection, and the event reporting to the requested party.

To support monitoring features in roaming scenarios, a roaming agreement needs to be made between the HPLMN and the VPLMN. If the AMF/SMF in the VPLMN determine that normalisation of a an event report is required, the AMF/SMF normalises the event report before sending it to the NEF.

The set of capabilities required for monitoring shall be accessible via NEF to NFs in 5GS. Monitoring Events via the UDM, the AMF, the SMF and the GMLC enables NEF to configure a given Monitor Event at UDM, AMF, SMF or GMLC, and reporting of the event via UDM and/or AMF, SMF or GMLC. Depending on the specific monitoring event or information, it is the AMF, GMLC or the UDM that is aware of the monitoring event or information and makes it reported via the NEF.

The following table enumerates the monitoring events and their detection criteria:

Table 4.15.3.1-1: List of events for monitoring capability

| Event | Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Loss of Connectivity | Network detects that the UE is no longer reachable for either signalling or user plane communication (see NOTE 4). The AF may provide a Maximum Detection Time, which indicates the maximum period of time without any communication with the UE after which the AF is to be informed that the UE is considered to be unreachable (see NOTE 7). | AMF |
| UE reachability | Detected when the UE transitions to CM-CONNECTED state or when the UE will become reachable for paging, e.g., Periodic Registration Update timer. It indicates when the UE becomes reachable for sending downlink data to the UE. The AF may provide the following parameters: 1) Maximum Latency; 2) Maximum Response Time; 3) Suggested number of downlink packets. (see NOTE 5 and NOTE 7). This event requires the Reachability Filter set to UE reachable for DL traffic” (see clause 5.2.2.3.1-1). For the usage of this event, see clauses 4.2.5.2 and 4.2.5.3. | AMF, UDM |
| Location Reporting | This event is detected based on the Event Reporting Information Parameters that were received in the Monitoring Request (one-time reporting, maximum number of reports, maximum duration of reporting, periodicity, etc., as specified in clause 4.15.1). It indicates either the Current Location or the Last Known Location of a UE. When AMF is the detecting NF: One-time and Continuous Location Reporting are supported for the Current Location. For Continuous Location Reporting the serving node(s) sends a notification every time it becomes aware of a location change, with the granularity depending on the accepted accuracy of location. (see NOTE 1) For Last Known Location only One-time Reporting is supported. When AMF is the detecting NF: If the immediate reporting flag is set, the AMF reports the UE Current Location (In case the AMF does not have the UE current location, the AMF get that information via NG-RAN Location reporting procedure as defined in clause 4.10). When GMLC is the detecting NF: Immediate and Deferred Location Reporting is supported. For Deferred Location Reporting the event types UE availability, Area, Periodic Location and Motion are supported. | AMF, GMLC |
| Change of SUPI-PEI association | This event is detected when the association between PEI and subscription (SUPI) changes (USIM change). | UDM |
| Roaming status | This event is detected when the UE’s current roaming status (the serving PLMN and/or whether the UE is in its HPLMN) and notification when that status changes. (see NOTE 2). | UDM |
| Communication failure | This event is detected when RAN or NAS level failure is detected based on connection release and it identifies RAN/NAS release code. | AMF |
| Availability after Downlink Data Notification failure | This event is detected when the UE becomes reachable again after downlink data delivery failure. | AMF |
| PDU Session Status | This event is detected when PDU session is established or released. (see NOTE 6) | SMF |
| Number of Ues present in a geographical area | This event is detected based on the Event Reporting Information Parameters that were received in the Monitoring Request (Level of aggregation, Sampling ratio, see clause 4.15.1). It indicates the number of Ues that are in the geographic area described by the AF. The AF may ask for the Ues that the system knows by its normal operation to be within the area (Last Known Location) or the AF may request the system to also actively look for the Ues within the area (Current Location). | AMF |
| CN Type change | The event is detected when the UE moves between EPC and 5GC. It indicates the current CN type for a UE or a group of Ues when detecting that the UE switches between being served by a MME and an AMF or when accepting the event subscription. (see NOTE 3) | UDM |
| Downlink data delivery status | It indicates the downlink data delivery status in the core network. Events are reported at the first occurrence of packets being buffered, transmitted or discarded, including: - Downlink data in extended buffering, including: - First data packet buffered event - Estimated buffering time, as per clause 4.2.3.3 - First downlink data transmitted event - First downlink data discarded event | SMF |
| UE reachability for SMS delivery | This event is detected when an SMSF is registered for a UE and the UE is reachable as determined by the AMF and the UDM. This enables the UE to receive an SMS. See clauses 4.2.5.2 and 4.2.5.3. | UDM |
| Slice SLA parameter status | It indicates the current number of Slice SLA parameters, including: - The number of registered UEs per indicated S-NSSAI - The number of established PDU Sessions per indicated S-NSSAI. | NSACF |
| NOTE 1: Location granularity for event request, or event report, or both could be at cell level (Cell ID) or TA level. The granularity can also be expressed by other formats such as geodetic uncertainty shapes (e.g. polygons, circles, etc.) or civic addresses (e.g. streets, districts, etc.) which can be mapped by NEF to AMF specific granularity levels. NOTE 2: Roaming status means whether the UE is in HPLMN or VPLMN. NOTE 3: CN type of CN Type change event is defined in clause 5.17.5.1 of TS 23.501 [2]. NOTE 4: In the case of UDM service operation information flow, the UDM should set the subscribed periodic registration timer to a smaller value than the value of Maximum Detection Time, since the value of the mobile reachable timer is larger than the value of the periodic registration timer. NOTE 5: Maximum Latency, Maximum Response Time and Suggested number of downlink packets are defined in clause 4.15.6.3a. NOTE 6: The NEF makes a mapping between the 5GS internal event "PDU Session Status" and the T8 API event "PDN Connectivity Status". NOTE 7: The preferred method for provisioning Network Configuration Parameters is External Parameter Provisioning specified in clause 4.15.6.3a. Provisioning event specific parameters as part of Monitoring Request is expected to be used only by the AF that does not support Parameter Provisioning procedure specified in clause 4.15.6.3a. |  |  |

# * * * * Third change * * * *

4.15.3.2.X	Information flow for exposure of slice SLA parameters

4.15.3.2.X.1	Exposure of slice SLA parameters subscribe/unsubscribe by AF via NEF

The procedure is used by the AF to subscribe to notifications and to explicitly cancel a previous subscription for the Network Slice Quota Status of the number of registered UEs or established PDU Sessions of an S-NSSAI.

Figure 4.15.3.2.X.1-1: Nnsacf_EventExposure_Subscribe, Unsubscribe and Notify service operation.

- The AF subscribes Network Slice SLA parameter status from NEF via Nnef_EventExposure_Subscribe (Event ID, Event Filter, Event Reporting information). The Event ID indicates the subscribed event ID, i.e.  Network Slice Registered UEs or Network Slice Established PDU Sessions. The Event Filter includes the information of S-NSSAI(s) and Event Reporting information includes reporting thresholds which indicate conditions to be met for the triggering of notification.

- The NEF checks whether the AF is authorised for the requested subscription. If this check successes, the NEF may query the NRF to find the NSACF and invoke Nnsacf_EventExposure_Subscribe Request (Event ID, Event Filter, Event Reporting Information) to the NSACF.

- The NSAC sends the Nnsacf_EventExposure_Subscribe Response message to the NEF.

- The NEF sends the Nnef_EventExposure_Subscribe Response message to the AF.

- [Conditional - depending on the Event in step 4]  Per the event ID, when the condition of event reporting information is met  the NSACF notifies the event. For example, the monitored number of registered UEs value exceeds the reporting threshold of the S-NSSAI, the NSACF invokes Nnsacf_EventExposure_Notify for the indicated event along with the time stamp.

- [Conditional - depending on the Event in step 5] The NEF notifies the event via Nnef_EventExposure_Notify service to the AF.

4.15.3.2.X.2	Exposure of slice SLA parameters query by AF via NEF

Figure 4.15.3.2.X.2-1: Nnsacf_SliceSLAInfo_Query service operations.

- The AF requests to receive Network Slice Quota status via NEF by invoking the Nnef_SliceSLAExposure_Query Request with input parameters, including slice SLA parameter to be queried, i.e. the number of registered UEs, or established PDU Sessions, or both and the information of S-NSSAI(s).

- Based on the request from the AF, the NEF check whether the AF is authorised for the requested query. If this check successes, the NEF may query the NRF to find the NSACF. The NEF invokes the Nnsacf_SliceSLAInfo_Query Request, which includes the slice SLA parameters to be queried, i.e. the number of registered UEs, established PDU Sessions or both, and the information of S-NSSAI(s) received from AF.

- The NSACF responds with the slice SLA information to the NEF.

- The NEF responds with the slice SLA information to the AF.

# * * * * Third change * * * *

5.2.xx	NSACF services

5.2.xx.1	General

The following table illustrates the NSACF services.

Table 5.2.xx-1: List of NSACF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnsacf_EventExposure | Subscribe | Subscribe/Notify | NEF |
|  | Unsubscribe |  | NEF |
|  | Notify |  | NEF |
| Nnsacf_SliceSLAInfo | Query | Request/Response | NEF |

5.2.xx.2	Nnsacf_EventExposure Services

5.2.xx.2.1	General

Service description: This service provides events related to network slice SLA parameters for the number of registered UEs, or established PDU session, or both towards a consumer NF (e.g., NEF). The following are the key functionalities of this NF service.

-	Allow consumer NFs to Subscribe and unsubscribe for Event ID(s) on network slice SLA parameters for the number of registered UEs, or established PDU session, or both; and

-	Notifying events on the network slice SLA parameters to the subscribed NFs.

The following events can be subscribed by a NF consumer.

-	The number of registered UEs status of an S-NSSAI.

-	The number of established PDU Sessions status of an S-NSSAI.

5.2.xx.2.2	Nnsacf_EventExposure_Subscribe service operation

Service operation name: Nnsacf_EventExposure_Subscribe

Description: This service operation is used by an NF (e.g., NEF) to subscribe or modify a subscription for event notifications on slice SLA parameters for the number of registered UEs, or established PDU session, or both.

Inputs, Required: Event ID(s), S-NSSAI(s), Notification Target Address (+ Notification Correlation ID), Event Reporting Information.

Inputs, Optional: None.

Outputs, Required: Subscription Correlation ID, Expiry time.

Outputs, Optional: None.

5.2.xx.2.3	Nnsacf_EventExposure_Unsubscribe service operation

Service operation name: Nnsacf_EventExposure_UnSubscribe.

Description: This service operation is used by an NF (e.g., NEF) to unsubscribe event notifications.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: None.

Output, Optional: None.

5.2.xx.2.4	Nnsacf_EventExposure_Notify service operation

Service operation name: Nnsacf _EventExposure_Notify

Description: NSACF reports the event to the consumer that has previously subscribed.

Inputs, Required: Event ID, Notification Correlation Information, time stamp.

Inputs, Optional: Event information (defined on a per Event ID basis).

Outputs, Required: Operation execution result indication.

5.2.xx.3	Nnsacf_ SliceSLAInfo Services

5.2.xx.3.1	General

This service is for allowing NF Service Consumer to ask network slice SLA information for the number of registered UEs, established PDU session, or both.

5.2.xx.3.2	Nnsacf_ SliceSLAInfo_Query service operation

Service operation name: Nnsacf_ SliceSLAInfo_Request

Description: The NEF requests slice SLA information.

Inputs Required: slice SLA parameters, S-NSSAI(s).

Inputs, Optional: None.

Outputs Required: Slice SLA information of the number of registered UEs, or established PDU Sessions, or both.

Outputs, Optional: None.

# * * * * Fourth change * * * *

5.2.6	NEF Services

5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef SliceSLAExposure | Query | Request/Response | AF |

5.2.6.2	Nnef_EventExposure service

5.2.6. xx	Nnef_SliceSLAExposure service

5.2.6.xx.1	General

5.2.6.xx.2	Nnef_SliceSLAExposure_Query service operation

Service operation name: Nnef_SliceSLAExposure_Query

Description: The NF consumer requests slice SLA information.

Inputs Required: slice SLA parameters, S-NSSAI(s).

Inputs, Optional: None.

Outputs Required: Slice SLA information of the number of registered UEs, or established PDU Sessions, or both.

Outputs, Optional: None.

# * * * * End of changes * * * *