3GPP TSG-SA/WG2 Meeting #143e	S2-2100780

# Elbonia, 24 Feb-9 Mar, 2021

Source:	ZTE

Title:	KI#3 conclusion

Document for:	Agreement

Agenda Item: 	8.4.1

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes conclusion for KI#3

1	Discussion

RAN2 LS S2-2100139 confirmed that RAN based solution (sol#22) is feasible therefore it is proposed to endorse the solution#22 for normative work.

When RAN based solution is not possible (i.e. no update) there is need to provide alternative solution. Solution 20(method 1) has impact only on interface between UDR and PCF, therefore it is proposed to endorse the solution 20(method 1) for normative work. Please note solution 20(method 1) can be used only when same PCF can be selected to serve all DNNs to the same S-NSSAI. For roaming with LBO case, the V-PCF cannot retrieve the subscription from the UDR in HPLMN, therefore it is proposed to use a local Slice-MBR in VPLMN.

2	Proposal

It is proposed to agree the changes in TR23.700-40:

/********************Start of Change***************/

8.3	conclusion for Key Issue #3

For Key Issue #3 (Limitation of data rate per network slice in UL and DL per UE):

-	A new QoS parameter, Slice-MBR per S-NSSAI is defined. The UL/DL Slice-MBR limits the aggregate data rate in UL and DL per UE across all GBR and Non-GBR QoS Flows for all PDU sessions associated with a S-NSSAI for the UE. The value of UL/DL Slice-MBR is stored in the UDR.

-	The AMF receives the Subscribed Slice-MBR from the UDM and send it to the (R)AN. The AMF or PCF may further adjust the Slice-MBR based on local configuration. (R)AN enforce the Slice-MBR as described in solution 22.

-	Alternatively, when same PCF is selected to serve all DNNs to the same S-NSSAI, the PCF receives the Subscribed Slice-MBR from the UDR and control the MBR and Session-AMBR for each PDU Session as described in solution 20 (Method 1). For roaming with LBO case, the V-PCF has no subscription information, but the V-PCF may still enforce the Slice-MBR based on local configuration.

/*******************End of Change****************/