SA WG2 Meeting #143-e (e-meeting)	S2-2100801

Elbonia, 24 February-9 March 2021	(was S2-200xxxx)

Source:	Qualcomm Incorporated

Title:	UE capabilities indication in UPU

Document for:	Discussion/Approval

Agenda Item:	8.2.1

Work Item / Release:	FS_eNPN/Rel.17

Abstract of the contribution: UE support for parameters provisioned with UPU procedure.

# Background

In TR 23.700-07 it is already agreed to use UE Parameters Update (UPU) procedure (what is called also “CP provisioning”) for the following parameters:

- Provisioning of NSSAA credentials (as per conclusion of KI#4 in TR 23.700-07)

- Provisioning of PDU session authentication credentials (as per conclusion of KI#4 in TR 23.700-07)

- Preferred SNPN and Group ID list (KI#1 in TR 23.700-07)

In addition, it is also quite probable, depending on the outcome of SA2#142 that the following will be also supported with UPU:

- Provisioning of SNPN credentials (tentative agreement in KI#4 in TR 23.700-07)

In rel.16, UPU procedure was used for provisioning of Routing ID and default S-NSSAI.  Support for update of Routing ID and default configured NSSAI is currently mandatory at the UE and therefore there is no indication of UE support.

While specifying the UPU procedure though in rel.15, CT1 made the code points for additional parameters “reserved” instead of “spare”, which means that if these code points are used toward a legacy UE, the UE will discard the whole container (even if the container includes parameters which the UE supports updating, like e.g. a Routing ID). Taking into account that support for the new parameters that SA2 agreed to be provisioned with UPU are going to become optional in NAS there is a need for an indication of UE support for each of these new parameters.

Since the UPU procedure was mostly specified between CT1 and SA3 and SA2 only did alignment CRs in TS 23.502, it is proposed to send an LS to CT1 and SA3 to highlight the issue and ask them to find a solution for capability negotiation for the parameters contained in UPU procedure.

# Conclusion

It is proposed to agree to send an LS to CT1 and SA3 and ask them to find a solution for UE capability indication of support of specific parameters in UPU procedure.