3GPP TSG-WG SA2 Meeting #143E e-meeting  	S2-2100875

# Elbonia, Feb 24 – Mar 09, 2021	rev S2-2008312

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2575 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Intra NG-RAN handover via CN for NR satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The satellites being considered by this work item are likely to provide coverage in multiple countries, but only have a more limited number of (e.g. 1) feeder links to satellite ground station(s). As a result, the satellite ground station is likely to act as an NG-RAN node to multiple core network nodes in multiple countries. In order to help satisfy LI requirements, when a UE moves in RRC connected state from one country to another country, this NG-RAN node may need to perform an intra NG-RAN node handover via the core network so that the target AMF is located in the country in which the UE is now located. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that N2 based handover can be used for intra NG-RAN handover. Existing handover procedures for network sharing scenarios (in 4.9.1.3) specify that the target PLMN ID is included in the TAI in the Handover Required message. Alternative approaches based on a “virtual gNB” concept could simply lead to “Xn handovers between the virtual gNBs” without any change of AMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LI requirements could be circumvented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The draft CR in S2-2008312 was approved in the October SA2 #141E emeeting. Other than updating to the latest version of TS 23.502, no specification changes have been made compared to S2-2008312. |  |  |  |  |  |  |  |  |

*** Start of Changes ***

### 4.9.1	Handover procedures in 3GPP access

#### 4.9.1.1	General

These procedures are used to hand over a UE from a source NG-RAN node to a target NG-RAN node using the Xn or N2 reference points. This can be triggered, for example, due to new radio conditions, load balancing or due to specific service e.g. in the presence of QoS Flow for voice, the source NG-RAN node being NR may trigger handover to E-UTRA connected to 5GC.

The Inter NG-RAN node N2 based handover procedure specified in clause 4.9.1.3 may also be used for intra-NG-RAN node handover.

NOTE: 	One use case for intra-NG-RAN handover to be performed by the Inter NG-RAN node N2 based handover procedure is when an NG-RAN node serves a satellite access system that covers more than one country. In such a situation, the UE might move from a "cell" in one country into a "cell" in another country, and the NG-RAN node may need to cause the AMF to change to an AMF serving the UE’s new country.

The RRC Inactive Assistance Information is included in N2 Path Switch Request Ack message for Xn based handover or Handover Request message for N2 based handover (see TS 23.501 [2] clause 5.3.3.2.5).

*** End of Change ***