3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2101016

24 Feb - 03 March 2021, Electronic	(revision of S2-200xxxx)

Source:	Nokia, Nokia Shanghai-Bell

Title:	KI#1: Conclusion update related to ROHC

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution updates the conclusions for key issue 1 according to the RAN2 decision that RAN will perform ROHC.

# Introduction

From S2-2100142 (Reply LS from RAN 2 on 5MBS progress and issues to address (R2-2102480)):

SA2 also asked RAN2 to provide feedback on the following question from SA4:

SA2 also received the following question from SA4 and believe RAN WGs are more suitable to respond to this first: SA4 Question: “The existing BM-SC hosts the SYNC (for time synchronization) and RoHC function. The prime reason here is MBSFN operation. SA4 understands that the 5MBS feature does not yet have a requirement for synchronization across adjacent cells, but that the related RAN normative work item does not preclude its introduction in a later release. Does SA2 have any view on the need of SYNC and/or RoHC support in the MBSF-U?”

RAN2 response:

SYNC protocol is not supported in the specifications in Rel-17. RAN2 has agreed that ROHC is to be located in RAN.

The current text allows that ROHC is activated upon AF request. This would lead to extra complexity if ROHC is performed by RAN. RAN should rather be allowed to decide autonomously whether to activate ROHC.

# 2. Text Proposal

It is proposed to agree the following changes for TR 23.757.

### 8.2.2.2	Multicast session

-	The Multicast session model is depicted in Figure 8.2.2.2-1, with the following conclusions:

-	The Multicast Service Context applies when the MBSF is used.

-	The SMF/MB-SMF based approach (i.e., SMF/MB-SMF handles session management for the UE) for Multicast session is adopted.

Editor's note:	The following aspects need to be studied:

-	How the SMF gets MB-SMF ID

-	For location dependent MB service: how the additional identifier (e.g. Flow Id or Area Session identifier) handled;

-	UE joining 5MBS may get rejected causing denial of service attack type situation

-	Roaming aspects

-	UE join MB service when involving ETSUN procedure

-	For support of 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method:

-	Both 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method shall be standardized for multicast data delivery. 5GC Shared MBS traffic delivery method is always mandatory, and 5GC Individual MBS traffic delivery is required to support UE mobility to/from non MBS-capable NG-RAN nodes, but otherwise optional.

-	The network shall be able to support selection of 5GC Shared MBS traffic delivery method or 5GC Individual MBS traffic delivery method based on criteria of whether RAN node supports 5MBS or not.

-	MB-UPF acts as the MBS session anchor when 5GC shared MBS traffic delivery method is used, and UPF acts as the unicast session anchor when 5GC individual MBS traffic delivery method is used. MBSF-U acts as the media entry point for the 5GS when MBSF is used.

Editor's note:	The following is ffs: If 5GC individual MBS traffic delivery method is used, the (MB-)UPF receives MBS traffic over N9 or N6 interface.

-	Establishment of the associated PDU Session for 5GC Individual MBS traffic delivery method is based on service requirements, networking configuration, local policy, etc.

Editor's note:	When and whether to establish or update the associated PDU session for 5GC individual MBS traffic delivery is ffs.

-	It shall be possible to establish an Associated PDU session for cases, if not exists, where mobility to non-5GMBS-supporting cells happens.

-	It shall be possible to update the associated PDU session with associated QoS flows when the UE joins the MBS Session.

Figure 8.2.2.2-1: Merged MBMS session model

NOTE 1a:	The model above needs clarification for its application on UE, 5GC and RAN side, this will be part of normative work.

NOTE 1b: The terms "stop", "release", "deactivated" (as well as "establishment", "start", "activation") used as MBS session actions need clear definitions, this will be part of normative work.

NOTE 2:  When the MBS Session is deactivated, whether the multicast flow need be removed from the MBS Session context is to be decided in normative phase.

-	For multicast session establishment/join/leave/release:

-	The UE may perform application level join/leave to a multicast session, the 5GC shall support multicast session join/leave operation for a user, e.g based on AF request.

-	UE shall support multicast session join/leave operation via CP (NAS signalling for SM procedure)

Editor's note:	UP Join is FFS.

-	The UE shall indicate leaving an MBS session in CM-CONNECTED with RRC-CONNECTED state.

Editor's note:	Whether the UE can stop receiving traffic of a multicast session without indicating leaving in CM-IDLE state or CM-CONNECTED with RRC-INACTIVE state relies on RAN WG feedback.

-	The 5GC shall be able to reject UE joining to a multicast session when the multicast session will not start soon or has not started..

-	ROHC for MBS traffic is supported by the NG RAN.

-	MBSF-C and MBSF-U functionality is supported based on A.3.

Editor's note:	Coordination with SA4 is required to determine MBSF-C and MBSF-U functionality.

-	The network shall support selection of MB-SMF or SMF at session join.

-	The (MB-)SMF decides whether to accept join requests, which may be based on input from NEF/MBSF-C if MBSF is used, and stores that the served UE is participating in a multicast session.

-	The PDU session which is used to send the join is the same as the associated PDU Session which is for 5GC Individual MBS traffic delivery.

-	The AMF shall select an SMF that supports 5MBS for multicast session join during PDU session establishment, which is used for sending join (i.e. handling of join requests for 5MBS and/or fallback to individual delivery).

Editor's note:	RAN and/or SA3 is assumed to determine the handling of the security for MBS traffic.

-	For N3 transport of the 5GC Shared MBS traffic delivery method, GTP-U tunnelling using a transport layer IP multicast method and shared N3 (GTP-U) Point-to-Point tunnel shall be supported with support for QoS.

-	For N3 transport of the 5GC Shared MBS traffic delivery method, for unicast transport there shall be 1-1 mapping between MBS Session and GTP-U tunnel towards a RAN node, and for multicast transport there shall be 1-1 mapping between MBS Session and the GTP-U tunnel.

-	For N9 transport of the 5GC Shared MBS traffic delivery method, GTP-U tunnelling using a transport layer IP multicast method and shared N9 (GTP-U) Point-to-Point tunnel shall be supported.

-	For N9 transport of the 5GC Shared MBS traffic delivery method, for unicast transport there shall be 1-1 mapping between MBS Session and GTP-U tunnel towards a UPF, and for multicast transport there shall be 1-1 mapping between MBS Session and the GTP-U tunnel.

-	The network supports indicating of N6 tunnel information for receiving traffic of a MBS session to the AF or MBSF(and to MBSF-U).

-	5GC provides information of MBS sessions/groups (e.g. TMGI) and QoS requirements of a MBS service to RAN for MBS AN resources handling.

-	For multicast service parameters storage, the UDR shall be able to store the AF provisioned or preconfigured service parameters per MBS session.

-	The PCF shall be able to provide policy and QoS requirement per MBS session to the MB-SMF

-	For UE receiving MBS traffic moving from one RAN node to another in CM-CONNECTED and RRC-CONNECTED state, handover procedure with MB context shall be supported by UE and network.

-	When MBS session is released, the N3 transport of the 5GC shared MBS delivery method is released and the radio resource associated with the MBS QoS Flows are released, or the N3/N9 transport of the 5GC Individual MBS traffic delivery method is released and the radio resource associated with the QoS Flows are released.

-	MB-UPF is used as the MBS Session anchor for the 5GC shared MBS delivery.

-	Interactions between the MBSF-C and MBSF-U will be defined in coordination with 3GPP SA4.

-	The 5GC shall be able to trigger NG-RAN nodes to notify session start/activation of an MBS session to UEs.

Editor's note:	How the NG-RAN node notify session activation to UEs relies on RAN WG feedback.

-	When an MBS session is established upon AF request, it may be immediately activated (i.e. one AF request combining of session announcement and session activation). If an application requires to allow UEs to join prior to the start/activation of the multicast transmission, the AF may interact with the NEF separately to announce the multicast service and subsequently to request start/activation of the multicast service.

-	If a UE joins prior to the activation of the multicast session, the network indicates to the UE that the session is not yet activated.

-	The following bullets apply if MB-UPF detecting multicast data triggers MBS Session deactivation or activation.

NOTE 3:	Whether the terms "stop/deactivated" or "start/activation" denote the same actions needs to be further clarified.

-	The MBS Session may be deactivated when the MB-UPF detects no multicast data for a configurable period. When the MBS Session is deactivated by 5GC, the MBS Session context is kept in 5GC, but the AN resource with context and N3 tunnel for 5GC Shared MBS delivery method are released. UEs that have joined that multicast session can become IDLE:

NOTE 4:	Whether the MBS QoS flow need be removed from the MBS Session context is to be decided in normative phase.

-	The MBS Session may be activated when the MB-UPF detects multicast data. When the MBS Session needs to be activated, the MB-UPF sends message to the MB-SMF. When the MB-SMF starts the MBS session activation for establishing the transmission resources, the MB-SMF notifies the session activation to NG-RANs via SMFs/AMFs serving UEs within the multicast session. UEs are notified by NG-RAN about the session activation.

-	The following bullets apply if AF decides to start or stop a multicast session:

NOTE 5:	Whether the terms "stop/deactivated" or "start/activation" denote the same actions needs to be further clarified.

-	The MBS Session may be stopped upon AF requests. When the MBS Session is stopped, the MBS Session context is kept in 5GC, but the AN resource with context and N3 tunnel for 5GC Shared MBS delivery method are released. The multicast QoS flow information are removed from the MBS Session context stored at the UE and 5GC NF. UEs that have joined that multicast session can become IDLE.

-	The MBS Session may be activated/started upon AF requests. When the MBS Session needs to be activated/started, the NEF or MBSF sends message to the MB-SMF for establishing the transmission resources. The MB-SMF obtains the related Multicast QoS flow information from PCF. When the MB-SMF restarts the MBS session, the MB-SMF notifies the session activation to NG-RANs via SMFs/AMFs serving UEs within the multicast session. UEs are notified by NG-RAN about the session activation.

Editor's note:	Coordination with RAN WGs are needed.