SA WG2 Meeting #S2-143E	S2-210xxxx

24 February - 09 March, 2021, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0xxx | rev | - | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Principles of multicast and broadcast communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Principles for multicast and broadcast communication needs to be stated in specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add principles for multicast and broadcast communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Principles for multicast and broadcast communication is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change (all new text)********************

4.1	Principles of multicast and broadcast communication

For multicast and broadcast communication, the following common architectural principles and requirements apply:

-	Follow the 5G System architectural principles as defined in TS 23.501 [X], including flexibility and modularity for newly introduced functionalities.

-	Provide an efficient transport for a variety of multicast and broadcast services.

-	Minimize impact to existing external services.

-	Only NR of NG-RAN connected to 5GC is considered in this release.

MBS traffic needs to be delivered from a single data source (e.g. MBS application server) to multiple UEs. Depending on many factors, different delivery methods as described below are used to deliver multicast traffic in the 5GS.

NOTE 1:	For clarity, delivery methods are not referred to as unicast/multicast/broadcast. The term "unicast delivery" refers to a mechanism by which application data and signalling between the UE and the application server are delivered using PDU Session within the 3GPP network and using individual UE and application server addresses (e.g. IP addresses) between the 3GPP network and the application server. It is not equivalent to 5GC Individual MBS traffic delivery method defined in this clause.

For transmission of MBS data packets from 5GC to RAN, two delivery methods are possible:

-	5GC Shared MBS traffic delivery method: 5GC receives a single copy of MBS data packets and delivers a single copy of those MBS data packets to a RAN node, which then delivers them to one or multiple UEs.

-	5GC Individual MBS traffic delivery method: 5GC receives a single copy of MBS data packets and delivers separate copies of those MBS data packets to individual UEs via per-UE PDU sessions, hence for each such UE one PDU session is required to be associated with a multicast session.

The MBS data packets of the same multicast session may be delivered by the CN via both 5GC Shared MBS traffic delivery method for some UEs and 5GC Individual MBS traffic delivery method for other UE(s), if 5GC Individual MBS traffic delivery method is supported.

If the NG-RAN node supports 5MBS, the network shall use the 5GC Shared MBS traffic delivery method for multicast session packet transfer.

Switching between 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method shall be supported, e.g. due to UE mobility between a RAN node supporting 5MBS and a RAN node not supporting 5MBS. See clause 6.3.

For transmission of MBS data packets from RAN to UE over radio, in the case of 5GC Shared MBS traffic delivery method, two delivery methods are available:

-	Point-to-Multipoint (PTM) delivery method: a RAN node delivers a single copy of MBS data packets over radio to a set of UEs.

-	Point-to-Point (PTP) delivery method: a RAN node delivers separate copies of MBS data packets over radio to individual UEs.

A RAN node may use a combination of PTP/PTM to deliver MBS data packets to UEs.

Switching between PTP and PTM delivery methods for 5GC Shared MBS traffic delivery shall be supported. NG-RAN is the decision point for switching between PTP and PTM delivery methods.

Editor's note:	Whether any assistance information from CN is needed, e.g. for PTP/PTM delivery method decision and switching, needs further confirmation when the relevant conclusion is reached in RAN WGs.

NOTE 2:	The PTP and PTM delivery methods are defined in RAN WGs and they are listed here for reference only.

As depicted in Figure 4.1-1, PTP or PTM delivery (with 5GC shared delivery method) and 5GC Individual delivery method may be used at the same time for a multicast session.

Figure 4.1-1: Schematic showing delivery methods

For MBS broadcast service, only 5GC Shared MBS traffic delivery method with PTM delivery is applicable.

******************** End of Change ********************