SA WG2 Meeting #143E	S2-2100304r01

Feb 24 – Mar 9, 2021, Electronic

Source:	Futurewei, Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Cisco

Title:	UE Considerations for EAS rediscovery

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	eEdge_5GC /Rel-17

Abstract of the contribution: This contribution introduces considerations on OS/user for EAS (re-)discovery.

1	Discussion

The case that OS/user overrides operator DNS setting is part of the considerations in TR clause 9.1.1. The new text proposed below captures the recommendations and limitations in Annex X.

2	Proposal

It is proposed to adopt the following changes into TS23.548.]

* * * * 1st Change * * * *

Annex X (informative): UE considerations for EAS (re)discovery

X.1	General

DNS records obtained from a network resolver contains a time-to-live (TTL) value. This is a hint provided by the network resolver and maybe used to determine the length of time that the record is cached. DNS records maybe be cached in the UE by a system wide stub resolver and by application layer name resolution caches. The application (L7) cache is managed on a per application basis while the OS/system DNS cache is common to applications. Name resolution caches in various applications also have different policies and behaviour. Some applications cache the name records for the length of the application session while others have a time limit. Coordination across the 5GC and application domain for name resolver caching behaviour is required to support some of the solutions in this TS.

The following clauses describe the appropriate DNS configuration for the EAS (re)-discovery to work in the UE.

X.2	Impact of Unicast/Anycast IP Addresses for DNS resolver and EAS

The UE/application may be configured to use unicast or anycast IP addresses for DNS resolver (DNS server).

The UE can be configured with a unicast or anycast IP address for the MNO DNS resolver using ePCO or IPv6 Router Advertisement (RA). 5GC may reconfigure the MNO DNS resolver IP address for each network change event with the right MNO DNS resolver address. . In case of anycast, the 5GC is responsible for routing to the “closest” instance of the MNO DNS resolver.

Applications in the UE can request the UE DNS resolver to resolve the FQDN. However, applications can also be configured with their own DNS resolver address and use encrypted messaging based on DNS over HTTPS (DoH) or DNS over TLS (DoT). Configuration of application DNS resolvers is out of scope of 5GS. DNS messages delivered over DoT or DoH should be forwarded transparently to the destination address of DNS resolver in the DNS query. The application DNS resolver may be operated by the MNO or by a third party.

A network interface change or NAS SM EAS rediscovery indication can result in the UE OS clearing name/IP address translations in its DNS cache. An application can continue the L4 connection with the old EAS IP address until the application resets new EAS IP address.

EAS rediscovery or routing of anycast address from a new UE location can result in the selection of new EAS instances. Any session state needs to be handled by the UE application and EAS.

X.3	UE requirements for EAS re-discovery

An application that complies with EAS (re-)discovery described in this specification in the UE is not recommended to override operator-provided DNS settings.

The OS DNS server configuration is not recommended to override the operator provided DNS in a UE compliant to the EAS (re-)discovery procedure. This is necessary for the “closest” EAS server to be selected.

NOTE 1:	If a non-compliant OS, user or non-compliant applications override the operator-provided DNS settings, the DNS resolvers or servers in the third party can take the source IP address of the DNS request as the location information of UE, which may correspond to the local/remote PSA UPF or other entities (e.g., a NAT server) on the N6 interface.

NOTE 2:	If the DNS server configuration in a non-compliant OS overrides the operator provided DNS, the DNS queries continue to be sent over the correct PDU session for the application.

NOTE 3:	The UE modem transparently forwards DNS messages for tethered devices that are loosely coupled.

NOTE 4: 	If the UE (OS or application) uses a DNS resolver that is different than the one provided by the operator, then:

-	the solutions in this specification will only work if the recursive DNS resolution using the non-operator provided DNS resolver gets UE location to resolve the IP address of the proximate EAS for that UPF-PSA/IP subnet, and

-	for the session breakout modes A, B, C in 6.2.3.2, recursive DNS resolution should result in edge EASDF being in the operator DNS resolver chain.

X.4	UE procedures for session breakout

In the session breakout connectivity model, the selection of a new session breakout path does not result in a new network interface indication at the UE.

NOTE 1:	In case of multiple sessions or distributed anchor point connectivity models, when there is a change of network interface, indication of network interface change may be used to flush the UE OS DNS cache.

For session breakout case, the SMF may provide EAS rediscovery indication and its optional associated impact field within NAS message to UE. Based on the received EAS rediscovery indication and its associated impact field, when a new connection to EAS need to be established, the UE can re-discover the new EAS without reusing the cached old EAS information. This does not impact the application client.

* * * * End of Changes * * * *