3GPP TSG-SA WG2 Meeting #143-e	S2-2100357r11

# E-Meeting, 24th February – 09th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2562 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE onboarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm, MediaTek Inc., Nokia, Nokia Shanghai Bell, Futurewei, Convida Wireless, Intel, InterDigital, OPPO, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The functionality to support UE onboarding as per TR 23.700-07 conclusion includes changes to enable the UE to select network that supports UE onboarding as well as enable the UE to get connectivity for a subsequent provisioning of the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A general introduction to the feature, definitions and abbreviations and an architecture. NG-RAN impacts, and UE impacts for network selection. 3GPP connectivity for UE to realize a subequent remote provisioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE onboarding is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 5.30.1, 5.30.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of changes *****

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5G VN Group: A set of UEs using private communication for 5G LAN-type service.

5G Access Network: An access network comprising a NG-RAN and/or non-3GPP AN connecting to a 5G Core Network.

5G Core Network: The core network specified in the present document. It connects to a 5G Access Network.

5G LAN-Type Service: A service over the 5G system offering private communication using IP and/or non-IP type communications.

5G LAN-Virtual Network: A virtual network over the 5G system capable of supporting 5G LAN-type service.

5G QoS Flow or QoS Flow: The finest granularity for QoS forwarding treatment in the 5G System. All traffic mapped to the same 5G QoS Flow receive the same forwarding treatment (e.g. scheduling policy, queue management policy, rate shaping policy, RLC configuration, etc.). Providing different QoS forwarding treatment requires separate 5G QoS Flow.

5G QoS Identifier: A scalar that is used as a reference to a specific QoS forwarding behaviour (e.g. packet loss rate, packet delay budget) to be provided to a 5G QoS Flow. This may be implemented in the access network by the 5QI referencing node specific parameters that control the QoS forwarding treatment (e.g. scheduling weights, admission thresholds, queue management thresholds, link layer protocol configuration, etc.).

5G System: 3GPP system consisting of 5G Access Network (AN), 5G Core Network and UE.

5G-BRG: The 5G-BRG is a 5G-RG defined in BBF.

5G-CRG: The 5G-CRG is a 5G-RG specified in DOCSIS MULPI [89].

5G-RG: A 5G-RG is a RG capable of connecting to 5GC playing the role of a UE with regard to the 5G core. It supports secure element and exchanges N1 signalling with 5GC. The 5G-RG can be either a 5G-BRG or 5G-CRG.

Access Traffic Steering: The procedure that selects an access network for a new data flow and transfers the traffic of this data flow over the selected access network. Access traffic steering is applicable between one 3GPP access and one non-3GPP access.

Access Traffic Switching: The procedure that moves all traffic of an ongoing data flow from one access network to another access network in a way that maintains the continuity of the data flow. Access traffic switching is applicable between one 3GPP access and one non-3GPP access.

Access Traffic Splitting: The procedure that splits the traffic of a data flow across multiple access networks. When traffic splitting is applied to a data flow, some traffic of the data flow is transferred via one access and some other traffic of the same data flow is transferred via another access. Access traffic splitting is applicable between one 3GPP access and one non-3GPP access.

Allowed NSSAI: NSSAI provided by the Serving PLMN during e.g. a Registration procedure, indicating the S-NSSAIs values the UE could use in the Serving PLMN for the current Registration Area.

Allowed Area: Area where the UE is allowed to initiate communication as specified in clause 5.3.2.3.

AMF Region: An AMF Region consists of one or multiple AMF Sets.

AMF Set: An AMF Set consists of some AMFs that serve a given area and Network Slice(s). AMF Set is unique within an AMF Region and it comprises of AMFs that support the same Network Slice(s). Multiple AMF Sets may be defined per AMF Region. The AMF instances in the same AMF Set may be geographically distributed but have access to the same context data.

Application Identifier: An identifier that can be mapped to a specific application traffic detection rule.

AUSF Group ID: This refers to one or more AUSF instances managing a specific set of SUPIs. An AUSF Group consists of one or multiple AUSF Sets.

Binding Indication: Information included by a NF service producer to a NF service consumer in request responses or notifications to convey the scope within which selection/reselection of target NF/NF Services may be performed, or information included by the NF service consumer in requests or subscriptions to convey the scope within which selection/reselection of notification targets or the selection of other service(s) that the NF consumer produces for the same data context may be performed. See clause 6.3.1.0.

Configured NSSAI: NSSAI provisioned in the UE applicable to one or more PLMNs.

CHF Group ID: This refers to one or more CHF instances managing a specific set of SUPIs.

Default UE credentials: Information configured in the UE to make the UE uniquely identifiable and verifiably secure to perform UE onboarding.

Default Credential Server (DCS): An entity that can authenticate the UE based on the Default UE credentials or provide means for another entity to authenticate the UE based on the Default UE credentials.

Delegated Discovery: This refers to delegating the discovery and associated selection of NF instances or NF service instances to an SCP.

Direct Communication: This refers to the communication between NFs or NF services without using an SCP.

DN Access Identifier (DNAI): Identifier of a user plane access to one or more DN(s) where applications are deployed.

Emergency Registered: A UE is considered Emergency Registered over an Access Type in a PLMN when registered for emergency services only over this Access Type in this PLMN.

Endpoint Address: An address in the format of an IP address or FQDN, which is used to determine the host/authority part of the target URI. This Target URI is used to access an NF service (i.e. to invoke service operations) of an NF service producer or for notifications to an NF service consumer.

En-gNB: as defined in TS 37.340 [31].

Expected UE Behaviour: Set of parameters provisioned by an external party to 5G network functions on the foreseen or expected UE behaviour, see clause 5.20.

Fixed Network Residential Gateway: A Fixed Network RG (FN-RG) is a RG that it does not support N1 signalling and it is not 5GC capable.

Fixed Network Broadband Residential Gateway: A Fixed Network RG (FN-BRG) is a FN-RG specified in BBF TR-124 [90].

Fixed Network Cable Residential Gateway: A Fixed Network Cable RG (FN-CRG) is a FN-RG with cable modem specified in DOCSIS MULPI [89].

Forbidden Area: An area where the UE is not allowed to initiate communication as specified in clause 5.3.2.3.

GBR QoS Flow: A QoS Flow using the GBR resource type or the Delay-critical GBR resource type and requiring guaranteed flow bit rate.

IAB-donor: This is a NG-RAN node that supports Integrated access and backhaul (IAB) feature and provides connection to the core network to IAB-nodes. It supports the CU function of the CU/DU architecture for IAB defined in TS 38.401 [42].

IAB-node: A relay node that supports wireless in-band and out-of-band relaying of NR access traffic via NR Uu backhaul links. It supports the UE function and the DU function of the CU/DU architecture for IAB defined in TS 38.401 [42].

Indirect Communication: This refers to the communication between NFs or NF services via an SCP.

Initial Registration: UE registration in RM-DEREGISTERED state as specified in clause 5.3.2.

Intermediate SMF (I-SMF): An SMF that is inserted to support a PDU session as the UE is located in an area which cannot be controlled by the original SMF because the UPF(s) belong to a different SMF Service Area.

Local Area Data Network: a DN that is accessible by the UE only in specific locations, that provides connectivity to a specific DNN, and whose availability is provided to the UE.

Local Break Out (LBO): Roaming scenario for a PDU Session where the PDU Session Anchor and its controlling SMF are located in the serving PLMN (VPLMN).

LTE-M: a 3GPP RAT type Identifier used in the Core Network only, which is a sub-type of E-UTRA RAT type, and defined to identify in the Core Network the E-UTRA when used by a UE indicating Category M.

MA PDU Session: A PDU Session that provides a PDU connectivity service, which can use one access network at a time, or simultaneously one 3GPP access network and one non-3GPP access network.

Mobility Pattern: Network concept of determining within the AMF the UE mobility parameters as specified in clause 5.3.2.4.

Mobility Registration Update: UE re-registration when entering new TA outside the TAI List as specified in clause 5.3.2.

MPS-subscribed UE: A UE having a USIM with MPS subscription.

NB-IoT UE Priority: Numerical value used by the NG-RAN to prioritise between different UEs accessing via NB-IoT.

NGAP UE association: The logical per UE association between a 5G-AN node and an AMF.

NGAP UE-TNLA-binding: The binding between a NGAP UE association and a specific TNL association for a given UE.

Network Function: A 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces.

NOTE 1:	A network function can be implemented either as a network element on a dedicated hardware, as a software instance running on a dedicated hardware, or as a virtualised function instantiated on an appropriate platform, e.g. on a cloud infrastructure.

Network Instance: Information identifying a domain. Used by the UPF for traffic detection and routing.

Network Slice: A logical network that provides specific network capabilities and network characteristics.

Network Slice instance: A set of Network Function instances and the required resources (e.g. compute, storage and networking resources) which form a deployed Network Slice.

Non-GBR QoS Flow: A QoS Flow using the Non-GBR resource type and not requiring guaranteed flow bit rate.

NSI ID: an identifier for identifying the Core Network part of a Network Slice instance when multiple Network Slice instances of the same Network Slice are deployed, and there is a need to differentiate between them in the 5GC.

NF instance: an identifiable instance of the NF.

NF service: a functionality exposed by a NF through a service based interface and consumed by other authorized NFs.

NF service instance: an identifiable instance of the NF service.

NF service operation: An elementary unit a NF service is composed of.

NF Service Set: A group of interchangeable NF service instances of the same service type within an NF instance. The NF service instances in the same NF Service Set have access to the same context data.

NF Set: A group of interchangeable NF instances of the same type, supporting the same services and the same Network Slice(s). The NF instances in the same NF Set may be geographically distributed but have access to the same context data.

NG-RAN: A radio access network that supports one or more of the following options with the common characteristics that it connects to 5GC:

1)	Standalone New Radio.

2)	New Radio is the anchor with E-UTRA extensions.

3)	Standalone E-UTRA.

4)	E-UTRA is the anchor with New Radio extensions.

Non-Allowed Area: Area where the UE is allowed to initiate Registration procedure but no other communication as specified in clause 5.3.2.3.

Non-Public Network: See definition in TS 22.261 [2].

Non-Seamless Non-3GPP offload: The offload of user plane traffic via non-3GPP access without traversing either N3IWF/TNGF or UPF.

PCF Group ID: This refers to one or more PCF instances managing a specific set of SUPIs. A PCF Group consists of one or multiple PCF Sets.

Pending NSSAI: NSSAI provided by the Serving PLMN during a Registration procedure, indicating the S-NSSAI(s) for which the network slice-specific authentication and authorization procedure is pending.

PDU Connectivity Service: A service that provides exchange of PDUs between a UE and a Data Network.

PDU Session: Association between the UE and a Data Network that provides a PDU connectivity service.

PDU Session Type: The type of PDU Session which can be IPv4, IPv6, IPv4v6, Ethernet or Unstructured.

Periodic Registration Update: UE re-registration at expiry of periodic registration timer as specified in clause 5.3.2.

Pre-configured 5QI: Pre-defined QoS characteristics configured in the AN and 5GC and referenced via a non-standardized 5QI value.

Private communication: See definition in TS 22.261 [2].

Public network integrated NPN: A non-public network deployed with the support of a PLMN.

(Radio) Access Network: See 5G Access Network.

RAT type: Identifies the transmission technology used in the access network for both 3GPP accesses and non-3GPP Accesses, for example, NR, NB-IOT, Untrusted Non-3GPP, Trusted Non-3GPP, Trusted IEEE 802.11 Non-3GPP access, Wireline, Wireline-Cable, Wireline-BBF, etc.

Requested NSSAI: NSSAI provided by the UE to the Serving PLMN during registration.

Residential Gateway: The Residential Gateway (RG) is a device providing, for example voice, data, broadcast video, video on demand, to other devices in customer premises.

Routing Binding Indication: Information included in a request or notification and that can be used by the SCP for discovery and associated selection to of a suitable target. See clauses 6.3.1.0 and 7.1.2

Routing Indicator: Indicator that allows together with SUCI/SUPI Home Network Identifier to route network signalling to AUSF and UDM instances capable to serve the subscriber.

SCP Domain: A configured group of one or more SCP(s) and zero or more NF instances(s). An SCP within the group can communicate with any NF instance or SCP within the same group directly, i.e. without passing through an intermediate SCP.

SNPN enabled UE: A UE configured to use stand-alone Non-Public Networks.

SNPN access mode: A UE operating in SNPN access mode only selects stand-alone Non-Public Networks over Uu.

Service based interface: It represents how a set of services is provided/exposed by a given NF.

Service Continuity: The uninterrupted user experience of a service, including the cases where the IP address and/or anchoring point change.

Service Data Flow Filter: A set of packet flow header parameter values/ranges used to identify one or more of the packet (IP or Ethernet) flows constituting a Service Data Flow.

Service Data Flow Template: The set of Service Data Flow filters in a policy rule or an application identifier in a policy rule referring to an application detection filter, required for defining a Service Data Flow.

Session Continuity: The continuity of a PDU Session. For PDU Session of IPv4 or IPv6 or IPv4v6 type "session continuity" implies that the IP address is preserved for the lifetime of the PDU Session.

SMF Service Area: The collection of UPF Service Areas of all UPFs which can be controlled by one SMF.

Stand-alone Non-Public Network: A non-public network not relying on network functions provided by a PLMN

Subscribed S-NSSAI: S-NSSAI based on subscriber information, which a UE is subscribed to use in a PLMN

Time Sensitive Communication (TSC): A communication service that supports deterministic communication and/or isochronous communication with high reliability and availability. It is about providing packet transport with QoS characteristics such as bounds on latency, loss, and reliability, where end systems and relay/transmit nodes can be strictly synchronized.

TSN working domain: Synchronization domain for a localized set of devices collaborating on a specific task or work function in a TSN network, corresponding to a gPTP domain defined in IEEE 802.1AS [104].

UDM Group ID: This refers to one or more UDM instances managing a specific set of SUPIs. An UDM Group consists of one or multiple UDM Sets.

UDR Group ID: This refers to one or more UDR instances managing a specific set of SUPIs. An UDR Group consists of one or multiple UDR Sets.

UPF Service Area: An area consisting of one or more TA(s) within which PDU Session associated with the UPF can be served by (R)AN nodes via a N3 interface between the (R)AN and the UPF without need to add a new UPF in between or to remove/re-allocate the UPF.

Uplink Classifier: UPF functionality that aims at diverting Uplink traffic, based on filter rules provided by SMF, towards Data Network.

WB-E-UTRA: In the RAN, WB-E-UTRA is the part of E-UTRA that excludes NB-IoT. In the Core Network, WB-E-UTRA also excludes LTE-M.

Wireline 5G Access Network: The Wireline 5G Access Network (W-5GAN) is a wireline AN that connects to a 5GC via N2 and N3 reference points. The W-5GAN can be either a W-5GBAN or W-5GCAN.

Wireline 5G Cable Access Network: The Wireline 5G Cable Access Network (W-5GCAN) is the Access Network defined in CableLabs.

Wireline BBF Access Network: The Wireline 5G BBF Access Network (W-5GBAN) is the Access Network defined in BBF.

Wireline Access Gateway Function (W-AGF): The Wireline Access Gateway Function (W-AGF) is a Network function in W-5GAN that provides connectivity to the 5G Core to 5G-RG and FN-RG.

NOTE 2:	If one AUSF/PCF/UDR/UDM group consists of multiple AUSF/PCF/UDR/UDM Sets, AUSF/PCF/UDR/UDM instance from different Set may be selected to serve the same UE. The temporary data which is not shared across different Sets may be lost, e.g. the event subscriptions stored at one UDM instance are lost if another UDM instance from different Set is selected and no data shared across the UDM Sets.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5G-VN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-AN PDB	5G Access Network Packet Delay Budget

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G GM	5G Grand Master

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G VN	5G Virtual Network

5QI	5G QoS Identifier

AF	Application Function

AMF	Access and Mobility Management Function

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BMCA	Best Master Clock Algorithm

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

EUI	Extended Unique Identifier

FAR	Forwarding Action Rule

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GFBR	Guaranteed Flow Bit Rate

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

IMEI/TAC	IMEI Type Allocation Code

IPUPS	Inter PLMN UP Security

I-SMF	Intermediate SMF

I-UPF	Intermediate UPF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LMF	Location Management Function

LoA	Level of Automation

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

N3IWF	Non-3GPP InterWorking Function

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NSI ID	Network Slice Instance Identifier

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAAF	Network Slice-Specific Authentication and Authorization Function

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

ON	Onboarding Network

O-SNPN	Onboarding Standalone Non-Public Network

PCF	Policy Control Function

PDB	Packet Delay Budget

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PEI	Permanent Equipment Identifier

PER	Packet Error Rate

PFD	Packet Flow Description

PNI-NPN	Public Network Integrated Non-Public Network

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

PTP	Precision Time Protocol

PVS	Provisioning Server

QFI	QoS Flow Identifier

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

(R)AN	(Radio) Access Network

RG	Residential Gateway

RIM	Remote Interference Management

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SO-SNPN	Subscription Owner Standalone Non-Public Network

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSCAI	TSC Assistance Information

TSN	Time Sensitive Networking

TSN GM	TSN Grand Master

TSP	Traffic Steering Policy

TT	TSN Translator

TWIF	Trusted WLAN Interworking Function

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

***** Next change *****

-

5.30.2.X	Onboarding of UEs for SNPNs

5.30.2.X.1	General

Editor’s Note: 	This clause should have an overview or general description of UE onboarding for SNPNs, including an introduction to the two components below.

Onboarding of UEs for SNPNs allows the UE to access an Onboarding Network based on default UE credentials for the purpose of provisioning the UE with SNPN credentials for primary authentication and other information to enable SNPN access.

To provision SNPN credentials in a UE that does not have NPN credentials, the UE selects an SNPN as Onboarding Network and establishes a secure connection with that SNPN referred to as Onboarding SNPN (O-SNPN), see more details in clause 5.30.2.X.2.

To provision SNPN credentials in a UE that equips with a USIM configured with default PLMN credentials, the UE selects a PLMN as Onboarding Network and establishes a secure connection with that PLMN, see more details in clause 5.30.2.X.3.

After the secure connection is established, the UE is provisioned with SNPN credentials and possibly other data to enable access to an SNPN referred to as the Subscription Owner SNPN (SO-SNPN), see more details in clause 5.30.2.X.4.

Onboarding SNPN and Subscription Owner SNPN are roles taken by an SNPN. It is possible for the same network to be in both roles with respect to a specific UE.

5.30.2.X.2	Onboarding Network is an SNPN

5.30.2.X.2.1	General

5.30.2.X.2.2	Architecture

Editor’s Note:	Architectural aspects should be described in this clause.

Figure 5.30.X.2.2-1 depicts the architecture for Onboarding of UEs in an O-SNPN.

Figure 5.30.X.2.2-1: Architecture for UE Onboarding in O-SNPN

NOTE 1:	AUSF in the Onboarding Network interfaces the DCS owned by an entity external to the Onboarding Network. The functionality beyond AUSF, and other interfaces required for security, is dependent on feedback from SA WG3.

NOTE 2:	The functionality with respect to exchange of information between PVS and SO-SNPN in relation to provisioning NPN credentials from the SO-SNPN in the UE is not specified in this release.

Editor’s Note:	The Architectural of Figure 5.30.X.2.2-1 is assumed as baseline but should be confirmed by SA3.

The DCS is used to authenticate the UE based on the UE default credentials during the Onboarding procedure.

The PVS is an entity that interacts with the Subscription Owner, using mechanisms out of 3GPP scope, for the purpose of provisioning network credentials and other data in the UE to enable SNPN access.

NOTE 3:	 The DCS and PVS can be owned by an administrative entity that can be different from either the O-SNPN or SO-SNPN. The ownership of DCS and PVS is outside the scope of 3GPP.

5.30.2.x.2.3	Broadcast system information

Editor’s Note:	This clause should describe the broadcasting information for supporting onboarding in NG-RAN

NG-RAN nodes which provide access to SNPNs broadcast the information listed in clause 5.30.2.2 and when the SNPN supports UE onboarding, the NG-RAN additionally broadcasts the following information:

-	An onboarding enabled indication per SNPN.

Editor's note:	It is FFS whether Group IDs can be used for UE onboarding and in such case it is FFS if there is an additional onboarding indication per Group ID.

Editor’s note: If Group IDs can be used for UE onboarding, it is FFS whether the SO-SNPN Group IDs can be used.

5.30.2.x.2.4	UE Configuration Aspects

Editor’s Note:	This clause should describe the configuration required or assumed in the UE for supporting onboarding.

A UE enabled to support UE Onboarding, shall be pre-configured with Default UE credentials, and the UE may be pre-configured with additional information for selection of Onboarding SNPN (see clause 5.30.2.x.2.5).

Editor's note: It is FFS whether the network can update the information for selection of Onboarding SNPN and it is FFS whether the information needs to be formally listed and the format to be defined.

NOTE:	It is assumed that the UE is not pre-configured with a S-NSSAI for the purpose of UE onboarding in the O-SNPN.

5.30.2.x.2.5	Network selection

This clause applies only when the UE is in SNPN access mode.

When the UE wants to perform UE onboarding using an SNPN, the UE shall perform Onboarding SNPN (O-SNPN) selection as described below. An O-SNPN is an SNPN broadcasting the Onboarding enabled indication.

NOTE 1:	The trigger for the UE to initiate the UE Onboarding procedure is UE implementation dependent (e.g. the trigger can be that the UE, which has not yet been provisioned with credentials, is powered on or the trigger can be based on UE interaction with the user).

For automatic selection, the UE may be pre-configured with a list of O-SNPN network selection information (e.g. O-SNPN network identifiers) that helps the UE in selecting an appropriate O-SNPN.

If the UE is pre-configured with O-SNPN network selection information, the UE selects and attempts to register to an O-SNPN according to the pre-configured information as described in clause 5.30.2.x.2.6, If the registration cannot be achieved the UE may select and attempt to register to a different O-SNPN.

If there is no O-SNPN available according to the pre-configured information or the UE has no pre-configured network selection information, the UE may select and attempt to register to any available O-SNPN which broadcasts onboarding enabled indicator, according to UE implementation-specific logic. If the registration cannot be achieved the UE may select and attempt to register to a different O-SNPN.

For manual selection, the UE may provide to the user the list of all available O-SNPNs (each is identified by a PLMN ID and NID) i.e. SNPNs that broadcast the onboarding enabled indicator and the corresponding human-readable names (if available). Upon selection of an O-SNPN by the user, the UE performs registration as described in clause 5.30.2.x.5.2. If the registration cannot be achieved, the user may select a different O-SNPN if available.

Editor’s note:	It is FFS whether the pre-configuration information for onboarding SNPN selection can include more information than SNPN identifier(s) e.g. Group ID(s).

Editor’s note:	It is FFS whether the O-SNPN selection using the pre-configured network selection information is to be specified. In such case the network selection lists and the order of usage with possibly separate logic for automatic and manual selection is to be specified.

5.30.2.x.2.6	Registration for UE onboarding

When the user or UE has selected an O-SNPN according to clause 5.30.2.X.2.5, the UE establishes an RRC connection towards the NG-RAN node of the O-SNPN. The UE provides an indication in RRC Connection Establishment that the RRC connection is for onboarding as defined in TS 38.331 [28]. This indication allows the NG-RAN node to select an appropriate AMF that supports the UE onboarding procedures. The UE indicates that the O-SNPN is the selected network, and the NG-RAN node shall indicate the selected PLMN ID and NID of the O-SNPN to the AMF.

NOTE x:	As the configuration information in the UE does not include a Requested NSSAI for onboarding, the UE does not include NSSAI information in RRC.

The UE shall initiate the NAS registration procedure by sending a NAS Registration Request message with the following characteristics:

-	The UE shall set the 5GS Registration Type to the value "SNPN Onboarding".

-	The UE shall provide an Onboarding SUCI derived from an Onboarding SUPI. The Onboarding SUPI shall be unique, derived from the Default UE Credentials and encoded as a network specific identifier taking the format of a NAI (i.e. user@realm) using the RFC 7542 [20] based user identification defined in TS 23.003 [19] clause 28.7.2.

NOTE y: 	As the configuration information in the UE does not include a Requested NSSAI for onboarding, the UE does not include a Requested NSSAI in NAS signalling.

NOTE z:	The details for creating the Onboarding SUCI are specified in TS 23.003 [19] and TS 33.501 [29].

The AMF supporting UE onboarding is configured with AMF Onboarding Configuration Data that includes e.g.:

-	S-NSSAI and DNN to be used for UE onboarding;

-	Information to enable User Plane Remote Provisioning, see clause 5.30.2.x.4.

When the AMF receives a NAS Registration Request with a 5GS Registration Type set to "SNPN Onboarding", the AMF starts an authentication procedure towards the AUSF. The AMF selects an appropriate AUSF as described in clause 6.3.4.

NOTE x:	The AUSF within the O-SNPN can register at the NRF of the O-SNPN indicating its support for Onboarding or for Onboarding register certain SUPIs based on the realm of the Onboarding SUPI. Local configuration in AMF is also possible.

NOTE y: 	Authentication details are specified in TS 33.501 [29].

Upon successful authentication from AUSF, the AMF informs the UE about the result of the registration. In case the UE is not successfully authenticated, the AMF may reject the initial registration procedure for onboarding, and the UE may select a different O-SNPN to try registration. In case User Plane is used for remote provisioning, the information to enable User Plane Remote Provisioning, S-NSSAI and DNN to be used for UE onboarding should be provided to UE over registration accept message.

Editor’s Note:	Additional information provided to the UE and AMF is FFS.

In order to prevent UEs from staying registered in O-SNPN in the name of UE onboarding endlessly, the AMF needs to start the timer configured for UE onboarding to restrict usage only for UE onboarding during the Registration for UE onboarding. When the remote provisioning for SNPN credentials is finished the UE or the O-SNPN shall initiate de-registration from the O-SNPN. The O-SNPN shall also initiate de-registration when the timer configured for UE onboarding expires irrespective of the results of remote provisioning for SNPN credentials.

5.30.2.X.3	Onboarding Network is a PLMN

5.30.2.X.3.1	General

5.30.2.X.3.2	Network selection and Registration

This clause applies only when the UE is not in SNPN access mdoe.

When the UE is using PLMN credentials for accessing a PLMN as the Onboarding Network (ON), then regular network selection, as per TS 23.122 [17] and regular initial registration procedures apply, as per TS 23.502 [3] with the following clarification:

-	UE subscription data contains the S-NSSAI and DNN, or information  to be used for UE onboarding;

-	The AMF supporting UE onboarding is configured with AMF Onboarding Configuration Data that includes information to enable User Plane Remote Provisioning, see clause 5.30.2.x.4;

-	In case user plane is used for remote provisioning, the Information to enable User Plane Remote Provisioning, S-NSSAI and DNN to be used for UE onboarding should be provided to UE over registration accept message.

-	In order to prevent UEs from staying registered in PLMN in the name of UE onboarding endlessly, the AMF needs to start the timer configured for UE onboarding to restrict usage only for UE onboarding during the Registration for UE onboarding. When the remote provisioning for SNPN credentials is finished the UE or the PLMN shall initiate de-registration from the PLMN. The PLMN shall also initiate de-registration when the timer configured for UE onboarding expires irrespective of the results of remote provisioning for SNPN credentials.

NOTE X:	In this scenario it is assumed that the Onboarding Network is a PLMN.

.

5.30.2.X.4	Remote Provisioning of UEs in Onboarding Network

5.30.2.X.4.1	General

Editor’s Note:	This clause should introduce the following subclauses.

5.30.2.X.4.2	Onboarding configuration for the UE

Editor’s Note:	This clause should include the configuration data in the UE for UP provisioning.

5.30.2.X.4.3	User Plane Remote Provisioning of UEs

Editor’s Note:	This clause should specify the user plane remote provisioning procedures of UEs.

5.30.2.X.4.4	Control Plane Remote Provisioning of UEs

Editor’s Note:	If to be supported, this clause should specify the control plane remote provisioning procedures of UEs.

5.30.3.X	Onboarding of UEs for PNI-NPNs

5.30.3.X.1	General

Editor’s Note: 	This clause should have an overview or general description of UE onboarding for PNI-NPN, including an introduction to the two components below.

5.30.3.X.2	Architecture

Editor’s Note:	Architectural aspects should be described in this clause

5.30.3.X.3	Connectivity

Editor’s Note: 	This clause should indicate that the UE is assumed to have regular PLMN credentials, so the initial connectivity is granted by the PLMN.

5.30.3.X.4	Remote Provisioning of UEs for PNI-NPNs

5.30.3.X.4.1	General

Editor’s Note:	This clause should introduce the following clauses.

5.30.3.X.4.2	User Plane Remote Provisioning of UEs for PNI-NPNs

Editor’s Note:	This clause should specify the user plane remote provisioning procedures of UEs in PNI-NPNs.

5.30.3.X.4.3	Control Plane Remote Provisioning of UEs for PNI-NPNs

Editor’s Note:	This clause should specify the control plane remote provisioning procedures of UEs in PNI-NPNs.

***** End of changes *****