SA WG2 Meeting #S2-143E	S2-2100058

24 February - 09 March, 2021, Electronic meeting

3GPP TSG-CT WG3 Meeting #113e	C3-210388

# E-Meeting, 25th – 29th January 2021

Title:	LS on Support of AF instance change

Response to:

Release:	Rel-16

Work Item:	5G_URLLC

Source:	CT3

To:	SA2

Cc:

Contact person:	Yali Yan

yanyali@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

CT3 is discussing the requirement on support of AF instance change as described in clause 4.3.6.3 of TS 23.502 Notification of user plane management event procedure.

#### 4.3.6.3	Notification of User Plane Management Events

…….

4d.	When the AF receives either the Nnef_EventExposure_Notify message or the Nsmf_EventExposure_Notify message, the AF checks whether it can serve the target DNAI. If the AF instance change is needed, the AF determines the proper target AF for the target DNAI and performs the AF migration.

NOTE 1:	The determination of the target AF for the target DNAI and the AF migration to the target AF are out of the scope of this release.

4e.	The AF replies to Nnef_TrafficInfluence_Notify by invoking Nnef_TrafficInfluence_AppRelocationInfo service operation either immediately or after any required application relocation in the target DNAI is completed. AF includes N6 traffic routing details corresponding to the target DNAI. AF may reply in negative e.g. if the AF determines that the application relocation cannot be completed successfully on time. If the AF instance change happens in step 4d, the AF includes the indication of AF change, target AF ID and notification target address of the target AF in the Nnef_TrafficInfluence_AppRelocationInfo message.

…….

Moreover, as defined in clause 4.3.6.2 of TS 23.502:

#### 4.3.6.2	Processing AF requests to influence traffic routing for Sessions not identified by an UE address

…….

5.	The PCF determines if existing PDU Sessions are potentially impacted by the AF request. For each of these PDU Sessions, the PCF updates the SMF with corresponding new PCC rule(s) by invoking Npcf_SMPolicyControl_UpdateNotify service operation as described in steps 5 and 6 in clause 4.16.5.

If the AF request includes a notification reporting request for UP path change, the PCF includes in the PCC rule(s) the information required for reporting the event, including the Notification Target Address pointing to the NEF or AF and the Notification Correlation ID containing the AF Transaction Internal ID.

…….

CT3 has following questions need for further stage 2 clarification:

Q1: If the AF needs to update the target AF instance information, does the AF need to trigger the Nnef_TrafficInfluence_Update service operation to the NEF, and the update of AF influence control on traffic routing is delivered to the SMF within the PCC rule via N7?

Q2: If answer of Q1 is yes, what’s the motivation to use Nnef_TrafficInfluence_AppRelocationInfo to include the target AF instance information?

Q3: If answer of Q1 is no, how will the SMF consolidate the target AF information received by Nsmf_EventExposure_AppRelocationInfo with existing PCC rules if the information is not received via N7?

# 2	Actions

To SA2

ACTION: 	CT3 kindly asks SA2 to provide answers and update the related Stage 2 specifications accordingly, if necessary.

# 3	Dates of next TSG CT WG 3 meetings

3GPP TSG CT3#114e	24th February –5th March 2021		E-Meeting

3GPP TSG CT3#115e	14th -23rd April 2021		E-Meeting