SA WG2 Meeting #145e	S2-2105054

# May 17th – May 28th, 2021; Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2609 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Service Specific Authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current service parameters architecture (e.g. application guidance for URSP determination), if the AF or NEF include the DNN/S-NSSAI (also in the future the SSC mode) associated to the service requested do not correspond the the UE provisioned data, the AF/NEF will store anyway the service parameters in UDR and reply successfully to the request. Later, if PCF determines that the service parameters are not authorized for the UE (e.g. DNN is not part of UE’s subscription data), the URSP rules will not be updated at the UE, and it is not clear how the network should behave, e.g. who should remove the service parameters from the UDR; whether and how will AF/NEF realize of this error, what is expected from the AF point of view, etc. This is due to a lack of proper authorization and subscription data consistency check at the moment of the creation of the service parameters. NEF could query that information by consuming nudm-sdm service for access and mobility subscription data and smf selection data, but this is not a good option since NEF should not be able to retrieve tons of information which is not of interest. In addition, if further checks need to be done by UDM (e.g. the Corporate or MTC provider represented by the AF) in the future, SDM service does not support to convey such information. Additionally, if the authorization requires a validity time (e.g. similar to NIDD), Validity time is tied to ongoing requests or future requests, e.g. if the AF intends to use a new request for the same UE/DNN/S-NSSAI and same service (e.g. URSP), then NEF can skip the authorization towards UDM (if desired), this cannot be achieved using SDM service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to define a new “Service Specific authorization” produced by UDM, so that NEF may use it at the early stage of the AF request and can reject with an accurate information. This way, AF will be aware that the request is forbidden for the UE and NEF can avoid storing information in UDR which is not authorized. This new service is considered to be future proof since it can be extended for other types of authorization on a per UE basis for different types of subscription data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service Parameters are stored in UDR when they’ll be of no use and the AF will not be aware of it, which allows the AF to keep sending requests which are not authorized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.10, new clause 5.2.3.7.x1, new clause 5.2.3.7.x2, new clause 5.2.6.11.x |  |  |  |  |  |  |  |  |
| ; |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.15.6.10	Application guidance for URSP determination

This clause describes the procedures to allow an AF to provide guidance for URSP determination to 5G system via NEF. The AF may belong to the operator or to an external party. The PCF considered in this clause is in the Home PLMN as it is the PCF that determines the URSP for the UE.

NOTE 1:	The operator can negotiate with external party (typically a Corporate represented by an AF) dedicated DNN(s) and/or S-NSSAI(s) for the traffic of UE(s) of this external party. UE(s) of the external party can be identified by a group identifier.

The guidance for URSP determination may be used to provide 5GC with guidance for the URSPs depending on the UE location. This is further described in TS 23.548 [xx].

For providing guidance for URSP determination, the procedure defined in clause 4.15.6.7 is performed with the following considerations:

1)	Service Description indicates an AF Identifier.

2)	Service Parameters.

Information on the AF guidance for URSP determination which consists of a list of rules that associate an application traffic descriptor with requested features for the candidate PDU sessions the application traffic may use:

-	An application traffic descriptor, whose definition corresponds to that of the URSP Traffic Descriptors (as defined for the URSP rule in TS 23.503[4] Table 6.6.2.1-2).

-	one or more sets of Route selection parameters, each parameter may correspond to:

-	(DNN, S-NSSAI). This may be provided by the AF or determined by the NEF based on the AF Identifier when it is not provided by the AF and the AF provides only one instance of AF guidance for URSP determination

Editors Note: it is FFS whether the AF can provide SSC mode.

-	a default Route selection precedence value to be used for the application traffic when Route selection precedence with a corresponding spatial validity condition is not provided.

-	Route selection precedence with a corresponding spatial validity condition that indicates where the Route selection parameters apply. This may correspond to a geographical area (i.e. geographic zone identifier).

Editors Note: it is FFS whether the spatial validity condition may include DNAI(s). DNAI is already known and handled by PCF to build PCC rules and would be translated into identifiers that the UE can handle such as Cell Id(s) and TAI(s). Whether and how to carry out such translation is FFS

NOTE 2:	The different sets of Route selection parameters indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic descriptor. Each set is meant to apply for a specific (set of) spatial validity condition. Each set is associated with a Route selection precedence to cope with the case where multiple spatial validity conditions overlap.

If the AF provides a geographical area as spatial validity condition, it is up to the NEF or PCF to transform this information into 3GPP identifiers (e.g. TAI(s)).

Editors Note: it is FFS whether the spatial would be translated by NEF or PCF

NEF may, based on local configuration, complement missing service parameters. Additionally, based on operator’s local policy, NEF may request for service specific authorization for the service parameters for an individual UE (e.g. to authorize the Corporate or MTC provider represented by the AF and the requested DNN, S-NSSAI for the related UE) before storing the service parameters into the UDR via Nnef_ServiceParameter_Create operation. If the request is targeting a group of UEs, NEF may also request for service specific authorization for the group related data (see table 4.15.6.3b-1), i.e. the DNN, S-NSSAI associated to the group.

Editor’s Note: Support for service specific authorization for a group of UE in this procedure is FFS, since it needs to be decided if it is the operator’s reponsibilty to ensure the consistency between the group related data and the UE group members subscription data, i.e. if a group is authorized for a given DNN, it needs to be assumed that all members of the group are provisioned with the DNN and no individual UE check is required.

Editor’s Note: Support for service specific authorization for any UE in this procedure is FFS, since it needs to be decided if NEF authorizes the request based on local policy without requesting for any service specific authorization from UDM.

NOTE 3:	AF guidance for application traffic is not related with 5G VN group.

3)	a specific UE, or a group of UE(s) or any UE that the AF request may be associated with.

The usage of the AF guidance for application traffic is described in clause 6.2.4 in TS 23.548 [x].

Figure 4.15.6.10-1 shows the enhanced procedure as defined in clause 4.15.6.7 for Application guidance for URSP determination.

Figure 4.15.6.10-1: Service Specific Authorization

- The AF initiates the procedure as specified in clause 4.15.6.7.

- The NEF sends Nudm_ServiceSpecificAuthorisation_Get Service Request including S-NSSAI/DNN and service type received from AF.

- The UDM checks the list of subscribed/allowed S-NSSAI/DNNs for the UEand other service info (e.g. MTC provider is authorized for the UE).

- The UDM responds to the NEF with the service authorization result. If authorization fails (e.g. DNN is not subscribed for the UE, UE subscription does not allow to modify URSP rules dynamically by an AF or by such specific AF or MTC provider), UDM returns a negative response with an appropriate error code and the NEF rejects the request with the proper error code to inform the AF about the request not authorized.

NOTE 4:	The MTC Provider Information can be used by any type of Service Providers (MTC or non-MTC) or Corporate or External Parties for, e.g. to distinguish their different customers.

- The procedure continues as specified in clause 4.15.6.7.

Figure 4.15.6.10-2 illustrates the procedure for updating or revoking an existing Service Specific Authorization.

Figure 4.15.6.10-2: Service Specific Authorization Update procedure

- The UDM may send a Service Specific Authorization Update information using Nudm_ServiceSpecificAuthorisation_UpdateNotify Request (GPSI, S-NSSAI, DNN, Result) message to the NEF to update a user's authorization.

- The NEF sends Nudm_ServiceSpecificAuthorisation_UpdateNotify Response (cause) message to the UDM to acknowledge the authorization update.

- If the authorization is revoked, the NEF removes the service specific parameters from the UDR.

- The NEF informs the AF that the service parameters authorisation status has changed by sending Nnef_ServiceParameter_Notify Request (GPSI, TLTRI, Result) message to the AF.

- The AF responds to the NEF with Nnef_ServiceParameter_Notify Response message.

NEXT CHANGE

##### 5.2.3.7.X1	Nudm_ServiceSpecificAuthorisation_Get service operation

Service operation name: Nudm_ServiceSpecificAuthorisation_Get

Description: The consumer requests authorisation for a specific service configuration

Inputs, Required: GPSI, DNN, S-NSSAI, Service Type (e.g. AF guidance for URSP).

Inputs, Optional: MTC Provider Information, Update Notification address.

Outputs, Required: Authorisation result indication.

Outputs, Optional: Validity time.

NEXT CHANGE

##### 5.2.3.7.X2	Nudm_ ServiceSpecificAuthorisation _UpdateNotify service operation

Service operation name: Nudm_ServiceSpecificAuthorisation_UpdateNotify

Description: This service operation is used by the UDM to notify a Service Specific Authorisation Update to NF consumer.

Inputs, Required: GPSI, Result.

Inputs, Optional: DNN, S-NSSAI.

Outputs, Required: Cause (e.g. authorisation revoked).

NEXT CHANGE

##### 5.2.6.11.X	Nnef_ServiceParameter_Notify operation

Service operation name: Nnef_ServiceParameter_Notify

Description: This service operation is used by the NEF to notify a Service Parameter Authorisation Update to AF.

Inputs, Required: Transaction Reference ID, GPSI, Result.

Inputs, Optional: DNN, S-NSSAI.

Outputs, Required: Operation execution result indication.

END OF CHANGES