SA WG2 Meeting #146e	S2-2106498r05

16 - 27 Aug, 2021	(revision of S2-21xxxxx)

- Source:	Intel, Telecom Italia, Spreadtrum, Sandvine, Convida Wireless, KPN, InterDigital, Microsoft, Matrixx, KDDI, AT&T, Deutsche Telekom, Cisco, Charter Communications

- Title:	New SID: Study on System Enabler for Service Function Chaining

- Document for:	Approval

Agenda Item:	7.1

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title: 	Study on System Enabler for Service Function Chaining

## Acronym: FS_SFC

## Unique identifier:

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X | Edge Hosting Environment |
| No | X |  | X |  |  |
| Don’t know |  | X |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent and child Work Items

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure.}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 740005 | Enhancements to the Service-Based 5G System Architecture (5G_eSBA) | 5G system enhancement |
| 760066 | Management and orchestration of 5G networks and network slicing (NETSLICE) | Support of 5G network orchestration and management |
| 900016 | Stage 2 of eEDGE_5GC (eEDGE_5GC) | System support of Interaction with Edge Hosting Environment |
| 870029 | Study on enhancements of edge computing management (FS_Eedge_Mgt) | Study of the management aspect of edge computing. |
| 870030 | Study on charging aspects of Edge Computing (FS_EDGE_CH) | Study of the charging aspect of edge computing. |
| 860006 | Architecture for enabling Edge Applications (EDGEAPP) | Edge Computing Application Architecture |

## 3	Justification

In Rel-18, stage 1 service requirements in TS22.101 clause 30.1, TS22.261 clause 6.35, and TS22.115 clause 5.2.14, have been approved for the enhancement of service function chaining (SFC) for 5G networks, including aspects such as allowing third parties to request utilisation of specific service functions and service function chaining for their applications as well as management and charging of service functions and chains of service functions requested by the third parties. Considering different SFC deployments for 5G network including 5G core network and/or Edge Hosting Environment, some issues are identified based on the Rel-17 SA2 specifications:

- Currently, the SMF may be configured with the traffic steering policy related to the mechanism enabling traffic steering to the N6-LAN, DN and/or DNAIs associated with N6 traffic routing requirements provided by the AF. Based on the following service requirement in TS 22.261 clause 6.35, the definition of SFC policy is needed for the 5G network to identify/detect user plane traffic with enough granularity, perform traffic classification, and steer the traffic flow for SFC processing in different SFC deployment.

- The network operator shall be able to define and modify service function chaining policies for steering traffic on per application per UE basis through required service function chaining with ordered service functions to improve the user’s QoE.

- New northbound APIs are required for authorized AF to request to use a service function chain provided by the network operator and to request monitoring and reporting of events according to the status of the SFC processing, etc., based on service level agreement with the third party.

- .

- For continuing the SFC processing, some mechanisms and enhancement of the roaming interfaces are needed to apply the same SFC policies for UE mobility cases that result in the change of routing paths for user plane traffics requiring SFC including:

- Upon UE moves within the same operator’s network depending on the applicable SFC deployments, e.g. at the 5G core network and/or Edge Hosting Environment.

- Upon UE moves with the changes of operator’s networks, e.g. from HPLMN to VPLMN for local breakout roaming scenarios, and from the same serving network to different operator’s Edge Hosting Environments.

- For handling SFC processing based on SFC policy, the enhancement of service-based architecture will be needed for some NFs and their services, e.g. PCF, NEF, UPF, SMF, etc., as well as the corresponding interfaces and procedures for different SFC deployments. For example:

- a UPF with SFC capabilities that can support different service functions and flexible SFC configuration for a PDU session that requires different SFC processing for applications.

- the procedures to enforce both of SFC policies and traffic steering policies to traffic flows of the UEs requiring service function chains for their applications.

## Objective

Following objectives will be studied:

- Considering different SFC deployments, investigate the required coordination between the 5G Network and the Edge Hosting Environment for SFC processing.

- Providing a definition of SFC policies for the 5G network with SFC capabilities to identify/detect/classify user plane traffic with enough granularities,

- Studying the relation between the SFC policy and Traffic steering policy.

- Enabling northbound APIs for allowing an AF to request network capability exposure functionalities, e.g. request an service chain for a certain traffic flow, based on service level agreement with the third party, and reporting events according to the status of the SFC use , etc., based on service level agreement with the third party.

- Investigating solutions for applying the same SFC policies to a UE when the UE moves and results in the traffic routing path changes for SFC processing in different SFC deployments scenarios. This objective focuses on intra-PLMN scenarios.

- NOTE: This study considers only traffic handled over N6 by PSA UPF(s).

NOTE: The definition of terms in RFC 7665 may be re-used when applicable. The study targets the re-use of concepts for traffic steering e.g. defined by 3GPP (FMSS) and SFC mechanisms defined in IETF, as applicable, not at redefining them. The study will ensure that existing (per 3GPP R17) flexibility in deployments of SFC at N6 is preserved.

TU Plan: estimate 10 TUs.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 1) | Rapporteur(s) (see note 2) | For info at TSG# | For approval at TSG# | Remarks |
| New TR 23.7xy | Internal TR | Intel | TSG#nn (MM1, 2022) | TSG#nn+1 (MM2, 2022) | Including the study outputs on key issues, solutions, and architecture impacts. |

Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
“Internal TR” is intended for 3GPP internal use only whereas “External TR” may be transposed by Ops.

Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as “Remarks”.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |
| --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# |

## 6	Work item Rapporteur(s)

Ellen Liao, ellen.c.liao@intel.com

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

SA3 for security aspects. SA5 for management and charging aspects. SA6 for application layer aspects.

## 9	Supporting Individual Members

Supporting IM name

Intel

Telecom Italia

Spreadtrum

Sandvine

Convida Wireless

KPN

InterDigital

Microsoft

Matrixx

KDDI

AT&T

Deutsche Telekom

Cisco

Charter Communications

Spirent

Lenovo

Motorola Mobility

Allot