SA WG2 Meeting #147E	S2-2107117

October 18 – 22, 2021, E-Meeting

# Title:	[DRAFT] Reply LS on network slice admission control for SNPN onboarding

# Response to:	(S2-2107028/ C1-215159)

# Release:	Rel-17

# Work Item:	eNPN, eNS_Ph2

Source:	[Ericsson, will be SA2]

To:	CT1

Cc:	-

Contact Person:

Name:	Peter Hedman

E-mail Address:	peter (dot) hedman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.501, CR# 3241

1. Overall Description:

SA2 thanks CT1 for the LS on network slice admission control for SNPN onboarding.

CT1 stated:

For onboarding of UEs for SNPN, when the onboarding network is an SNPN, CT1 noted that the configuration information in the UE does not include any S-NSSAI used for onboarding, the UE does not include a Requested NSSAI to the AMF during the registration for onboarding and the S-NSSAI included in the AMF Onboarding Configuration Data is used for onboarding.

For onboarding of UEs for SNPN, when the onboarding network is a PLMN, CT1 noted that the subscription data of a UE shall contain the DNN/S-NSSAI used for onboarding.

CT1 also noted the Network Slice Admission Control Function (NSACF) has provided the admission control per network slice under eNS_Ph2, including maximum number of UEs per network slice and maximum number of PDU sessions per network slice.

CT1 asked:

CT1 would like to ask SA2 when the UE is registering or registered for onboarding services in SNPN or PLMN, whether the network slice admission control needs to be performed for slice(s) used for onboarding or not?

SA2 discussed the question and agreed that the network slice used for onboarding is exempted from admission control performed by NSAC, as per attached CR.

2. Actions:

To CT1:

ACTION: 	SA2 asks CT1 to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #148E		November 15 – 19, 2021		Elbonia

TSG-SA2 Meeting #149E	February 14 – 25, 2022		Elbonia