# 3GPP TSG SA WG2 Meeting #146-e	S2-2107238

# Electronic Meeting, 2021-10-18 – 2021-10-22

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3144 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS Node level correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | These changes align with changes made in TS 23.548 CR#0002 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inserting data keys and data subkeys for EAS deployment information. Correcting Nnef_EASDeployment service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 23.502 not aligned with TS 23.548 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.12.2.1, 5.2.6.26.1, 5.2.6.26.2, 5.2.6.26.3, 5.2.6.26.4, 5.2.6.26.5, 5.2.6.26.6, 5.2.6.26.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Reference to clause 7.X, where X will be as determined in CR#0002 of 23.548 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start Change *****

##### 5.2.12.2.1	General

The operations defined for Nudr_DM service use following set of parameters defined in this clause:

-	Data Set Identifier: uniquely identifies the requested set of data within the UDR (see clause 4.2.5).

-	Data Subset Identifier: it uniquely identifies the data subset within each Data Set Identifier. As specified in the procedures in clause 4, e.g. subscription data can consist of subsets particularised for specific procedures like mobility, session, etc.

-	Data Keys defined in Table 5.2.12.2.1-1.

For Nudr_DM_Subscribe and Nudr_DM_Notify operations:

-	The Target of Event Reporting is made up of a Data Key and possibly a Data Sub Key both defined in Table 5.2.12.2.1-1. When a Data Sub Key is defined in the table but not present in the Nudr_DM_Subscribe this means that all values of the Data Sub Key are targeted.

-	The Data Set Identifier plus (if present) the (set of) Data Subset Identifier(s) corresponds to a (set of) Event ID(s) as defined in clause 4.15.1.

An NF Service Consumer may include an indicator when it invokes Nudr_DM Query/Create/Update service operation to subscribe the changes of the data, to avoid a separate Nudr_DM_Subscribe service operation.

Depending on the use case, it is possible to use a Data Key and/or one or multiple Data sub keys to further identify the corresponding data, as defined in Table 5.2.12.2.1-1 below.

Table 5.2.12.2.1-1: Data keys

| Data Set | Data Subset | Data Key | Data Sub Key |
| --- | --- | --- | --- |
|  | Access and Mobility Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMF Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | UE context in SMF data | SUPI | PDU Session ID or DNN |
| Subscription Data (see clause 5.2.3.3.1) | SMS Management Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMS Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Session Management Subscription data | SUPI | S-NSSAI |
|  |  |  | DNN |
|  |  |  | Serving PLMN ID and optionally NID |
|  | Slice Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Group Data | Internal Group Identifier or External Group Identifier | - |
|  | Identifier translation | GPSI |  |
|  |  | SUPI | Application Port ID |
|  | Intersystem continuity Context | SUPI | DNN |
|  | LCS privacy | SUPI | - |
|  | LCS mobile origination | SUPI | - |
|  | UE reachability | SUPI | - |
|  | Group Identifier Translation | Internal Group Identifier or External Group Identifier | - |
|  | UE context in SMSF data | SUPI | - |
|  | V2X Subscription data | SUPI | - |
|  | ProSe Subscription data | SUPI | - |
|  | User consent | SUPI | Purpose |
|  | ECS Address Configuration Information (See Table 4.15.6.3d-1) | SUPI |  |
| Application data | Packet Flow Descriptions (PFDs) | Application Identifier | - |
|  | AF traffic influence request information | AF transaction internal ID |  |
|  | (See clause 5.6.7 and clause 6.3.7.2 of TS 23.501 [2]) | S-NSSAI and DNN and/or Internal Group Identifier or SUPI |  |
|  | Background Data Transfer (NOTE 3) | Internal Group Identifier or SUPI |  |
|  | Service specific information (See clause 4.15.6.7) | S-NSSAI and DNN or Internal Group Identifier or SUPI |  |
|  | EAS Deployment Information (See clause 7.X of TS 23.548 [74]) | DNN and S-NSSAI | FQDN or Application ID |
|  | AM policy influence request information (See clause 4.15.6.9.3) | AF transaction internal ID |  |
|  |  | S-NSSAI and DNN and/or Internal Group Identifier or SUPI |  |
|  | Time-Sync data (See clause 4.15.9.3 and 4.15.9.4) | DNN and S-NSSAI Internal Group ID SUPI |  |
| Policy Data | UE context policy control data (See clause 6.2.1.3 of TS 23.503 [20]) | SUPI |  |
|  | PDU Session policy control data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 of TS 23.503 [20]) |  | DNN |
|  | Policy Set Entry data (See clause 6.2.1.3 of TS 23.503 [20]) | SUPI (for the UDR in HPLMN) |  |
|  |  | PLMN ID (for the UDR in VPLMN) |  |
|  | Remaining allowed Usage data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 of TS 23.503 [20]) |  | DNN |
|  | Sponsored data connectivity profiles (See clause 6.2.1.6 of TS 23.503 [20]) | Sponsor Identity |  |
|  | Background Data Transfer data (See clause 6.2.1.6 of TS 23.503 [20]) | Background Data Transfer Reference ID. (NOTE 2) |  |
|  |  | None. (NOTE 1) |  |
|  | Network Slice Specific Control Data (See clause 6.2.1.3 of TS 23.503 [20]) | S-NSSAI |  |
| Exposure Data | Access and Mobility Information | SUPI or GPSI | PDU Session ID or |
| (see clause 5.2.12.1) | Session Management information | SUPI or GPSI | UE IP address or DNN |
| NOTE 1: Retrieval of the stored Background Data Transfer References for all ASP identifiers in the UDR requires Data Subset but no Data Key or Data Subkey(s). NOTE 2: Update of a Background Data Transfer Reference in the UDR requires a Data key to refer to a Background Data Transfer Reference as input data. NOTE 3: The Background Data Transfer includes the Background Data Reference ID and the ASP Identifier that requests to apply the Background Data Reference ID to the UE(s). Furthermore, the Background Data Transfer includes the relevant information received from the AF as defined in clause 6.1.2.4 of TS 23.503 [20]. |  |  |  |

The content of the UDR storage for (Data Set Id= Application Data, Data Subset Id = AF TrafficInfluence request information) is specified in clause 5.6.7, Table 5.6.7-1 of TS 23.501 [2]. This information is written by the NEF and read by the PCF(s). PCF(s) may also subscribe to changes onto this information.

***** Next Changes *****

#### 5.2.6.26	Nnef_EASDeployment service

##### 5.2.6.26.1	General

Service description: The Node Level EAS Deployment information management in the NEF provides functions for Node Level EAS Deployment information provision from AFand for subscribing and retrieving Node Level EAS Deployment Information in the UDR and providing EAS Deployment Information to the SMF. For related procedures see TS23.548 [74] clause 6.2.3.4.

##### 5.2.6.26.2	Nnef_EASDeployment_Create service operation

Service operation name: Nnef_EASDeployment_Create

Description: The NF consumer requests to create EAS Deployment Information.

Inputs, Required:  EAS Deployment Information.

Inputs, Optional: DNN, S-NSSAI.

Outputs, Required: Success or failure.

Outputs, Optional: None.

##### 5.2.6.26.3	Nnef_ EASDeployment _Update service operation

Service operation name: Nnef_EASDeployment _Update

Description: The NF consumer requests to Update EAS Deployment Information.

Inputs, Required:  EAS Deployment Information.

NOTE:	The Data Subkeys in the EAS Deployment information uniquelly identify the data to be updated.

Inputs, Optional: S-NSSAI, DNN.

Outputs, Required: Result.

Outputs, Optional: None.

##### 5.2.6.26.4	Nnef_ EASDeployment _Delete service operation

Service operation name: Nnef_EASDeployment _Delete

Description: The NF consumer requests to delete EAS Deployment Information.

Inputs, Required: EAS Deployment Information .

NOTE:	The Data Subkeys in the EAS Deployment information uniquelly identify the data to be deleted.

Inputs, Optional: S-NSSAI, DNNNone.

Outputs, Required: result.

Outputs, Optional: None.

##### 5.2.6.26.6	Nnef_ EASDeployment _Subscribe service operation

Service operation name: Nnef_EASDeployment _Subscribe

Description: provided by the NEF for NF consumers to explicitly subscribe the notification of changes of

EAS Deployment Information.

Inputs, Required: Notification Endpoint, DNN, S-NSSAI.

Inputs, Optional: FQDN or Application ID.

Outputs, Required: Subsrciption Correlation ID.

Outputs, Optional: None.

##### 5.2.6.26.7	Nnef_ EASDeployment _Unsubscribe service operation

Service operation name: Nnef_EASDeployment _Unsubscribe

Description: provided by the NEF for NF consumers to explicitly unsubscribe the notification of changes of

EAS Deployment Information.

Inputs, Required: Subscription Correlation ID (reference of the subscription).

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

##### 5.2.6.26.8	Nnef_ EASDeployment _Notify service operation

Service operation name: Nnef_EASDeployment _Notify

Description: Provides subscribed event information, e.g. updated EAS Deployment Information of Application Identifier, to the NF Consumer.

Inputs, Required: EAS Deployment Information.

Inputs, Optional:.

Outputs, Required: None.

Outputs, Optional: None.

***** End of Changes *****