3GPP TSG SA WG2 Meeting #147-e	S2-2107240

# Electronic Meeting, 2021-10-18 – 2021-10-22

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0006 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction related to AF request for simultaneous connectivity over source and target PSA at edge relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 6.3.4 describes how the simultaneous connectivity over Source and Target PSA may be provided at Edge relocation when Session Breakout Connectivity Model is applied (using Release 16 mechanisms) but it does not mention the network procedures that exist for the same purpose when the Distributed Anchor or Multiple Sessions connectivity models are used. In case of Distributed Anchor or Multiple Sessions, if the AF indicates the need for simultaneous connectivity, the SMF may decide (depending on UE support and other settings) to use SSC Mode #3 re-anchoring mechanisms instead of SSC Mode #2. It is also proposed to have different indications from the AF for mainaining existing PSA after the establishment of the new PSA or after detecting inactivity of the traffic concerned, respectively. |  |  |  |  |  |  |  |  |
| 6.3.4 |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add information in Clause 6.3.4 on how simultaneous connectivity over Source and Target PSA may be provided at Edge relocation when Distributed anchor or Multiple Sessions connectivity models are applied in 5GS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete description of impact of AF Request for simultaneous connectivity over Source and Target PSA at Edge Relocation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start Change *****

### 6.3.4	AF Request for Simultaneous Connectivity over Source and Target PSA at Edge Relocation

EAS relocation can make use of network capabilities that, at PSA change, provide simultaneous connectivity over the source and the target PSA during a transient period. This is described in Annex F.

AF may issue a request to the network on whether to provide simultaneous connectivity over the source and the target PSA at edge relocation. This may trigger the SMF to use a re-anchoring procedure that provides simultaneous connectivity over the source and target PSA, as described in TS 23.502 [3]:

-	For Session Breakout, in clause 4.3.5.7 for Simultaneous change of Branching Point or UL CL and additional PSA for a PDU Session. This could involve the establishment of a temporary N9 forwarding tunnel between the source UL CL and target UL CL.

-	For Distributed Anchor or Multiple PDU Sessions, in clause 4.3.5.2 for Change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions or in clause 4.3.5.3 for Change of SSC mode 3 PDU Session Anchor with IPv6 Multi-homed PDU Session.

The AF request may include the following information:

-	"Keep existing PSA" indication: If this indication is included, the SMF may decide to use a re-anchoring procedure that provides simultaneous connectivity over the source and target PSA, as described above.

NOTE: this indication cannot cause the change of the SSC Mode associated with an existing PDU session.

-	"Keep existing PSA timer": its value indicates the minimum time interval for how long the existing PSA should be maintained after the connectivity through the new PSA is established. It may overwrite the related configurable time interval in the SMF that is used in the procedures for simultaneous connectivity listed above.

-	"Keep existing PSA inactivity timer": its value indicates the minimum time interval to be considered for inactivity for the traffic described. It may overwrite the SMF configurable period of time for how long the existing PSA is to be maintained after all active traffic ceases to flow on it in the procedure in clause 4.3.5.7 of TS 23.502 [3] for Simultaneous change of Branching Point or UL CL and additional PSA for a PDU Session.

AF traffic influence request via NEF is described in TS 23.502 [3], clause 5.2.6.7. The request to PCF is described in TS 23.502 [3], clauses 5.2.5.3.2 and 5.2.5.3.3. The AF request for simultaneous connectivity over the source and the target PSA at relocation is authorized by PCF. The PCF checks whether the AF has an authority to make such a request.

Once the simultaneous connectivity over the source and the target PSA at relocation requested by AF is authorized by the PCF, the AF request including the requirements is informed to the SMF via AF influenced Traffic Steering Enforcement Control (see TS 23.503 [4] clause 6.3.1) in PCC rules.

***** End of Changes *****