3GPP TSG-SA2 Meeting #147e	S2-2107584

Electronic, October 18-22, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3319 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Paging Early Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, NR_UE_pow_sav_enh |  |  |  |  |  | Date: |  |  | 2021-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Paging Early Indication is being introduced for NR as part of Rel-17 UE power saving, see discussion in S2-2107583. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Support for NR Paging Early Indication in RRC_IDLE and RRC_INACTIVE. - The functionality introduced for WUS (E-UTRA) is re-used. - NOTE that “PEI” is used in RAN specifications, however to avoid confusion with PEI (Permanent Equipment Identifier), PEIND is proposed in this CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for Paging Early Indication over NR and related inconsistency with RAN specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4a, 5.4.9a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

### 5.4.4a	UE MM Core Network Capability handling

The UE MM Core Network Capability is split into the S1 UE network capability (mostly for E-UTRAN access related core network parameters) and the UE 5GMM Core Network Capability (mostly to include other UE capabilities related to 5GCN or interworking with EPS) as defined in TS 24.501 [47] and contains non radio-related capabilities, e.g. the NAS security algorithms, etc. The S1 UE network capability is transferred between all CN nodes at AMF to AMF, AMF to MME, MME to MME, and MME to AMF changes. The UE 5GMM Core Network Capability is transferred only at AMF to AMF changes.

In order to ensure that the UE MM Core Network Capability information stored in the AMF is up to date (e.g. to handle the situation when the USIM is moved into a different device while out of coverage, and the old device did not send the Detach message; and the cases of inter-RAT Registration Area Update), the UE shall send the UE MM Core Network Capability information to the AMF during the Initial Registration and Mobility Registration Update procedure within the NAS message.

The AMF shall store always the latest UE MM Core Network Capability received from the UE. Any UE MM Core Network Capability that an AMF receives from an old AMF/MME is replaced when the UE provides the UE MM Core Network Capability with Registration signalling.

If the UE's UE MM Core Network Capability information changes (in either CM-CONNECTED or in CM-IDLE state), the UE shall perform a Mobility Registration Update procedure when it next returns to NG-RAN coverage. See clause 4.2.2 of TS 23.502 [3].

The UE shall indicate in the UE 5GMM Core Network Capability if the UE supports:

-	Attach in EPC with Request type "Handover" in PDN CONNECTIVITY Request message (clause 5.3.2.1 of TS 23.401 [26]).

-	EPC NAS.

-	SMS over NAS.

-	LCS.

-	5G SRVCC from NG-RAN to UTRAN, as specified in TS 23.216 [88].

-	Radio Capabilities Signalling optimisation (RACS).

-	Network Slice-Specific Authentication and Authorization.

-	Parameters in Supported Network Behaviour for 5G CIoT as described in clause 5.31.2.

-	Receiving WUS Assistance Information (E-UTRA) and/or PEIND Assistance Information (NR).

-	CAG, see clause 5.30.3.3.

-	Subscription-based restrictions to simultaneous registration of network slices (see clause 5.15.12).

If a UE operating two or more USIMs, supports and intends to use one or more Multi-USIM features (see clause 5.38) in a PLMN for a USIM, it shall indicate in the UE 5GMM Core Network Capability for this USIM in this PLMN that it supports these one or more Multi-USIM features with the following indications:

-	Connection Release Supported.

-	Paging Cause Indication for Voice Service Supported.

-	Reject Paging Request Supported.

-	Paging Restriction Supported.

Otherwise, the UE with the capabilities of Multi-USIM features but does not intend to use them shall not indicate support of these one or more Multi-USIM features.

A UE not operating two or more USIMs shall indicate the Multi-USIM features are not supported.

NOTE:	It is not necessary for a UE operating two or more USIMs to use Multi-USIM features with all USIMs.

**** NEXT CHANGE ****

### 5.4.9	Wake Up Signal Assistance

### 5.4.9a	Paging Early Indication Assistance

### 5.4.9a.1	General

NOTE:	The PEIND is called "PEI" in RAN TSs. To avoid confusion with Permanent Equipment Identifier, PEIND is used in the present TS.

The RAN and UE may use a Paging Early Indication (PEIND) to reduce the UE's power consumption in RRC_IDLE and RRC_INACTIVE over NR. The RAN sends the PEIND shortly before the UE's paging occasion. The PEIND feature enables UEs to determine that in the paging occasions immediately following their PEIND occasion they will not be paged if their PEIND is not transmitted, or that they might be paged if their PEIND is transmitted (see TS 38.304 [50]).

To avoid waking up UEs in RRC_IDLE mode due to an AMF paging other UEs across multiple cells (e.g. due to frequent UE mobility and/or for low paging latency services such as VoNR), the use of PEIND by the gNB and the UE is restricted to the last used cell, i.e. the cell in which the UE's RRC connection was last released. To support this:

a)	gNBs should provide the Recommended Cells for Paging IE in the Information on Recommended Cells and RAN Nodes for Paging IE (see TS 38.413 [34]) to the AMF in the NGAP UE Context Release Complete, UE Context Suspend Request and UE Context Resume Request messages;

b)	if received during the last NGAP UE Context Release Complete or UE Context Suspend Request message, the AMF provides (without modification) the Recommended Cells for Paging as Assistance Data for Recommended Cells IE in the Assistance Data for Paging IE within the NGAP Paging message to the RAN (see also TS 38.413 [34]); and

c)	the AMF shall delete (or mark as invalid) the Information On Recommended Cells And RAND nodes For Paging when a new NGAP association is established for the UE.

In the NGAP Paging message, the last used cell ID is sent in the Assistance Data for Recommended Cells IE in the Assistance Data for Paging IE (see TS 38.413 [34]). When receiving an NGAP Paging message for a PEIND-capable UE that also includes the Assistance Data for Recommended Cells IE then a PEIND-capable gNB shall only broadcast the PEIND on the cell that matches the last used cell ID.

To avoid waking up UEs in RRC_INACTIVE mode due to RAN paging other UEs across a RAN paging area, the use of PEIND by the gNB is restricted to the last used cell, i.e. the cell in which the UE last entered RRC_INACTIVE mode, as specified in TS 38.304 [50].

### 5.4.9a.2	PEIND sub-grouping

To support the Paging Early Indication (PEIND), the PEIND Assistance Information is used by the gNB to help determine the PEIND sub-group used when paging the UE (see TS 38.300 [27]).

The content of the PEIND Assistance Information consists of [FFS].

The UE may in the Registration Request message provide its capability to support receiving PEIND Assistance Information. If PEIND Assistance Information is supported by the UE, then the UE in the Registration Request message may provide [FFS: information]. The AMF may use the [FFS: information], local configuration and/or previous statistical information for the UE, when determining the PEIND Assistance Information. If the UE supports PEIND Assistance Information, the AMF may assign PEIND Assistance Information to the UE, even when the UE has not provided the [FFS: information].

Editor’s Note:	[FFS] will be filled in once clarification is received from RAN2.

If the AMF has determined PEIND Assistance Information for the UE, the AMF provides it to the UE in every Registration Accept message. The AMF stores the PEIND Assistance Information parameter in the MM context and provides it to the gNB when paging the UE. In addition, in order to support PEIND sub-grouping for UEs in RRC_INACTIVE mode, the AMF provides the PEIND Assistance Information to NG-RAN as part of the RRC Inactive Assistance Information upon Registration Accept if the AMF does not release the signalling connection, see TS 23.502 [3] clause 4.2.2.2.2.

UE and AMF shall not signal PEIND Assistance Information in Registration Request, Registration Accept messages when the UE has an active emergency PDU session.

**** END OF CHANGES ****

**** FOR INFORMATION ****

### 5.4.9	Wake Up Signal Assistance

#### 5.4.9.1	General

The RAN and UE may use a Wake Up Signal (WUS) to reduce the UE's idle mode power consumption. The RAN sends the WUS shortly before the UE's paging occasion. The WUS feature enables UEs to determine that in the paging occasions immediately following their WUS occasion they will not be paged if their WUS is not transmitted, or that they might be paged if their WUS is transmitted (see TS 36.304 [52]).

To avoid waking up UEs due to an AMF paging other UEs across multiple cells (e.g. due to frequent UE mobility and/or for low paging latency services such as VoLTE), the use of WUS by the ng-eNB and the UE is restricted to the last used cell, i.e. the cell in which the UE's RRC connection was last released. To support this:

a)	ng-eNBs should provide the Recommended Cells for Paging IE in the Information on Recommended Cells and RAN Nodes for Paging IE (see TS 38.413 [34]) to the AMF in the NGAP UE Context Release Complete, UE Context Suspend Request and UE Context Resume Request messages;

b)	if received during the last NGAP UE Context Release Complete or UE Context Suspend Request message, the AMF provides (without modification) the Recommended Cells for Paging as Assistance Data for Recommended Cells IE in the Assistance Data for Paging IE within the NGAP Paging message to the RAN (see also TS 38.413 [34]); and

c)	the AMF shall delete (or mark as invalid) the Information On Recommended Cells And RAND nodes For Paging when a new NGAP association is established for the UE.

In the NGAP Paging message, the last used cell ID is sent in the Assistance Data for Recommended Cells IE in the Assistance Data for Paging IE (see TS 38.413 [34]). When receiving an NGAP Paging message for a WUS-capable UE that also includes the Assistance Data for Recommended Cells IE then a WUS-capable ng-eNB shall only broadcast the WUS on the cell that matches the last used cell ID.

#### 5.4.9.2	Group Wake Up Signal

To support the Wake Up Signal (WUS), the WUS Assistance Information is used by the ng-eNB to help determine the WUS group used when paging the UE (see TS 36.300 [30]).

The content of the WUS Assistance Information consists of the paging probability information. The paging probability information provides a metric on the probability of a UE receiving a paging message based on, e.g. statistical information.

The UE may in the Registration Request message provide its capability to support receiving WUS Assistance Information. If WUS Assistance Information is supported by the UE, then the UE in the Registration Request message may provide the additional UE paging probability information. The AMF may use the UE provided paging probability, local configuration and/or previous statistical information for the UE, when determining the WUS Assistance Information. If the UE supports WUS Assistance Information, the AMF may assign WUS Assistance Information to the UE, even when the UE has not provided the additional UE paging probability information.

If the AMF has determined WUS Assistance Information for the UE, the AMF provides it to the UE in every Registration Accept message. The AMF stores the WUS Assistance Information parameter in the MM context and provides it to the ng-eNB when paging the UE.

UE and AMF shall not signal WUS Assistance Information in Registration Request, Registration Accept messages when the UE has an active emergency PDU session.