SA WG2 Meeting #S2-147E	S2-2107662

18 October – 22 October 2021, Electronic, Elbonia	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3209 | rev | <-> | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the Time Synchronization exposure procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the Notes of clause 4.15.9.1, it said “The AF can use either the procedure for configuring the (g)PTP instance in 5GS as described in clause 4.15.9.3 or the procedure for controlling the 5G access stratum time distribution as described in clause 4.15.9.4 for a particular UE. The procedures are not intended to be used in conjunction.” But in the clause 4.15.9.4 step 5, it describe two trigger/case: - AF request to influence the 5G access stratum time distribution - PTP instance activation, modification, or deactivation in the TSCTSF. They conflict with each other. For the AF request to influence the 5G access stratum time distribution case, the TSCTSF behavoir is missing. For the PTP instance activation, modification, or deactivation in the TSCTSFcase, it should be SUPI (with DNN/S-NSSAI). “If the TSCTSF receives multiple time time synchronization error budgets for a given UE” shall be applied for both case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the restriction (not intended to be used in conjunction) in the Note. Clarify the TSCTSF behavoir in AF request to influence the 5G access stratum time distribution. In the PTP instance activation, modification, or deactivation in the TSCTSF, The TSCTSF associates the 5G access stratum time information parameters with a SUPI (with DNN, S-NSSAI) when storing them to the UDR. Clarify “TSCTSF receives multiple time time synchronization error budgets for a given UE” is applied for both case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is confliction betyween clasue 4.15.9.1 and 4.15.9.4. The TSCTSF behavoir is missing in the AF request to influence the 5G access stratum time distribution case. The TSCTSF behavoir in “PTP instance activation, modification, or deactivation” is not correct. And the “multiple time time synchronization error budgets for a given UE” is not only for “PTP instance activation, modification, or deactivation” case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.9.1, 4.15.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 4.15.9	Time Synchronization exposure

#### 4.15.9.1	General

Time synchronization exposure allows an AF to configure time synchronization in 5GS. Depending on the time distribution method to use for the service (e.g. (g)PTP or 5G clock sync), the AF may require retrieving 5GS time synchronization capabilities prior to sending the time synchronization service request as described in clause 4.15.9.2. For (g)PTP operation, the Time synchronization service allows an AF to subscribe to the UE availability for time synchronization service and to configure the (g)PTP instance in 5GS as described in clause 4.15.9.3. For 5G access stratum based time distribution, the AF can influence the 5G access stratum time distribution as described in clause 4.15.9.4. The time synchronization exposure is provided by NEF that uses the service provided by TSCTSF. The AF that is part of operator's trust domain may invoke the services directly with TSCTSF.

NOTE:	The AF can use either the procedure for configuring the (g)PTP instance in 5GS as described in clause 4.15.9.3 or the procedure for controlling the 5G access stratum time distribution as described in clause 4.15.9.4 for a particular UE.

Next CHANGE

#### 4.15.9.4	Procedures for management of 5G access stratum time distribution

The AF can use the procedure to activate, update or delete the 5G access stratum time distribution for one UE, group of UEs or any UE using the DNN and S-NSSAI.

The Nnef_TimeSynchronization_ASTICreate and Nnef_TimeSynchronization_ASTIUpdate request may contain the parameters as described in Table 4.15.9.4-1.

Table 4.15.9.4-1: Description of 5G access stratum time distribution parameters

| Parameter | Description |
| --- | --- |
| 5G access stratum time distribution (enable, disable) | Indicates that the access stratum time distribution via Uu reference point should be activated or deactivated for the associated UE identities. |
| Time synchronization error budget | Indicates the time synchronization error budget for the time synchronization service (as described in clause 5.27.1.8.1 of TS 23.501 [2]). [optional] |
| Temporal Validity Condition | Indicates start-time and stop-time attributes that describe the time period when the time synchronization service is active. [optional] |

Figure 4.15.9.4: Management of 5G access stratum time information

1.	AM Policy Association establishment as described in clause 4.16.1.

2.	The PCF subscribes to policy data related to time synchronization (Data Set = Application Data; Data Subset = Time-Sync data, Data Key = S-NSSAI and DNN and/or Internal Group Identifier or SUPI).

3a.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	To create a new request, the AF provides time synchronization access stratum time information data to the NEF using the Nnef_TimeSynchronizationRTI_Create service operation (together with the AF identifier and potentially further inputs as specified in table 4.15.9.4-1), including a target (one UE identified by SUPI or GPSI, a group of UEs identified by an External Group Identifier, or all UEs connected to a combination of DNN/S-NSSAI). The request may also contain a temporal validity condition. The NEF maps the External Group Identifier to an Internal Group Identifier and any GPSI to a SUPI.

-	To update or remove an existing request, the AF invokes an Nnef_TimeSynchronizationRTI_Update or Nnef_TimeSynchronizationRTI_Delete service operation providing the corresponding AF Transaction Id.

The AF that is part of operator's trust domain may invoke the services directly with TSCTSF.

NOTE 1:	Steps 1, 2, and 3 can occur in any order.

4.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	The NEF authorizes the request. After successful authorization, the NEF invokes the Ntsctsf_TimeSynchronization_RTICreate service operation with the corresponding TSCTSF.

5.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	The TSCTSF determines whether the targeted UE is part of a PTP instance in 5GS, if so the TSCTSF rejects the request. The TSCTSF associates the 5G access stratum time information parameters with a SUPI (without DNN, S-NSSAI) when storing them to the UDR.

(When the procedure is triggered by PTP instance activation, modification, or deactivation in the TSCTSF):

-	If time synchronization error budget is provided by the AF, the TSCTSF may use the PTP port state of each DS-TT to determine an error budget for corresponding SUPIs that are part of the PTP instance. The TSCTSF associates the 5G access stratum time information parameters with a SUPI (with DNN, S-NSSAI) when storing them to the UDR.

For both case, if the TSCTSF receives multiple time time synchronization error budgets for a given UE, then the TSCTSF picks the most stringent budget. The TSCTSF calculates the Uu time synchronization error budget as described in clause 5.27.1.9 of TS 23.501 [2]. Then, TSCTSF stores the calculated Uu time synchronization error budget together with the time distribution indication as Time Sync Data Subset in the UDR.

6.	The UDR informs the TSCTSF about the result of the operation of step 5.

7.	The UDR notifies the PCF(s) that have a matching subscription (from step 2) about the data stored, updated, or removed in step 5. If matching entries already existed in the UDR when step 5 is performed, this shall be immediately reported to the PCF.

8.	If the access stratum time information parameters in UDR are associated with a DNN/S-NSSAI for the SUPI (i.e. the parameters are valid only for a DNN/S-NSSAI), the PCF for the UE may discover the PCF for the PDU Session using Nbsf_Management_Subscribe with SUPI and (DNN, S-NSSAI) as parameters.

9.	Upon PDU Session establishment, if the PCF of the UE has subscribed for discovery of the PCF for the PDU Session from the BSF, the BSF provides to the PCF of the UE the identity of the PCF for the PDU Session and the UE address for the requested SUPI and (DNN, S-NSSAI) combination via an Nbsf_Management_Notify operation. If a matching entry already exists in the BSF when step 8 is performed, this shall be immediately reported to the PCF for the UE.

10.	The PCF takes a policy decision and then it may initiate an AM Policy Association Modification procedure initiated by the PCF for the UE may be performed as described in clause 4.16.2.2. As part of this, the time distribution indication and the Uu time synchronization error budget are provided to NG-RAN. Based on this, NG-RAN provides the 5GS access stratum time to the UE according to the Uu time synchronization error budget as provided by the TSCTSF (if supported by UE and NG-RAN).

NOTE 2:	This release of the specification assumes that deployments ensure that the targeted UEs and the NG-RAN nodes serving those UEs support Rel-17 propagation delay compensation as defined in TS 38.300 [30].

11.	The TSCTSF responds the AF with the Ntsctsf_TimeSynchronization_ASTICreate response, including a reference to the time synchronization service configuration.

12.	The NEF informs the AF about the result of the Nnef_TimeSynchronization_ASTI operation performed in step 3.

END OF CHANGES