3GPP TSG-SA WG2 Meeting #147e	S2-2107968

# Elbonia,  Oct 18 - 22, 2021	(revison of S2-2107665r03)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0026 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction and simplification on UAV-C replacement procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2021-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. the steps related to C2 pairing policy update (Step 4-7) can be replaced by a single reference to the steps in 5.2.5.4.1. 2, Step 8 using UAV re-authentication procedure is wrong. UAV re-authorization procedure should be used instead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Replace steps 4-7 with reference to Figure 5.2.5.4.1-1. 2. Change Step 8 to UAV re-authorication procedure in Figure 5.2.4.3-1. 3. The EN at the beginning of 5.2.8 is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong procedure is used and the new C2 authorization information is not delivered to the UAV. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st change * * * *

### 5.2.8	UAV Controller Replacement

### 5.2.8.1	UAV controller replacement in 5GS

If USS determines that UAV controller replacement is required the USS invokes an Nnef_AFsessionWithQoS_Update service operation to the UAS NF including in the request authorization information (i.e. new pairing information). NEF authorizes the request from the USS followed by interacting with PCF triggering a Npcf_PolicyAuthorization_Update request and provides relevant parameters to the PCF. The PCF uses the information provided by the NEF to derive new PCC rules to allow C2 communication between the UAV and the new UAV controller.

The procedure for UAV-C replacement is as follows:

Figure 5.2.8-1: UAV-C replacement procedure

0.	A UAV has already established user plane connectivity for C2 operation. The USS determines that the UAV-C controlling the UAV needs to be replaced (e.g. if the UAV is misbehaving).

1.	The USS initiates the UAV-C replacement by invoking the Nnef_AFSessionWithQoS_Update request including USS Identity/AF Identifier, Transaction Reference ID, UAV-UAVC Pairing info/Flow description(s) (including the UAV-C IP address), QoS reference. See step 1 in TS 23.502 [3] clause 4.15.6.6a, AF session with required QoS update procedure.

2.	NEF authorizes the request from the USS followed by interacting with PCF triggering a Npcf_PolicyAuthorization_Update request and provides relevant parameters to the PCF. PCF determines whether the request is authorized and if the requested QoS is allowed. PCF informs NEF if the request is accepted by invoking Npcf_PolicyAuthorization_Update Response. See Steps 2 - 4 in TS 23.502 [3] figure 4.15.6.6.6a-1.

3.	NEF sends a Nnef_AFsessionWithQoS_Update response message (Transaction Reference ID, Result) to the USS. Result indicates whether the request is granted or not. See step 5 in TS 23.502 [3] figure 4.15.6.6.6a-1.

4.	 Steps 4 - 7 in Figure 5.2.5.4.1-1.

5.	USS invokes the UAV Re-authorization procedure in Figure 5.2.4.3-1 to deliver the new pairing information to the UE. The USS includes the 3GPP UAV ID, the IP address of the PDU session, the C2 authorization payload ( that includes the IP address of the UAV-C) and security information which is further forwarded to the UE.

* * * * End of changes * * * *