SA WG2 Meeting #147E (e-meeting)	S2-2108530

# 18 - 22 October, 2021, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3301 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of selecting a single TAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Arch |  |  |  |  |  | Date: |  |  | 2021-11-02 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the LS from RAN2 (2107015), SA2 should work on the solution for selecting a single TAC from the list of multiple TACs per PLMN indicated by the AS layer. Thus the specification needs to be updated to reflect this RAN2 conclusion. As UE has no knowledge of the correlation between geo area and TAs, the UE can only select a TA by its implementation, e.g. select a TA within its registration area if possible. Moreover, we think the AMF shall take UE location into account to enforce mobility restriction.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying that UE selects a single TAC from the broadcasted TAC list based on its implementation; The AMF takes UE location into account to enforce mobility restriction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Solution for multiple TACs per cell is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

5.4.11.6	Support of Mobility Registration Update

A radio cell for NR satellite access may indicate support for one or more TACs for each PLMN. A UE that is registered with a PLMN may access a radio cell and select a single TAC from the TAC list indicated by AS layer based on it implementation, e.g., a TAC within its Registration Area if possible. The UE does not need to perform a Mobility Registration Update procedure as long as at least one supported TAC for the RPLMN or equivalent to the RPLMN broadcast in the radio cell is part of the UE Registration Area. A UE shall perform a Mobility Registration Update procedure when accessing a radio cell where none of the supported TACs for the RPLMN or equivalent to the RPLMN are part of the UE Registration Area.

During the Mobility Registration Update procedure or Service Request procedure initiated in the radio cell with more than one TACs, the NG-RAN has to report all TACs to the AMF. If the TAC list reported by the gNB contains a TAC within forbidden area or non-allow area of the UE, the AMF shall verify UE location to enforce mobility restriction.

* * * End of changes * * *