3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108727

# Elbonia, November 11 – 15, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3281 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.502 Update procedure of Registration with Onboarding SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Firstly, since there are two different Onboarding architectures which are DCS hosting AUSF/UDM and DCS hosting AAA server, the procedure of registration with Onboarding SNPN shall be updated correspondingly. - Secondly, when DCS includes AAA server, the AUSF in SNPN skips UDM selection which is different from regular registration. Therefore, the AUSF shall be awared that the UE is performing Onboarding registration rather than regular registration. According to the agreed CR S2-2107185, AUSF skips the UDM selection based on local configuration. However, local configuration is not clear enough, which shall be specified. A simpler way is to use the existing information such as the identification of DCS hosting AAA server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Provide separate call flow for each architecture since different entities are involved in different Onboarding architectures. - Specify the local configuration in AUSF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure of registration with Onboarding SNPN is not aligned with TS 23.501 and the descirption is not clear enough. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.2.2.2.4	Registration with Onboarding SNPN

This clause specifies how a UE can register to an ON-SNPN for provisioning the UE with SO-SNPN credentials and other information to enable SNPN access as defined in clause 5.30.2.10 of TS 23.501 [2].

The Registration procedure for onboarding SNPN shall be supported as specified in clause 4.2.2.2.2 with the following changes to the steps in the call flow represented in Figure 4.2.2.2.2-1 when DCS hosting AAA server and Figure 4.2.2.2.2-2 when DCS hosting AUSF/UDM.

Figure 4.2.2.2.4-1: UE Registration with ON-SNPN when DCS hosting AAA server

1.	UE to NG-RAN: AN parameters shall include Onboarding indication if the UE is accessing 5GS for Onboarding. The registration type "SNPN Onboarding" indicates that the UE wants to perform SNPN Onboarding Registration (i.e. allows the UE to access an ON-SNPN for the purpose of provisioning the UE with SO-SNPN credentials). For SNPN Onboarding Registration, a SUCI generated from a SUPI derived from Default UE Credentials shall be included as described in clause 5.30.2.10.2.6 of TS 23.501 [2].

If the UE has registered in the ON-SNPN for onboarding, it can perform a Mobility Registration Update, or a Periodic Registration Update as specified in clause 4.2.2.2.2. If the onboarding registered UE wants to perform a Mobility Registration Update the AN parameters shall also include an Onboarding indication that the UE is registered for onboarding.

NOTE:	When the UE is performing Registration for Onboarding to an ON-SNPN, the UE does not include a Requested NSSAI as the UE is not pre-configured with a S-NSSAI for the purpose of UE onboarding in the ON-SNPN.

2.	Based on the Onboarding indication in step 1, the NG-RAN selects an AMF as described in clause 6.3.5 of TS 23.501 [2].

3.	NG-RAN to AMF: The N2 message contains the Registration Request as described in step 1.

4.	[Conditional] new AMF to old AMF: Namf_Communication_UEContextTransfer (complete Registration Request).

5.	[Conditional] old AMF to new AMF: Response to Namf_Communication_UEContextTransfer (SUPI, UE Context in AMF (as per Table 5.2.2.2.2-1)). Once the registration is completed successfully, the new AMF may start the implementation specific timer for when to deregister the onboarding registered UE if the UE context contains the indication that the UE is registered for onboarding.

6-7.	Skipped.

8.	When the AMF receives a NAS Registration Request with the 5GS Registration Type set to "SNPN Onboarding", the AMF applies locally configured AMF Configuration Data for Onboarding in order to restrict UE network usage to only onboarding and stores in the UE Context in AMF an indication that the UE is registered for onboarding. The AMF selects an AUSF as described in clause 5.30.2.10.2.6 of TS 23.501 [2]. Based on ON-SNPN policies, the AMF may start an implementation specific deregistration timer configured for UE Onboarding as described in TS 23.501 [2].

9.	Based on local configuration (e.g. identification of DCS hosting AAA server), when the realm part of the Onboarding SUCI corresponds to the DCS hosting AAA server, the AUSF skips the UDM selection as specified in clause 5.30.2.10.2.2 of TS 23.501 [2]. The authentication is performed as described in TS 33.501 [15]. The AUSF uses an NSSAAF to send the Onboarding SUCI and Default UE credentials received from the UE towards the DCS, which authenticates the UE based on received data from AUSF. During authentication procedure DCS may provide PVS FQDN or PVS IP address for the UE to the AUSF. AUSF provides PVS FQDN or PVS IP address to the AMF.

10.	[Conditional] new AMF to old AMF: Namf_Communication_RegistrationStatusUpdate.

11.	[Conditional] AMF to UE: Identity Request/Response (PEI).

If the PEI was not provided by the UE, the Identity Request procedure is initiated by AMF sending an Identity Request message to the UE to retrieve the PEI.

12.	Optionally the new AMF initiates ME identity check by invoking the N5g-eir_EquipmentIdentityCheck_Get service operation (see clause 5.2.4.2.2).

The PEI check is performed as described in clause 4.7.

13-20.	Skipped.

21.	AMF to UE: The AMF sends a Registration Accept message to the UE indicating that the Registration Request for Onboarding SNPN has been accepted. The Allowed NSSAI containing the S-NSSAI from the AMF Onboarding Configuration Data is included in the N2 message to NG-RAN.

21b.	Skipped.

22.	UE to AMF: The UE sends a Registration Complete message to the AMF.

23-25.	Skipped.

Figure 4.2.2.2.4-2: UE Registration with ON-SNPN when DCS hosting AUSF/UDM

The UE Registration with ON-SNPN when DCS hosting AUSF/UDM is the same procedure with the UE Registration with ON-SNPN when DCS hosting AAA server with the following differences:

-	In step 8, the AMF selects an AUSF in DCS domain as described in clause 5.30.2.10.2.6 of TS 23.501 [2].

-	In step 9, the AUSF in DCS domain performs UDM selection. The authentication is performed as described in TS 33.501 [15]. The AMF sends the Onboarding SUCI and Default UE credentials received from the UE towards the AUSF in DCS domain, which authenticates the UE based on received data from AMF.

# * * * * End of changes * * * *