SA WG2 Meeting #148E e-meeting	S2-2108933

# 15 – 19 November 2021, Elbonia	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3456 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications regarding TSCTSF discovery and selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. In the current specification the PCF shall notify the TSCTSF for any PDU Session with a matching DNN/S-NSSAI, using the notification address retrieved from the UDR (when local configuration of the TSCTSF notification address is not used). This shall be done regardless the PDU Session ever actually uses any of the QoS- or time-sync services. This means inefficient use of TSCTSF, since the DNN/S-NSSAI may be authorized for a large number of users, but very few PDU Sessions actually use the time-sync or AF-requested QoS service at certain point of time. The necessity to notify the PCF should be applicable only for Time-sync related service operations where 5GS operates as a PTP instance (because a single TSCTSF should be a central controller for all PDU Sessions that are part of the PTP instance). 2. The current specification assumes that a single TSCTSF NF instance is selected per DNN/S-NSSAI. When TSCTSF is discovered from the NRF, as multiple TSCTSF NF instances can register to the NRF for a given DNN/S-NSSAI, it is necessary to store the association of the selected TSCTSF NF instance and the DNN/S-NSSAI temporarily (into UDR). Instead, if it is assumed that only a single TSCTSF NF instance (or NF Set) may register to the NRF at a time, then such storing of association (in the UDR) is not necessary. 3. In the current specification it is assumed that ASTI and PTP instance related time-sync service operations shall be performed by the same TSCTSF NF instance for a given DNN/S-NSSAI. It may not be possible to select the same TSCTSF instance for these operations, because DNN/S-NSSAI may not be known with the ASTI related service operations; the ASTI service request is targeted to a SUPI or GPSI or group of UEs, and the UE may not have a PDU Session, therefore DNN/S-NSSAI is irrelevant. Or it is possible that the targeted UEs are using different DNN/S-NSSAIs served by different TSCTSF instances. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarify that the TSCTSF notification address that is stored to the UDR is used only for time-sync PTP service. This means the PCF does not need to notify the TSCTSF for a PDU Session, unless the TSCTSF has received a prior AF-request for time-sync capability subscription. 2. Remove the storing of TSCTSF NF instance in the UDR. Note that the storing of TSCTSF notification address for a time-sync capability subscription is still necessary so that the PCF can notify the TSCTSF for a PDU Session matching to the time-sync capability subscription. 3. Separate the ASTI-related service operations into a separate service. This means that a different TSCTSF instance may be selected for ASTI and time-sync services. 4. Clarify that TSCTSF deletes the TSCTSF Notification Target Address from the UDR after the subscriptions for "5GS and UE availability and capabilities for time synchronization service" are cancelled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inefficient use of TSCTSF as it is often invoked without any reason. TSCTSF discovery may fail, as invoking ASTI service operation may result a wrong TSCTSF instance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.24, 7.2.8, 7.2.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

### 6.3.24	TSCTSF Discovery

The NF consumers (e.g. NEF, AF) may utilize the NRF to discover TSCTSF instance(s) unless TSCTSF information is available by other means, e.g. locally configured in NF consumers. When discovering an NF service instance for Ntsctsf_TimeSynchronization service, the NRF provides NF service profile(s) of TSCTSF instance(s) belonging to single TSCTSF Set for a given DNN/S-NSSAI to the NF consumers.

The following factors may be considered for TSCTSF discovery and selection:

-	DNN and S-NSSAI;

-	GPSI or External Group Identifier;

-	SUPI or Internal Group ID.

If the TSCTSF is discovered from NRF for the Ntsctsf_TimeSynchronization service, it shall be ensured that the same TSCTSF NF instance or NF Set is selected in all NF consumers (e.g. NEF, AF) for the given DNN and S-NSSAI.

If the TSCTSF address is configured for the Ntsctsf_TimeSynchronization service, it shall be ensured that the same TSCTSF NF instance or NF Set is configured in all NF consumers (e.g. NEF, AF) for the given DNN and S-NSSAI, and a corresponding Notification Target Address is configured to the PCFs.

NOTE: it is assumed that there is only one TSCTSF Set for a given DNN/S-NSSAI for the Ntsctsf_TimeSynchronization service.

When the TSCTSF is discovered from NRF for the Ntsctsf_TimeSynchronization_ service, the discovered and selected TSCTSF shall store the Notification Target Address to the UDR for the given DNN and S-NSSAI. The PCF serving PDU Sessions for the given DNN and S-NSSAI shall  use the Notification Target Address as received from the UDR to notify the TSCTSF for a PDU Session. The TSCTSF deletes the Notification Target Address from the UDR after subscriptions for "5GS and UE availability and capabilities for time synchronization service" are cancelled. .

## ---Start of the 2nd Change---

### 7.2.8	NEF Services

The following NF services are specified for NEF:

Table 7.2.8-1: NF Services provided by NEF

| Service Name | Description | Reference in TS 23.502 [3] or other TS |
| --- | --- | --- |
| Nnef_EventExposure | Provides support for event exposure. | 5.2.6.2 |
| Nnef_PFDManagement | Provides support for PFDs management. | 5.2.6.3 |
| Nnef_ParameterProvision | Provides support to provision information which can be used for the UE in 5GS. | 5.2.6.4 |
| Nnef_Trigger | Provides support for device triggering. | 5.2.6.5 |
| Nnef_BDTPNegotiation | Provides support for background data transfer policy negotiation and optionally notification for the renegotiation. | 5.2.6.6 |
| Nnef_TrafficInfluence | Provide the ability to influence traffic routing. | 5.2.6.7 |
| Nnef_ChargeableParty | Requests to become the chargeable party for a data session for a UE. | 5.2.6.8 |
| Nnef_AFsessionWithQoS | Requests the network to provide a specific QoS for an AS session. | 5.2.6.9 |
| Nnef_MSISDN-less_MO_SMS | Used by the NEF to send MSISDN-less MO SM to the AF. | 5.2.6.10 |
| Nnef_ServiceParameter | Provides support to provision service specific information. | 5.2.6.11 |
| Nnef_APISupportCapability | Provides support for awareness on availability or expected level of a service API. | 5.2.6.12 |
| Nnef_NIDDConfiguration | Used for configuring necessary information for data delivery via the NIDD API. | 5.2.6.13 |
| Nnef_NIDD | Used for NEF anchored MO and MT unstructured data transport. | 5.2.6.14 |
| Nnef_SMContext | Provides the capability to create, update or release the SMF-NEF Connection. | 5.2.6.15 |
| Nnef_AnalyticsExposure | Provides support for exposure of network analytics. | 5.2.6.16 |
| Nnef_UCMFProvisioning | Provides the ability to configure the UCMF with dictionary entries consisting of UE manufacturer-assigned UE Radio Capability IDs, the corresponding UE radio capabilities, the corresponding UE Radio Capability for Paging and the (list of) associated IMEI/TAC value(s) via the NEF. The UE radio capabilities the NEF provides for a UE radio Capability ID can be in TS 36.331 [51] format, TS 38.331 [28] format or both formats. Also used for deletion (e.g. as no longer used) or update (e.g. to add or remove a (list of) IMEI/TAC value(s) associated to an entry) of dictionary entries in the UCMF. | 5.2.6.17 |
| Nnef_ECRestriction | Provides support for queuing status of enhanced coverage restriction, or enable/disable enhanced coverage restriction per individual UEs. | 5.2.6.18 |
| Nnef_ApplyPolicy | Provides the capability to apply a previously negotiated Background Data Transfer Policy to a UE or a group of UEs. | 5.2.6.19 |
| Nnef_Location | Provides the capability to deliver UE location to AF. | 5.2.6.21 |
| Nnef_AMInfluence | Provides the ability to influence access and mobility management related policies for one or multiple UEs. | 5.2.6.22 |
| Nnef_AMPolicyAuthorization | Provides the ability to provide inputs that can be used by the PCF for deciding access and mobility management related policies. | 5.2.6.23 |
| Nnef_AKMA | AKMA Application Key derivation service. | TS 33.535 [124] |
| Nnef_SliceStatus | Provides network slice status information related to the number of the UEs registered with a network slice or the number of PDU Sessions established on a network slice. | 5.2.6.24 |
| Nnef_EASDeployment | EAS deployment service. | 5.2.6.26 |
| Nnef_TimeSynchronization | Provides support for time synchronization service based on (g)PTP as described in clause 4.4.8.3. Allows the NF consumer to subscribe for the UE and 5GC capabilities for (g)PTP. Allows the NF consumer to configure the UEs and the 5GC for the (g)PTP service. | 4.15.9 |
| Nnef_QoSand TSCAssistance | Allows the NF consumer to provide QoS parameters and information to create TSC Assistance Container. | 4.15.6.6 |
| Nnef_ASTI | Allows the NF consumer to configure the UEs for the 5G access stratum time distribution method. | 4.15.9 |

## ---Start of the 3rd Change---

### 7.2.26	TSCTSF Services

The following NF services are specified for TSCTSF.

Table 7.2.26-1: NF Services provided by TSCTSF

| Service Name | Description | Reference in TS 23.502 [3] |
| --- | --- | --- |
| Ntsctsf_TimeSynchronization | Provides support for time synchronization service based on (g)PTP as described in clause 4.4.8.3. Allows the NF consumer to subscribe for the UE and 5GC capabilities for (g)PTPservice. Allows the NF consumer to configure the UEs and the 5GC for the (g)PTP. | 4.15.9 |
| Ntsctsf_QoSand TSCAssistance | Allows the NF consumer to provide QoS parameters and information to create TSC Assistance Container. | 4.15.6.6 |
| Ntsctsf_ASTI | Allows the NF consumer to configure the UEs for the 5G access stratum time distribution method. | 4.15.9 |

## ---End of Changes---