SA WG2 Meeting #149e	S2-2200132

# February 14th – 25th, 2022; Elbonia

Source: 	China Telecom

Title: 	FS_AIMLsys KI (WT#1.1b): Extensions of information exposure for 5G NF(s)

Document for: 	Approval

Agenda Item: 	9.21

Work Item / Release:	5G_AIMLsys/ Rel-18

Abstract of the contribution: This contribution proposes a KI on extensions of information exposure for 5G NF(s) to support Application AI/ML operation.

1	Discussion

As described in WT#1.1b in SP-211648, this key issue targets for the following objective:

Objective 1 (WT#1.1): Study the possible architectural and functional extensions to support the Application layer AI/ML operations defined in TS 22.261, more specifically:

b.	5GS information exposure extensions for 5GC NF(s) to expose UE and/or network conditions and performance prediction (e.g. location, QoS, load, congestion, etc.) and whether and how to expose such information to the UE and/or to the authorized 3rd party to assist the Application AI/ML operation.

Based on the requirements described in TS 22.261, the 5GS shall be able to expose some AI/ML operation related information and/or network competency (e.g. predicted network condition, monitoring and status information, event alert, etc.) to UE and/or to the authorized 3rd party to assist three main types of Application AI/ML operations:

-	AI/ML operation splitting between AI/ML endpoints

-	AI/ML model/data distribution and sharing over 5G system

-	Distributed/Federated Learning (FL) over 5G system

2 Proposal

It is proposed to add a new key issue about the extensions to information exposure for 5G NF(s) targeting for WT#1.1b in SP-211648 on 5G_AIMLsys.

The following changes are proposed for TR 23.700-80.

* * * * Start of Changes * * * *

5.X	Key Issue #X: Extensions of information exposure for 5G NF(s)

5.X.1	Description

As the requirements described in TS 22.261, the 5GS shall be able to expose some AI/ML operation related information and/or network competency (e.g. predicted network condition, monitoring and status information, event alert, etc.) to UE and/or to the authorized 3rd party to assist the Application AI/ML operation.

In this key issue, the following aspects will be studied:

- What kind of information should be exposure to UE and/or to the authorized 3rd party to assist the Application AI/ML operation? And what information for UE and what information for authorized 3rd party?

- How to expose this information to UE and/or to the authorized 3rd party, respectively? Whether existing 5GC NF(s) exposure can satisfy the requirement?

  - if not, which 5GC NF(s) needs extensions and what kind of exposure can be extended?

* * * * End of Changes * * * *