SA WG2 Meeting #149e	S2-2200187

# February 14th – 25th, 2022 Elbonia               	       (revision of S2-220xxxx)

Source: 	Ericsson

Title: 	WT#5: URSP Rules in EPC

Document for: 	Approval

Agenda Item: 	9.22

Work Item / Release:	FS_eUEPO/ Rel-18

Abstract:

# 1	Discussion

TS 23.501 clause 5.17.1.2 defines the conditions for the UE to use URSP Rules for associating an application to a PDN connection when attached to EPC as follows: ”If the UE does not have preconfigured rules for associating an application to a PDN connection (i.e. the UE does not have rules in UE local configuration and is not provisioned with ANDSF rules), the UE should use a matching URSP rule as defined in TS 23.503 [45], if available, to derive the parameters, e.g. APN, for the PDN connection establishment and associating an application to the PDN connection.”

Then, TS 24.526 defines the mapping for both the traffic descriptors and the Route Selection Description in the URSP rule defined in TS 23.503 into EPS parameters and is to be applied for a UE using S1 mode.

The FS_UEPO SID states in the objectives that the UE able to apply URSP rules in EPC but is not able to receive any URSP rule update when attached to EPC, therefore the following WT was added:

- 	WT#5: Investigate if enhancements are needed for provisioning UE with consistent URSP across 5GC and EPC and study the mechanism if needed.

NOTE 1: No new interface will be defined for WT#5.

# 2 Proposal

It is proposed to add the following key issues to the TR 23.700-85 "Study on enhancement of 5G UE Policy”

* * * 1st Change * * *

## 5.x	Key Issue #X: Distribution of URSP Rules to a UE attached to EPC

### 5.x.1 	Description

This key issue proposes to study how to provision, modify or delete URSP Rules to a UE, supporting S1 mode, when the UE is attached to EPC. This corresponds to WT#5: URSP Rules in EPC.

The working assumptions are a) any solution to transfer URSP Rules to the UE that is attached to EPC reuses existing EPC and 5GS interfaces (i.e., no new interface) and b) a URSP Rule associates the traffic of an application to a PDU Session in 5GS or to a PDU connection in EPC based on the mapping rules defined in TS 24.526 [x] and c) EPC interworking architecture in 23.501 [2] is reused.

The issues to be studied are described as follows:

-	How and when a UE attached to EPC reports the list of PSIs to 5GS for the UE.

-	How and when the 5GS distributes URSP Rules to a UE attached to EPC.

* * * End of Changes * * *