SA WG2 Meeting #S2-149E	S2-2200199

14 - 25 February, 2022, Electronic meeting

3GPP TSG-CT WG4 Meeting #107-bis-e					C4-220308

E-Meeting, 17th– 21st January 2022

Title:	LS on MBS session restoration for NG-RAN failure with or without restart

Release:	Rel-17

Work Item:	5MBS

Source:	CT4

To:	SA2

cc:	RAN3

Contact Person:	Frank Yong Yang

E-mail Address:	frank.yong.yang@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

CT4 has started discussing potential restoration procedures to restore an MBS session upon a failure with or without restart of any entity involved in the MBS service delivery, e.g. NFs at control plane, MB-UPF/UPF and NG-RAN.

With NF (service) Set deployment and relevant restoration procedures specified for CUPS interfaces, CT4 considers that the MBS session can be seamlessly restored when there is a failure in 5GC entities related to a MBS session, though CT4 needs to document this in their specification.

However, CT4 believes further specification is required to restore an MBS session when detecting a NG-RAN failure with restart, as well as to stop the delivery of MBS data towards an NG-RAN that fails without restart for unicast transport.

It was pointed out during the discussion that additional use cases exist, not linked to NG-RAN failure/restart, where an existing MBS session would also need to be started in an NG-RAN, e.g. when a (new) NG-RAN is switched on in a TAI or is reconfigured by OAM to serve a new TAI that is part of an MBS service area of an existing broadcast MBS session, where similar procedures as for an NG-RAN restart could possibly apply.

Those additional use cases are beyond restoration procedures, so CT4 would like to receive some feedback from SA2 on whether such procedure will be covered by stage 2 (TS 23.247), and if so, CT4 may refer to it when specifying restoration procedure for NG-RAN restart.

CT4 intends to specify the MBS restoration procedures, upon a failure with or without restart of any entity involved in the MBS service delivery, as part of its Rel-17 work for 5MBS.

2. Actions:

To SA2

ACTION: 	CT4 kindly asks SA2 to provide feedback on whether they will specify a procedure to start an MBS session (for an existing MBS session) in a (new) NG-RAN for the aforementioned non restoration related use cases, and feedback if any on the procedures to restore an MBS session upon detection of a NG-RAN failure/restart.

3. Date of Next CT4 Meetings:

3GPP TSG CT4#108e			02/2022	E-Meeting