3GPP TSG-SA WG2 Meeting #149E (e-meeting)	S2-2200216

# Feb 14 – 25, 2022, Elbonia	(rev of S2-2108382)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0027 | rev | 6 | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN resolve for mobility restriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [vivo, Samsung, Nokia, Nokia Shanghai Bell,] Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-01-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following EN is to be resolved: Editor's note: It is FFS on mobility restriction when the 5G ProSe UE-to-Network Relay and 5G ProSe Remote UE locate in Non-allowed area. In the previous meetings (i.e., SA2#147E and SA2#148E), the following aspects are discussed: #1 Whether L2 Relay UE in Non-Allowed Area should perform relay operation #2 For both L2 & L3 relaying, whether the emergency/priority access from the remote UE are allowed, and if yes how For #1, there are mainly two main views: - View-1: The L2 Relay UE does not advertise itself as candidate Relay UE. This basically follows what is specified in TS 23.501 that a UE in Non-Allowed Area is not allowed to use resource related to user data. “In a Non-Allowed Area a UE is service area restricted based on subscription. The UE and the network are not allowed to initiate Service Request, or any connection requests for user plane data, control plane data, exception data reporting, or SM signalling (except for PS Data Off status change reporting) to obtain user services that are not related to mobility”. When a UE acts as a Relay, it may be used for the remote UE to transfer the user data, and in this case the Relay UE will consume resource related to user data although the resource is not in the form of Relay’s UE’s DRB. - View-2: The L2 Relay UE may still perform Relay operations (e.g. Relay discovery or accept the relay connection setup since the traffic is for remote UE). The main argument of View-2 is that the remote UE does not use Relay UE’s DRB resource. For #2, there could be different emergency scenarios (e.g. UEs with / without SIM, different local rules and regulations), depending on the local regulation, the network may behave differently. The emergency aspect has been included as WT#10 in Rel-18 SID SP-211653. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the following to resolve the EN: For UE that can't communicate with the network due to Mobility Restrictions, the UE does not advertise itself as 5G ProSe UE-to-Network Relay. For Relay UE in Non-Allowed Area: - Either L2 or L3 Layer-3 Relay UE is not allowed to perform Relay operations For Remote UE in Non-Allowed Area: - L2 Remote UE follows the same principles as specified in clause 5.3.4.1 of TS 23.501for communication with the network via the 5G ProSe Layer-2 UE-to-Network Relay. - For L3 Remote UE, service Area Restriction is not applicable For emergency/priority access from a Remote UE to a Relay UE in non-allowed area, add a note that the emergency/priority access is not supported in this release of the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EN of how to handle the mobility restriction about non-allowed area still exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 5.4.3	Mobility Restrictions for 5G ProSe UE-to-Network Relay

The handling of Mobility Restrictions for 5G ProSe UE-to-Network Relay case follows the principles as specified in clause 5.3.4.1 of TS 23.501 [4] with the following additions:

If a UE is not able to communicate with the network due to Mobility Restrictions, the UE shall not advertise itself as 5G ProSe UE-to-Network Relay.

Forbidden Area:

-	In a Forbidden Area, a 5G ProSe UE-to-Network Relay and a 5G ProSe Remote UE shall follow the principles of a UE in limited-service state as specified in clause 5.9.

Non-Allowed Area:

-	In Non-Allowed area of its serving PLMN, the 5G ProSe L2 or Layer-3 UE-to-Network Relay is not allowed to perform Relay operations e.g. Relay discovery or accept the relay connection setup. If a UE acting as 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay enters Non-Allowed Area, the UE stops the relaying operation.

-	Service Area Restriction is not applicable to the 5G ProSe Layer-3 Remote UE.

-	In Non-Allowed Area of its serving PLMN, the 5G ProSe Layer-2 Remote UE follows the same principles as specified in clause 5.3.4.1 of TS 23.501 [4] for communication with the network via the 5G ProSe Layer-2 UE-to-Network Relay.

NOTE:  The emergency and priority access from a 5G ProSe Remote UE via a 5G ProSe UE-to-Network Relay UE in Non-allowed area is not supported in this release of the specification.

If a UE is not able to communicate with the network due to other Mobility Restrictions (e.g. Core Network Restriction, RAT Restriction), the UE shall not advertise itself as 5G ProSe UE-to-Network Relay.

* * * * Start of 1st Change * * * *

## 5.9	Support for 5G ProSe for UEs in limited service state

For UE in limited service state, as defined in TS 23.122 [14], 5G ProSe can be used over PC5 reference point with the following considerations.

UEs that are authorized to use 5G ProSe over PC5 reference point according to clause 5.1 shall be able to use the corresponding services following the principles defined in clause 5.1.2.2 for 5G ProSe Direct Discovery, clause 5.1.3.2 for 5G ProSe Direct Communication, and clause 5.1.4.2 for 5G ProSe UE-to-Network Relay when the UE enters in limited service state in 5GS:

-	because UE cannot find a suitable cell of the selected PLMN as described in TS 23.122 [14]; or

-	as the result of receiving one of the following reject reasons defined in TS 23.122 [14]:

-	a "PLMN not allowed" response to a registration request or;

-	a "5GS services not allowed" response to a registration request or service request.

A UE in limited service state shall only use the radio resources and procedure available in CM-IDLE mode for ProSe over PC5 reference point, for details see TS 36.300 [11] and TS 38.300 [12].

UEs shall not use 5G ProSe over PC5 reference point using the "operator-managed" radio resources, as specified in clauses 5.1.2.1, 5.1.3.1, 5.1.4.1, and 5.1.5.1, if the UE has entered in limited service state due to all other situations (e.g. no SIM in the MS, an "illegal MS" or "illegal ME" response to a registration request, or an "IMSI unknown in HLR" response to a registration request) defined in TS 23.122 [14], where the UE is unable to obtain normal service from a PLMN. The UEs may use ProSe over PC5 reference point using the "non-operator-managed" radio resources, as specified in clauses 5.1.2.1, 5.1.3.1, 5.1.4.1, and 5.1.5.1, according to the corresponding principles defined in clauses 5.1.2.2, 5.1.3.2, 5.1.4.2, and 5.1.5.2.

* * * * End of Changes * * * *