SA WG2 Meeting #S2-149E	S2-2200297

14 February – 25 February 2022, Electronic, Elbonia	(revision of S2-220xxxx)

Source:	Futurewei

Title:	Key Issue: Configuration of Vehicle Mounted Relay

Document for:	Approval

Agenda Item: 	9.9

Work Item / Release:	FS_VMR /Rel-18

Abstract of the contribution: This paper proposes a key issue on VMR system configuration to be studied in FS_VMR.

# 1	Discussion

The key issue on vehicle mounted relay and related functionality for TR 23.700-05 is proposed based on the study item description in SP-211636.

# 2	Proposal

It is proposed to adopt the following changes into TS23.700-05.

* * * * 1st Change * * * *

# 5	Key Issues

## 5.x	Key Issue #X: Vehicle Mounted Relay Configuration

### 5.x.1	Description

Vehicle mounted relays may attach or detach to a mobile network depending on its location and movement. When a vehicle mounted relay has just attached to the network, it lacks the configuration data to serve as a mobile relay. While a vehicle mounted relay may attach to 5GS using existing procedures, it does not have several policies to function effectively, including radio parameters for relaying, charging, and accounting data to be collected, and consistent forwarding of packets among others.

This key issue should investigate the policies and data to be configured for the vehicle mounted relay to be operational and the data to be provided by the vehicle mounted relay for accounting for the packets relayed.

The following aspects shall be studied:

- Study mechanisms needed to configure, provision, and control a vehicle mounted relay.

- Consider the functionality needed in 5GC for managing vehicle mounted relays.

- Investigate the data and mechanisms needed to account the packets that are relayed.

- How to handle vehicle mounted relay configuration and related network functionality for non-roaming, roaming and non-public networks.

* * * * End of Changes * * * *