3GPP TSG-SA WG2 Meeting #149E (e-meeting)	S2-2200373

# February 14 – 25, 2022, Elbonia											     (revision of S2-22xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0076 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN resolution on ULI to SMF for local multicast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following EN in clauses 7.2.4.2.1 and 7.2.4.3.2 needs to be resolved. Editor's note: How the AMF provides the ULI information to the SMF is FFS. The UE sends a PDU Session Establishment Request or a PDU Session Modification Request to join the multicast group. Then, the AMF provides UE location information when sending Nsmf_PDUSession_CreateSMContext Request or Nsmf_PDUSession_UpdateSMContext Request including the PDU Session Establishment Request or the PDU Session Modification Request, respectively, to the SMF as specified in clauses 4.3.2.2.1 and 4.3.3.2 of TS 23.502. TS 29.502 describes that TAI and NCGI are included as UE location information. Therefore, the above EN can be removed with some description for clarification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clauses 7.2.4.2.1 and 7.2.4.3.2: - Remove the aforementioned EN. - Add description about providing ULI from AMF to SMF by referring to TS 23.502. Clause 7.2.4.2.1: - Add description about PDU Session Establishment Request or PDU Session Modification Request as a Join Request as in clause 7.2.4.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved EN exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.4.2.1, 7.2.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

7.2.4.2.1	UE join location dependent multicast session and establishment procedure

The location dependent multicast session join and establishment procedure is performed as defined in clause 7.2.1 with the following additions:

-	The location dependent multicast session is created as described in clause 7.2.4.2.2.

-	The UE may have information about the location dependent multicast service including all MBS service areas of the location dependent MBS session.

-	If the UE determines that it is in MBS service area of the location dependent MBS session based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends the PDU Session Modification Request (MBS Session ID) or the PDU Session Establishment Request (MBS Session ID) as a Join Request to join the multicast group. If the UE determines that it is outside all MBS service area of the location dependent MBS session, the UE does not send the Join Request.

-	If SMF has no information about the multicast MBS session context for the indicated MBS Session ID, the SMF requests MB-SMF information via Nnrf_NFDiscovery_Request Request (MBS Session ID, UE location), the NRF provides information about the MB-SMF(s) serving the multicast session at the indicated location and service areas and service area IDs for the multicast session, via Nnrf_NFDiscovery_Request Response (MB-SMF profile (Area Session ID(s), MBS service area(s)). The SMF selects the MB-SMF based on the location area where the UE is residing and interacts with MB-SMF to retrieve QoS information of the multicast QoS flow(s) for the MBS Session ID.

-	The SMF check whether the UE is inside MBS service areas of the location dependent MBS session by comparing the User Location Information of the UE provided by the AMF as specified in clauses 4.3.2.2.1 and 4.3.3.2 of TS 23.502 [6] and received together with the join request with the MBS service areas received from the MB-SMF. If the UE is out of the all MBS service area of the location dependent MBS session, the SMF reject the multicast session join request.

-	If the Join Request from the UE is accepted, the SMF subscribes at the AMF using the Namf_EventExposure service to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. The SMF supplies all MBS service areas of the location-dependent MBS session as Area Of Interest.

-	The SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the Area Session ID and MBS service area associated with the cell where UE camps.

-	If the NG-RAN node supports MBS, the NG-RAN uses the received MBS Session ID and Area Session ID to determine the local multicast session context and whether the user plane for the local multicast session is already established. If the target RAN determines the shared delivery is not established for the MBS session ID and area session ID, the target NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4. The MB-SMF provides MBS service area information (Area session ID(s), MBS service area(s)) associated with the same MBS session to NG-RAN in the shared delivery establishment response.

Editor's note:	It is FFS whether the MB-SMF provides only the service areas associated with the target RAN node or all service areas associated with the MBS session in the shared delivery establishment response message.

-	If the NG-RAN node serving the UE does not support MBS and UE is in the MBS service area, the SMF apply individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session and service area via individual delivery within a PDU session of the UE. The SMF additionally subscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes (e.g. for a small MBS service area), or to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. In the latter case, the SMF supplies the service area associated with the multicast session where the UE resides as Area Of Interest.

* * * * Start of Next Change * * * *

7.2.4.3.2	Multicast session join and session establishment procedure for local MBS

For the case that the multicast service is only available within a limited area, the UEs outside the MBS service area are not allowed to join the multicast service.

The multicast session join and session establishment for multicast service available within a limited area are performed as specified in clause 7.2.1.3 with the following differences and clarifications:

-	The local multicast session has been created where the AF provided the MBS service area information as specified in clause 7.2.4.3.1.

-	The UE may have information about local multicast service including MBS service area via Service Announcement as specified in clause 7.2.4.3.1.

-	In step 1, if the UE determines that it is in the MBS service area based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends the PDU Session Modification Request (MBS Session ID) or the PDU Session Establishment Request (MBS Session ID) as a Join Request to join the multicast group. If the UE determines that it is outside the MBS service area, the UE does not send the Join Request.

-	In step 3, the SMF obtains the MBS service area (i.e. Cell ID list or TAI list) of the indicated MBS session from the MB-SMF, and the SMF determines the indicated MBS session corresponds to local multicast service based on the MBS service area.

-	In step 5, the SMF checks whether the UE is inside or outside the MBS service area.

-	The SMF determines whether the UE is inside the MBS service area by comparing the User Location Information of the UE provided by the AMF as specified in clauses 4.3.2.2.1 and 4.3.3.2 of TS 23.502 [6] and received together with the join request with the MBS service area received from the MB-SMF.

-	If the SMF detects that the UE is inside the MBS service area, the SMF continues the process of multicast session join as specified in clause 7.2.1.3 with the following additions:

-	The SMF sends the UE a PDU Session Modification Command or a PDU Session Establishment Accept indicating a Join Accept as a response to the Join Request. The Joint Accept includes the MBS service area (i.e. Cell ID list or TAI list).

-	The multicast session information sent by the SMF to the NG-RAN includes the MBS service area (i.e. Cell ID list or TAI list).

-	If the RAN node serving the UE does not support MBS the SMF applies individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session via individual delivery within a PDU session of the UE.

-	If the SMF detects that the UE is outside the MBS service area, the SMF rejects the multicast session join, so sends a Join Reject to the UE indicating that the MBS join is rejected. The Joint Reject includes the reject reason (outside of local service area) and the MBS service area (i.e. Cell ID list or TAI list).

In this case, the MBS Session establishment (i.e. resources establishment for MBS traffic delivery) for the UE is not performed.

NOTE 1:	There may be the case that the UE determines that it is inside the MBS service area based on the geographical area information or civic address information as MBS service area information provided by the AF, so sends a Join Request but the UE is outside the MBS service area.

NOTE 2:	Which SM NAS message is used to deliver the Join Reject (e.g. PDU Session Modification Reject) is defined in stage 3 specifications.

-	If the Join Request from the UE is accepted, the SMF subscribes to the UE mobility event notification from the AMF (e.g. UE moving into or out of Area Of Interest, which is set by MBS service area), by invoking Namf_EventExposure_Subscribe service operation as specified in clause 5.2.2.3.2 of TS 23.502 [6]. The SMF supplies the service area associated with the multicast session as Area Of Interest.

-	For the UE that received the Join Reject from the SMF, later, if the UE detects that it is inside the MBS service area based on the MBS service area provided in the Join Reject, the UE sends the PDU Session Modification Request (MBS Session ID) to join the multicast group.

* * * * End of Changes * * * *