SA WG2 Meeting #149e	S2-2200454

# February 14th – 25th, 2022; Elbonia

Source: 	Thales, Xiaomi

Title: 	FS_5GSAT_Ph2 TR architectural assumptions

Document for: 	Approval

Agenda Item: 	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract of the contribution: define architectural assumptions and principles for TR 23.700-28

# 1	Discussion

void

# 2 Proposal

It is proposed to update TR 23.700-28 on FS_5GSAT_Ph2 as follows

# 4	Architectural Assumptions and Principles

- Principles

The architecture for satellite access to 5GC as defined in TS 23.501 [2] is used as a baseline.

The architecture for satellite access to EPC as defined in TS 23.401 [6] is used as a baseline.

Solutions should be applicable to both EPS and 5GS.

Solutions shall minimize the impact on 5GS and EPS system protocols.

- Definition and assumptions:

Uu

UE

Satellite

Gateway

eNB/gNB

EPC/5GC

Feeder link

Service link

- Discontinuous satellite coverage can be characterized by the fact that Uu interface is available for the UE, at a given position, less than 100% of the time, due to predictable lack of satellite coverage.

- If Uu interface is available for the UE, both service link with satellite and feeder link with ground gateway are assumed to be available.

- Discontinuous satellite coverage under EPC and 5GC interworking is not considered in this release.

- Scenarios

The following scenarios are proposed for the study:

- S1: NGSO sparse constellation (e.g: during deployment phase)

- S2: NGSO sparse constellation because planned satellite/gateway unavailability

- S3: NGSO sparse constellation because not planned satellite/gateway unavailability

- S4: GEO planned satellite/gateway unavailability

- S5: GEO not planned satellite/gateway unavailability

- S6: GEO intermittent beam coverage (beam hopping)

- S7: GNSS unavailability on a given zone (considering UE with GNSS capacity, enabled)