SA WG2 Meeting #150e	S2-2202735

April 6th – 12th, 2022	  (revision of S2-220xxxx)

Source: 	China Mobile

Title: 	New Solution: Support QoS Monitoring results exposure by UPF

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS/ Rel-18

Abstract of the contribution: Solution proposal for UPF exposing QoS Monitoring results to NEF directly.

1 Discussion

The solution proposal presented here addresses the key issue proposal in S2-2201823.

This paper proposes to support the UPF exposes QoS Monitoring results to NEF directly to facilitate the local AF requesting information with low latency.

2 Proposal

It is proposed to update TR 23.700-62 v0.1.0 as follows.

****************************First change****************************

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #1: UPF event exposure service framework enhancements to support registration, deregistration and discovery via NRF | X |  |
| #2: QoS Monitoring results exposure by UPF |  | X |

****************************Second change (all text is new) ****************************

6.X	Solution #X: QoS Monitoring results exposure by UPF

6.X.1	Description

Some real time network information, e.g. user path latency, is useful for application layer. In R17, in order to expose network information timely to local AF, the L-PSA UPF may expose network information i.e. QoS monitoring results as defined in TS 23.501 [2], clause 5.33.3, to the local AF.

The UPF may be instructed to report information about a PDU Session directly to the local NEF/NEF/AF i.e. by passing the SMF and the PCF. This reporting may target an Edge Application Server (EAS) or a local AF that itself interfaces the EAS.

6.X.2	Procedures

The L-PSA UPF may support Nupf_EventExposure_Subscribe service operation, as defined in TS 23.502 [3] clause 5.2.26.2.X.

Figure 6.X.2-1: QoS Monitoring results exposure by UPF

0.	The UE establishes a PDU Session and UPF notifies the QoS monitoring information to Local NEF/NEF/AF as described in step 0-5 in clause 6.4.2.1 of TS 23.548 [7].

1.	For the subsequent AF request to subscribe direct notification of QoS monitoring for the same service data flow via Local NEF, the local NEF may directly invoke the Nupf_EventExposure_Subscribe service operation.

The L-UPF sends the notification related with QoS monitoring information over Nupf_EventExposure_Notify service operation to the local NEF, and local NEF reports to AF.

2.	Due to e.g. UE mobility, the PSA relocation and/or EAS relocation as well as new AF session establishment/removed may happen as described in step 6-8 in clause 6.4.2.1 of TS 23.548 [7].

**End of change**