3GPP TSG-WG SA2 Meeting #151E e-meeting  	S2-2204261

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Title:	[DRAFT] LS on Clarifications on Nmbstf_MBCDistributionSession service

Response to:	LS (S4-220575) on Clarifications on Nmbstf_MBCDistributionSession service from SA4

Release:	Release 17

Work Item:	5MBS, 5MBUSA

Source:	[Huawei to be] SA2

To:	SA4, CT4

Cc:	SA3

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	Raymond DOT Limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.247 CR0119

1. Overall Description:

SA2 would like to thank to SA4 for the questions related to the design of MBSF/MBSTF, and SA2 would like to provide the following responses.

Regarding the Issues listed in the LS:

Issue 1: Hence, the MB-UPF tunnel endpoint address must always be provided in the Nmbstf_MBSDistributionSession_Create service operation so that the unicast tunnel can be established between the MBSTF and the MB-UPF at the appropriate time, and that the cardinality of this parameter is correctly expressed as 1..1 in table 4.5.6-1 TS 26.502 V17.0.0. SA4 kindly asks SA2 to review SA4’s feedback in section 1 above and confirm whether SA4’s understanding is correct, in particular SA4’s answer to question 2.

SA2 answer:

SA2 discussed this issue and agreed that the tunnel between MBSTF and MB-UPF will be a unicast tunnel. SA2 finds the current TS 23.247 is with unclear wordings and SA2 agreed the attached CR to resolve that.

Issue 2: SA4 kindly asks SA2 to confirm that it is reasonable for the MBSTF to set a DS Code Point on the multicast IP packets it generates or forwards at Nmb9. Additionally, SA4 asks whether it is reasonable for this DS Code Point to be provisioned by the AF/AS at Nmb10/Nmb5 or whether the MBSF should determine the DS Code Point to use for traffic labelling in order to guarantee correct end-to-end multicast QoS operation between the AF/AS and the UE.

SA2 answer:

From SA2 perspective, DSCP can be used for the following scenarios:

1. Paging strategy handling. As per the definition of TS 23.247, existing QoS information for multicast seems to be sufficient:

-	"SMF may provide the most demanding ARP and 5QI of all MBS QoS Flow within the MBS session to the AMF;"

-	"AMF may take the received ARP and 5QI into consideration in paging differentiation."

2. Transport level marking. For unicast, UPF may mark the traffic with a DSCP value based on Packet Detection Rule / Forwarding Action Rule. Hence, it is also possible for the CP (MBSF) to configure the UP (MBSTF) for better transmitting the traffic in Nmb9.

Therefore, SA2 considers that it might be beneficial for MBSTF marking DSCP value to guarantee the QoS for Nmb9, and SA2 considers the DSCP value can be configured by the instruction of MBSF.

2. Actions:

To CT4 and SA4 group.

ACTION: 	SA2 asks CT4 and SA4 group to take above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #152E   	Aug xx – xx, 2022 				TBD.