SA WG2 Meeting ##152-e	S2-2205410

17-26 August 2022, Electronic Meeting

3GPP TSG-CT3 Meeting #122-e 	C3-223295

# E-Meeting, 12th – 20th May 2022

Title:	LS on Issues on Nadrf_DataManagement Service

Release:	Rel-17

Work Item:	eNA_Ph2

Source:	CT3

To:	SA2

Cc:

Contact person:	Xuefei Zhang

summer.xuefei@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall description

CT3 is specifying the procedures of Analytics Exposure via DCCF and Data Collection via DCCF in 3GPP TS 29.552. CT3 noticed that in Figure 6.1.4.3-1, Figure 6.1.4.5-1, Figure 6.2.6.3.3-1 and Figure 6.2.6.3.5-1 of 3GPP TS 23.288, only the Nadrf_DataManagement_RetrievalSubscribe service operation is used by the DCCF to retrieve the stored data or analytics from the ADRF. CT3 would like to kindly ask SA2 to answer the questions below:

Question 1: Can the Nadrf_DataManagement_RetrievalRequest service operation be used in these procedures?

CT3 also noticed the following descriptions in clause 10.2.5 of 3GPP TS 23.288,

### 10.2.5	Nadrf_DataManagement_RetrievalRequest service operation

Service operation name: Nadrf_DataManagement_RetrievalRequest

Description: The consumer NF uses this service operation to retrieve stored data or analytics from the ADRF. The Nadrf_DataManagement_RetrievalRequest response either contains the data or analytics, or provides instructions for fetching the data or analytics. The Nadrf_DataManagement_RetrievalRequest may be unsolicited or sent in response to a Fetch Instructions from the ADRF in an Nadrf_DataManagement_RetrievalNotify.

Question 2: Could SA2 please specify the "unsolicited" scenario?

# 2	Actions

To SA2:

ACTION: 	CT3 kindly asks SA2 to answer above questions and amend the SA2 specifications accordingly.

# 3	Dates of next TSG CT WG 3 meetings

3GPP TSG CT3#122e	12th - 20th May 2022	E-Meeting

3GPP TSG CT3#123	22th - 26th August 2022	Ordinary