3GPP TSG-SA WG2#152E e-meeting	S2-2205515

Elbonia, 17-26 August 2022	(was S2-220xxxx)

Source:	Qualcomm, China Telecom

Title:	KI#2: Merging solutions 15 and 16

Document for:	Approval

Agenda Item:	9.13

Work Item / Release:	FS_5WWC_Ph3 / Rel-18

Abstract of the contribution:. This contribution merges solutions 15 and 16

# 1.	Text proposal

It is proposed to agree the following changes to TR 23.700-17:

# >>>>BEGINNING OF CHANGES<<<<

## 6.15	Solution #15: Selecting N3IWF supporting the S-NSSAI needed by UE

### 6.15.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). (Sub) clause(s) may be added to capture details.

This solution aims at addressing Key Issue #2 about how to select a TNGF/N3IWF that supports the S-NSSAI(s) needed by the UE. In particular, this solution mainly focuses on how to select a N3IWF that supports the S-NSSAI(s) needed by the UE.

The solution provides two methods to select a N3IWF that supports the S-NSSAI(s) needed by the UE:

-	UE configuration: Key idea is to enhance the N3IWF identifier configuration and to amend the non-3GPP access node selection information with information about supported S-NSSAI(s) per N3IWF. Access Network Discovery & Selection policy (ANDSP) was designed to be provided to UE by the network for selection of N3IWF. Adding the supported S-NSSAI(s) in ANDSP allows UE to determine an appropriate N3IWF that supports the S-NSSAI(s) needed by the UE during N3IWF selection procedure.

-	NAS-based redirection:	When UE connects with a N3IWF that does not support the S-NSSAI(s) needed by the UE, AMF can determine the target N3IWF which supports the S-NSSAI(s) needed by the UE and provides the target N3IWF information to UE via Registration Accept/Reject message so that UE can use the target N3IWF information to connect with the target N3IWF in order to meet the requirement of the UE. Redirection can be applied when the UE has not been updated with the enhanced N3IWF identifier configuration or non-3GPP access node selection information.

### 6.15.2	N3IWF selection solution

#### 6.15.2.1	UE configuration

The H-PCF may provide the UE with the following information for the HPLMN (which the UE stores and applies as described further below):

-	Extended Home N3IWF identifier configuration (one or multiple entries)

-	FQDN or IP address of the N3IWF in the HPLMN and the S-NSSAIs supported by this N3IWF

-	Slice-specific N3IWF prefix information for the HPLMN (one or multiple entries)

-	List of supported S-NSSAIs

-	Prefix to be added to the existing Tracking Area (TA) or Operator Identifier (OI) FQDNs

The V-PCF may provide the UE with the following information for that VPLMN (which the UE stores and applies as described further below):

-	Slice-specific N3IWF prefix information for that VPLMN (one or multiple entries)

-	List of supported S-NSSAIs

-	Prefix to be added to the existing TA or OI FQDNs

NOTE 1:	It is assumed that the UE will indicate its supports of extended N3IWF configuration to the PCF. The details of the indication can be specified by CT1.

To enable the V-PCF to provide the UE with slice specific N3IWF configuration information, the AMF provides the V-PCF with the Configured NSSAI for the serving PLMN during UE Policy Association Establishment/Modification. The PCF (V-PCF in the roaming case) is assumed to be locally configured with information about the slices supported by the different N3IWFs in the serving PLMN.

NOTE 2:	The PCF already receives the subscribed NSSAI from the UDR, therefore there is no need for the AMF to provide the Configured NSSAI to the PCF in the non-roaming case.

#### 6.15.2.2	N3IWF selection

This clause describes N3IWF selection based on the additional information listed in the previous clause as delta on top of the existing N3IWF selection. Since the full details of N3IWF selection are only covered by Stage 3 specs, the delta is described on top of TS 24.502 [X]:

-	UE is located in its home country:

-	The procedures described in Rel-17 TS 24.502 [X] clauses 7.2.4.3 and 7.2.4.4 are applied with the following changes:

-	If the UE is configured with Extended Home N3IWF identifier configuration, then the UE uses the Extended Home N3IWF identifier configuration:

-	UE uses the FQDN or IP address from the Extended Home N3IWF identifier configuration that matches all (or most, in case there is no full match) of the S-NSSAIs that the UE is going to request in the subsequent Registration.

-	If the UE is not configured with Extended Home N3IWF identifier configuration and not configured with Rel-17 Home N3IWF identifier configuration but configured with slice-specific N3IWF prefix information for the HPLMN:

-	Whenever the UE constructs an N3IWF FQDN as per TS 24.502 [X] clauses 7.2.4.3 and 7.2.4.4, the UE first selects the Slice-specific N3IWF prefix information for the HPLMN that matches the S-NSSAIs the UE is going to request in the subsequent Registration procedure and adds the prefix to the Rel-17 TA or OI format FQDNs as follows:

-	<Prefix>.tac-lb<TAC-low-byte>.tac-hb<TAC-high-byte>.tac.n3iwf.5gc.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org

-	<Prefix>.n3iwf.5gc.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org

NOTE 1:	Which FQDN format (TA or OI) to construct follows the existing description in TS 24.502 [X] clauses 7.2.4.3 and 7.2.4.4.

-	If the UE has constructed an N3IWF FQDN including a prefix but the DNS does not return an IP address, then the UE attempts the same FQDN without the prefix.

-	If the UE is not configured with Extended Home N3IWF identifier configuration and not configured with Slice-specific N3IWF prefix information, then the existing procedures in Rel-17 TS 24.502 [X] clauses 7.2.4.3 and 7.2.4.4 apply.

-	UE is not located in its home country:

-	The procedures described in Rel-17 TS 24.502 [X] clauses 7.2.4.3 and 7.2.4.4 are applied with the following changes:

-	If the UE is configured with slice-specific N3IWF prefix information for the selected VPLMN:

-	Whenever the UE constructs an N3IWF FQDN as per TS 24.502 [X] clauses 7.2.4.3 and 7.2.4.4, the UE first selects the slice-specific N3IWF prefix information for the VPLMN that matches the S-NSSAIs the UE is going to request in the subsequent Registration procedure and adds the prefix to the Rel-17 TA or OI format FQDNs as follows:

-	<Prefix>.tac-lb<TAC-low-byte>.tac-hb<TAC-high-byte>.tac.n3iwf.5gc.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org

-	<Prefix>.n3iwf.5gc.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org

NOTE 2:	Which FQDN format (TA or OI) to construct follows the existing description in TS 24.502 [X] clauses 7.2.4.3 and 7.2.4.4.

-	For selecting the slice-specific N3IWF prefix information for the VPLMN, the UE applies the S-NSSAIs valid in the VPLMN.

-	If the UE has constructed an N3IWF FQDN including a prefix but the DNS does not return an IP address, then the UE attempts the same FQDN without the prefix.

-	If the UE determines that the visited country does not mandate the selection of N3IWF or ePDG in the visited country and if the N3AN node configuration information is not provisioned or the N3AN node configuration information is provisioned and the N3AN node selection information of the N3AN node configuration information excluding any PLMN in the list of "forbidden PLMNs for non-3GPP access to 5GCN" contains no PLMN in the visited country and the UE is configured with Extended Home N3IWF identifier configuration, then the UE uses the IP address or FQDN from the Extended Home N3IWF identifier configuration that matches the S-NSSAIs the UE is going to request in the subsequent Registration.

### 6.15.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

Figure 6.15.3-1: Registration via untrusted non-3GPP access

1.	The UE connects to an untrusted non-3GPP Access Network with any appropriate authentication procedure and it is assigned an IP address. For example, a non-3GPP authentication method can be used, e.g. no authentication (in the case of a free WLAN), EAP with pre-shared key, username/password, etc. When the UE decides to attach to 5GC network, the UE selects an N3IWF in a 5G PLMN as described in clause 6.15.2.2.

2.	The UE proceeds with the establishment of an IPsec Security Association (SA) with the selected N3IWF by initiating an IKE initial exchange according to RFC 7296 [3]. After step 2, all subsequent IKE messages are encrypted and integrity protected by using the IKE SA established in this step.

3.	The UE shall initiate an IKE_AUTH exchange by sending an IKE_AUTH request message. The AUTH payload is not included in the IKE_AUTH request message, which indicates that the IKE_AUTH exchange shall use EAP signalling (in this case EAP-5G signalling). If the UE supports MOBIKE, it shall include a Notify payload in the IKE_AUTH request, as specified in RFC 4555 [40], indicating that MOBIKE is supported. In addition, as specified in TS 33.501 [9], if the UE is provisioned with the N3IWF root certificate, it shall include the CERTREQ payload within the IKE_AUTH request message to request the N3IWF's certificate.

4.	The N3IWF responds with an IKE_AUTH response message, which includes an EAP-Request/5G-Start packet. The EAP-Request/5G-Start packet informs the UE to initiate an EAP-5G session, i.e. to start sending NAS messages encapsulated within EAP-5G packets. If the N3IWF has received a CERTREQ payload from the UE, the N3IWF shall include the CERT payload in the IKE_AUTH response message containing the N3IWF's certificate. How the UE uses the N3IWF's certificate is specified in TS 33.501 [9].

5.	The UE shall send an IKE_AUTH request, which includes an EAP-Response/5G-NAS packet that contains the Access Network parameters (AN parameters) and a Registration Request message. The AN parameters contain information that is used by the N3IWF for selecting an AMF in the 5G core network. This information includes e.g. the GUAMI, the Selected PLMN ID (or PLMN ID and NID, see clause 5.30 of TS 23.501 [2]), the Requested NSSAI and the Establishment cause. The Establishment cause provides the reason for requesting a signalling connection with 5GC. Whether and how the UE includes the Requested NSSAI as part of the AN parameters is dependent on the value of the Access Stratum Connection Establishment NSSAI Inclusion Mode parameter, as specified in clause 5.15.9 of TS 23.501 [2].

NOTE 1:	The N3IWF does not send an EAP-Identity request because the UE includes its identity in the first IKE_AUTH. This is in line with clause 3.16 of RFC 7296 [3].

6.	The N3IWF shall select an AMF based on the received AN parameters and local policy, as specified in clause 6.3.5 of TS 23.501 [2]. The N3IWF shall then forward the Registration Request received from the UE to the selected AMF within an N2 message. This message contains N2 parameters that include the Selected PLMN ID and the Establishment cause.

7.	AMF initiates authentication and security procedure as defined in TS 23.502 clause 4.12.2.2

8.	The selected AMF determines whether the N3IWF connected with UE currently (named S-N3IWF) supports at least one S-NSSAI needed by UE.

9.	If the selected N3IWF does not support at least one requested S-NSSAI, the AMF may trigger the UE AM Policy Association Establishment procedure to provide the UE with updated N3IWF selection information described in clause 6.15.2.1.

NOTE 2:	Whether the updated ANDSP is provided to the UE using DL NAS transport or as part of the subsequent Registration Reject message can be determine by CT1.

10a.	AMF sends a Registration Reject message to the UE. Optionally, the AMF may provide target N3IWF information (e.g. FQDN and/or IP address) to UE within Registration Reject message so that UE can use the target N3IWF information to select the target N3IWF to register to 5GC. For this, AMF may determine a target N3IWF that supports the request NSSAI based on the list of supported TAs and the corresponding list of supported slices for each TA obtained in RAN Configuration Update procedure as specified in TS 38.413 [7].

10.b	If the S-N3IWF supports at least one of the requested S-NSSAIs, AMF continues the registration procedure as defined in clause 4.12.2.2 and sends a Registration Accept to the UE. AMF only includes S-NSSAIs supported by the selected N3IWF in the Accepted NSSAI for the UE.

The N3IWF forwards the NAS Registration Accept message to UE via the established signalling IPsec SA. If the NAS Registration Accept message is received by the N3IWF before the IPsec SA is established, the N3IWF shall store it and forward it to the UE only after the establishment of the signalling IPsec SA.

11.	If UE is rejected, UE connects to T-N3IWF if the UE has been provided with T-N3IWF information in the previous Registration Reject, otherwise the UE performs N3IWF selection again using the updated N3IWF selection information.

### 6.15.4	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

UE impacts:

-	Select the N3IWF based on Extended Home N3IWF identifier configuration or based on Slice-specific N3IWF prefix information.

-	Re-select N3IWF based on the N3IWF information provided by the AMF via NAS Registration Accept or Registration Reject message.

AMF impacts:

-	Ability to determine target N3IWF information based on the list of supported TAs and the corresponding list of supported slices for each TA obtained in RAN Configuration Update procedure.

-	Provide UE with the N3IWF information via NAS Registration Accept or Registration Reject message.

-	Provide PCF with Configured NSSAI during UE Policy Association Establishment/Modification in the roaming case.

PCF impacts

-	Provide the UE with extended ANDSP: Extended Home N3IWF identifier configuration or Slice-specific N3IWF prefix information.

-	Receive Configured NSSAI from AMF during UE Policy Association Establishment/Modification in the roaming case.

## 6.16	Solution #16: N3IWF selection taking supported slices into account

### 6.16.1	Description

#### 6.16.1.1	Introduction

Key idea of this solution is to provide additional information to a UE to enable the UE to select an N3IWF that supports the network slices (identified by S-NSSAIs) that the UE is intending to register for, i.e. to select an N3IWF that supports the S-NSSAIs that the UE wants to include in the Requested NSSAI during the subsequent Registration procedure.

This solution has been merged into Solution 15 and will therefore not be progressed further.

#### 6.16.1.2	Solution principles

##### 6.16.1.2.1	General

-	PLMN selection prior to N3IWF selection is performed as defined in TS 23.501 [2]

##### 6.16.1.2.2	N3IWF selection when the UE is located in the home country and when the UE is not located in the home country

-	UE configuration

-	AMF may provide the UE with the following information for the current PLMN (which the UE stores and applies as described further below) as part of the Registration or the UE Configuration Update procedure:

-	Slice-specific N3IWF prefix information: one or multiple entries, each consisting of:

-	List of supported S-NSSAIs

-	FQDN format: Tracking Area (TA) FDQN format or Operator Identifier (OI) FQDN format

-	Prefix

Editor's note:	It is FFS how this information fits with N3AN node selection information and Home N3IWF identifier configuration defined in TS 24.526 [10].

-	N3IWF selection by the UE

-	If the UE has Slice-specific N3IWF prefix information for the PLMN that the UE wants to register with:

-	The UE selects a Slice-specific N3IWF prefix information entry that supports all (or most, in case there is no full match) of the S-NSSAIs that the UE intends to request in the Requested NSSAI, and creates an FQDN using the prefix from the selected prefix information as follows:

-	<Prefix>.tac-lb<TAC-low-byte>.tac-hb<TAC-high-byte>.tac.n3iwf.5gc.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org (if the FQDN format indicates TA FQDN format)

-	<Prefix>.n3iwf.5gc.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org (if the FQDN format indicates OI FQDN format)

-	If the UE has not received Slice-specific N3IWF prefix information from the AMF, then the UE selects an N3IWF using Rel-17 N3IWF selection procedures.

-	If the DNS returns an IP address for the FQDN and the UE has established the connection to the N3IWF but the AMF detects that the selected N3IWF does not support the S-NSSAIs requested by the UE in the Requested NSSAI, then the AMF may indicate updated Slice-specific N3IWF prefix information to the UE and may reject the UE's registration request. Based on this, the UE performs N3IWF selection again.

NOTE:	Alternatively, the AMF can, after updating the UE with new Slice-specific N3IWF prefix information, redirect the UE to a different N3IWF as proposed by solution 11.

### 6.16.2	Procedures

Existing procedures are reused.

### 6.16.3	Impacts on Existing Nodes and Functionality

-	UE:

-	Support N3IWF selection based on Slice-specific N3IWF prefix information from AMF

-	AMF:

-	Indicate Slice-specific N3IWF prefix information to the UE

# >>>>END OF CHANGES<<<<