SA WG2 Meeting #152E	S2-2205576

# 17 – 26 August 2022, e-meeting

Source:	Lenovo

Title:	KI#4: Evaluation and interim conclusions

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_PH3 / Rel-18

Abstract of the contribution:

# 1		Discussion

For Key Issue 4  the solutions agreed in TR23.700-71 are

Table 1 – Solutions for Key Issue 1

|  | Key Issues |
| --- | --- |
| Solutions | 4 |  |
| 10 | X |  |
| 11 | X |  |
| 12 | X |  |
| 13 | X |  |
| 18 | X |  |
| 20 | X |  |

The following table provides an evaluation of each solution.

|  | Solution Evaluation |  |  |
| --- | --- | --- | --- |
| Solutions | Evaluation | Impacts | Editor’s Note to be addressed |
| 10 | Main Principle of Solution NWDAF obtains location information by interfacing with GMLC using existing Ngmlc SBI. LMF uses WLAN performance analytics from NWDAF to select a positioning method at the UE location. Evaluation of Solution The NWDAF enhances UE mobility analytics with location data from GMLC/LMF. It is not specified what is the trigger for the NWDAF to collect location info from GMLC. | LMF: - Selects positioning method based on data analytics from NWDAF. GMLC: - If the UE identifier is SUPI list in the Ngmlc_Location_ProvideLocation Request, the GMLC triggers the existing positioning procedure for each SUPI in the list. |  |
| 11 | Main Principle of Solution LMF subscribes from NWDAF to determine location accuracy for a target area. When an LMF is triggered to estimate a location the LMF obtains accuracy analytics from NWDAF to estimate the accuracy of the location. Evaluation of Solution NWDAF determines location accuracy using MDT data. The NWDAF does not collect data from GMLC/LMF to derive location accuracy analytics. | Editor's note: Any further possible impacts are FFS. | Editor's note: Whether to use new Analytics ID or reuse existing one is FFS and may need to be decided in coordination with eNA_ph3. |
| 12 | Main Principle of Solution LMF subscribes from NWDAF to determine location accuracy for a target area. A new Analytic ID is proposed that trigger the NWDAF to collect data from LMF (Option 1) or GMLC (Option 2). LMF or AF can be a consumer of location accuracy analytics. LMF can use location accuray to determine position method. . Evaluation of Solution New Analytics ID is proposed where the NWDAF collects data from LMF/GMLC used as data source | - Applicable to Option 1: New Nlmf event notification service to allow the NWDAF to retrieve location measurement reports from the LMF. - Applicable to Option 2: NWDAF support Ngmlc service based interfaces to retrieve location information for a UE. | Editor's note: Whether the LMF can provide measurement information to NWDAF for location accuracy is FFS. |
| 13 | Main Principle of Solution .NWDAF obtains location information by interfacing with AMF. AMF selects an LMF for obtaining location of UE. . NWDAF provides UE mobility analytics with higher granularity location information. Evaluation of Solution The NWDAF enhances UE mobility analytics with location data from AMF It is not specified what is the trigger for the NWDAF to collect location info with higher granularity from AMF. | Impacts on AMF - Provides to NWDAF with finer granularity location data than TA/cell level and other information, e.g. location estimate, velocity estimate, timestamp, achieved QoS information. Impacts on NWDAF - Enhanced interface with AMF to request finer granularity location data than TA/cell level. | Editor's note: Whether the interface between NWDAF and GMLC can be supported is FFS. Editor's note: How to perform privacy check based on this architecture is FFS. Editors' note: How the user consent in NWDAF procedures and the privacy check in LCS procedures cooperate or work in this architecture is FFS. |
| 18 | Main Principle of Solution AMF obtains location analytics from NWDAF to verify location for a UE for satelite access Evaluation of Solution The solution should be evaluated as part of KI#9 | . | None |
| 20 | Main Principle of Solution This solution propose an LCS client to request UE's indoor or outdoor location in the "LCS Service Request". Such request triggers LMF to request/subscribe to analytics data from NWDAF. This is to request analytics information from NWDAF to assist LMF in determining whether UE is indoor or not. To support this, NWDAF is proposed to implement new analytics determination logic in order to determine indoor-outdoor prediction of the UE. Evaluation of Solution Details of the procedure are FFS. | . LCS Client: support indication of indoor/outdoor in the LCS Service Request LMF: support determination of UE's indoor or outdoor location NWDAF: support new analytics of indoor and outdoor decision | Editor's note: The new analytics information will be discussed and coordinated with FS_eNA_PH3. Editor's note: It is FFS how the LMF would use analytics data to determine indoor/outdoor status. |

Solutions are so far split into the following categories:

-	Solution proposing enhancement to the NWDAF data collection obtaining higher granularity location information:

-	Location infomration from AMF (AMF obtains location info from LMF) (Solution 13)

-	Location info from GMLC (Solution 10)

-	Solutions proposing new Analytics ID to be supported by the NWDAF

-	Analytics consumer requests Location Accuracy analytics. NWDAF obtains data from GMLC/LMF to identify location accuracy (e.g. vertical/horizontal accuracy) for a position method (Solution 12)

-	LMF subscribes from NWDAF to obtain Location Accuracy analytics. NWDAF obtains data from OAM (MDT data). LMF verifies the location accuracy by leveraging the location estimation from the NWDAF (Solution 11).

-	Solutions leveraging existing analytics

-	LMF uses WLAN performance analytics from NWDAF to select a positioning method at the UE location (Solution 10)

Solution proposing the NWDAF interfacing with AMF or GMLC for obtaining location data are also discussed in solutions within KI#x of FS_eNA_Ph3. As such it is proposed to let the decision on which NF (GMLC/AMF/LMF) will be used as data source by the NWDAF for collecting location information to FS_eNA_Ph3 conclusions.

Solutions proposing new analytics where the LMF or AF can be a consumer and use as a basis to make decisions are within the scope of FS_eLCS_Ph3. As interim conclusion it is proposed to agree that:

-	NWDAF provides new analytics for Location Estimation Accuracy (e.g. horizontal or vertical accuracy).

-	LMF as a consumer of such analytics uses Location Estimation Accuracy analytics to determine Position Method in the area where a UE is located.

# 2		Proposal

The following is proposed.

******************************** First change  *******************************

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

Evaluation of solutions for KI#4:

Solution proposing enhancement to the NWDAF data collection obtaining higher granularity location information:

-	Location infomration from AMF (AMF obtains location info from LMF) (Solution 13)

-	Location info from GMLC (Solution 10)

-	Solutions proposing new Analytics ID to be supported by the NWDAF

-	Analytics consumer requests Location Accuracy analytics. NWDAF obtains data from GMLC/LMF to identify location accuracy (e.g. vertical/horizontal accuracy) for a position method (Solution 12)

-	LMF subscribes from NWDAF to obtain Location Accuracy analytics. NWDAF obtains data from OAM (MDT data). LMF verifies the location accuracy by leveraging the location estimation from the NWDAF (Solution 11).

-	Solutions leveraging existing analytics

-	LMF uses WLAN performance analytics from NWDAF to select a positioning method at the UE location (Solution 10)

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## Interim Conclusions for KI#4:

-	NWDAF provides new analytics for Location Estimation Accuracy (e.g. horizontal or vertical accuracy).

-	LMF as a consumer of such analytics uses Location Estimation Accuracy analytics to determine Position Method in the area where a UE is located.

******************************** End of change *******************************