3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205718

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluation and conclusion of KI#3

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO /Rel-18

Abstract: It is proposed to have evaluation and conclusion for KI#3.

1. Introduction/Discussion

There are seven solutions for KI#3. The details are shown as Table 1-1

| Solutions | Title | Aspect | Providing method |
| --- | --- | --- | --- |
| 16 | URSPs into PCO for EPC | URSP provisioning | PGW (ePCO) |
| 17 | Indication of applicability of URSP to UE in EPS | URSP differentiation | PCF |
| 18 | Provisioning consistent URSP to UE across 5GS and EPS for Collocated PCF Deployment | URSP provisioning | PGW (ePCO) |
| 19 | Provision of URSP to UE when the UE attaches in the EPS | URSP provisioning | PGW (ePCO) |
| 20 | URSP delivery for the UE in EPS | URSP provisioning | PGW (ePCO) |
| 33 | Consistent URSP Provisioning across 5GS and EPS | URSP provisioning | PGW (ePCO) |
| 34 | Provisioning UE policy in EPS via N3IWF | URSP provisioning | PCF (via non-3GPP) |

Solution #17 is for URSP differentiation while all other solutions are for URSP provisioning in EPS.

They can be divided into two categories:

- ePCO based solution

Solution #16, #18, #19, #20 and #33 are proposing to use ePCO to transfer URSP in EPS. (Solution #33 can be considered as a consolidate solution for this method). It is assumed that the UE will have a PDU Session to transfer the ePCO.

- Non-3GPP based solution

Solution #34 proposes to access to 5GC via non-3GPP. This solution almost has no impact on the current system. But it is assumed that the UE can access to 5GC via non-3GPP during interworking. This assumption has a big limitation. They have no impact

However, in both categories the 5GC could update the URSP directly in EPC, i.e. 5GC just needs to make sure the UE will only use proper URSP in EPS. The PCF should be able to get the input for URSP decision from the Policy association (e.g. PSIs, the PCR Triggers and so on).  It is not necessary to let UE be aware of whether an URSP can be used in EPS or not. (The PCF can be aware whether the UE is in EPS)

In conclusion, it is proposed only Solution #33 will be adopted in normative phase.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-85.

# * * * * First change * * * *All new

7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

7.1	Evaluation on Key Issue #3

Solution #17 is for URSP differentiation while all other solutions are for URSP provisioning in EPS.

They can be divided into two categories:

- ePCO based solution

Solution #16, #18, #19, #20 and #33 are proposing to use ePCO to transfer URSP in EPS. (Solution #33 can be considered as a consolidate solution for this method). It is assumed that the UE will have a PDU Session to transfer the ePCO.

- Non-3GPP based solution

Solution #34 proposes to access to 5GC via non-3GPP. This solution almost has no impact on the current system. But it is assumed that the UE can access to 5GC via non-3GPP during interworking. This assumption has a big limitation. Besides, Solution #34 does not require normative work.

However, in both categories the 5GC could update the URSP directly in EPC, i.e. 5GC just needs to make sure the UE will only use proper URSP in EPS. The PCF is able to get the input for PCC decision for the URSP (e.g. PSIs, the PCR Triggers and so on).  It is not necessary to let UE be aware of whether an URSP can be used in EPS or not. (The PCF can be aware whether the UE is in EPS)

# * * * * Second change * * * * All new

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.1	Conclusion on Key Issue #3

In conclusion, it is agreed the following principles of Solution #33 will be adopted in normative phase.

The UE will get the URSP in ePCO in EPS side as described in Solution #33. The PCF send URSP to the SMF when it determines the UE is in EPS side and the SMF will include it in ePCO.

The PCF should be able to get the input for URSP decision from the SMF (e.g. PSIs, the PCR Triggers and so on). It is not necessary to let UE be aware of whether an URSP can be used in EPS or not.

Solution #34 does not require normative work.

# * * * * End of changes * * * *