d3GPP TSG-SA2 Meeting # 152E (e-meeting)	S2-2205834

Elbonia, August 17 - 26, 2022

Source: 	Ericsson

Title:	Discussion on way forward for dynamic PCC support for MBS

Document for:	Discussion/Agreement

Agenda Item:	8.9

Work Item / Release:	5MBS

Abstract of the contribution: This paper discusses and proposes a way forward for dynamic PCC support for MBS.

# Discussion

## Potential use case of MBS using PCC

Figure 1 shows the MBS Session creation by AF in trusted domain, no dynamic PCC is involved for MBS Session and transparent mode is applied.

NOTE 1: Dynamic PCC for unicast PDU Session (i.e., unicast delivery) is independent of dynamic PCC for MBS delivery.

Figure 1 MBS Session without PCC (AF in trusted domain, transparent mode)

Depending on deployment, NEF and/or MBSF/MBSTF may be involved as follows and the MBS session creation request from the AF may be sent to MB-SMF via NEF and/or MBSF.

AF – MBSF – MB-SMF

AF – NEF – MB-SMF

AF – NEF – MBSF – MB-SMF

Possible (very limited?) use case to have dynamic PCC for MBS:

PCF (instead of MB-SMF) decides whether the requested QoS is compliant with SLA.

## SA2 situation till TS 23.247 v17.3.0

Regarding how the interaction with PCF should be performed, different approaches have been discussed:

Approach-1 (i.e., “Alt-0” below): MB-SMF interact with PCF, AF/NEF/MBSF not aware of the dynamic PCC

In TS 23.247 v0.2.0 (also in TR 23.757 conclusion), the proposal was that MB-SMF optionally interacted with PCF without AF/NEF/MBSF being aware of the dynamic PCC deployment for MBS (see clause 7.1.1.1 of TS 23247 v0.2.0) (see Figure 2). This proposal (denoted as “Alt-0”, also proposed in S2-2203932) is streamlined and can fulfill the known use case of MBS.

Figure 2 Alt-0: Optional MB-SMF interacting with PCF, AF/NEF/MBSF not affected

Approach-2: AF, NEF or MBSF interacting with PCF depending on deployment

From TS 23.247 v0.3.0, a different proposal was introduced with the motivation to apply the PCC framework for unicast delivery to MBS without discussion of the possible MBS use case.

Due to the fundamental difference between unicast delivery and MBS delivery, the alignment of MBS PCC with PDU Session PCC (which is “Alt-B” in clause 7.1.1.3 of TS 23.247 v17.3.0) does not turn out to be a working-well solution after ~1 year’s discussion, e.g.,

The design that MB-SMF response to MBS Session creation relies on step 26 Npcf_MBSPolicyControl_UpdateNotify from PCF is quite unusual. What if PCF authorization fails and no UpdateNotify is sent by PCF?

EN “Editor's note: How multiple service requirements with multiple application IDs are handled is FFS” is not resolved.

From TS 23.247 v17.2.0, an additional alternative (i.e., Alt-A in clause 7.1.1.3 of TS 23.247 v17.3.0) was introduced. In Alt-A, same issue as in Alt-B that MB-SMF response to MBS Session creation relies on step 26 Npcf_MBSPolicyControl_UpdateNotify from PCF.

Besides, both Alt-A and Alt-B in clause 7.1.1.3 of TS 23.247 contain redundant steps (see below) which are not aligned with SA request (S2-2200077) to consider Energy Efficiency as a guiding principle when developing new solutions:

In Alt-A, steps 17 &18 & 26 are redundant as the needed policy/QoS information can already be provided in step 16.

In Alt-B, steps 20-26 are redundant as the needed policy/QoS information can already be provided in step 16.

For Alt-B, MB-SMF sending Nmbsmf_MBSSession_UpdateNotify Request to NEF or MBSF in step 20 does not seem to work because the MBS Session is not created successfully yet in NEF or MBSF. Alt-B also contains patch-like proposals (e.g., if no real policy update is required, the PCF repeats previous policies or sends an empty update message).

## New proposal (Alt-new) from Offline discussions

There are several offline discussions from June to Aug among a few interested companies, more options are proposed, one of them is as follows:

Figure 3 Dynamic PCC for MBS, AF or NEF or MBSF interacting with PCF depending on deployment

At MBS Session creation, depending on the deployment,

- the AF, or MBSF or NEF need to delay the creation request and perform “TMGI allocation”.

- The “authorization” introduces additional signalling for successful MBS session creation.

- Besides, the direction of session binding must be from N7 to N5, which is opposite to the session binding in PCC framework for unicast PDU Session.

## Proposal

Alt-0 is a streamlined approach that can fulfill the known MBS use case and does not impose additional impact to NEF, MBSF or AF (i.e., agnostic to the deployment).

For Alt-A and Alt-B, the complexity and redundant steps do not bring any additional benefit compared to Alt-0, and there are still open points in both Alt-A and Alt-B, therefore in our view both Alt-A or Alt-B should be replaced.

Alt-new is more streamlined than Alt-A &-B, however, it does not bring more benefit than Alt-0 but introduces additional signaling for every MBS Session creation (e.g., additional TMGI allocation, additional PCF interaction from multiple NFs) and additional complexity (e.g., new session binding mechanism).

Based on the above, the following are proposed:

[Proposal-1] It is proposed to take out Alt-A and Alt-B from clause 7.1.1.3 and other clauses of TS 23.247.

[Proposal-2] If Alt-0 (i.e., a streamlined approach) is agreeable, it is proposed to adopt Alt-0.

[Proposal-3] If Alt-0 (i.e., a streamlined approach) is still not agreeable, it is proposed to modify Alt-new by making the early authorization in PCF optional and independent of the format of MBS Service Information (e.g., 3GPP QoS, SDP or Media Component) provided by the AF.

[Proposal-4] If neither Alt-0 nor modified Alt-new (optional early PCF authorization) is agreeable, it is proposed to remove dynamic PCC support for MBS in Rel-17 and use preconfigured PCC in MB-SMF (similar handling as in eMBMS).

# 2 Proposal

[Proposal-1] It is proposed to take out Alt-A and Alt-B from clause 7.1.1.3 and other clauses of TS 23.247.

[Proposal-2] If Alt-0 (i.e., a streamlined approach) is agreeable, it is proposed to adopt Alt-0.

[Proposal-3] If Alt-0 (i.e., a streamlined approach) is still not agreeable, it is proposed to modify Alt-new by making the early authorization in PCF optional and independent of the format of MBS Service Information (e.g., 3GPP QoS, SDP or Media Component) provided by the AF.

[Proposal-4] If neither Alt-0 nor modified Alt-new (optional early PCF authorization) is agreeable, it is proposed to remove dynamic PCC support for MBS in Rel-17 and use preconfigured PCC in MB-SMF (similar handling as in eMBMS).

For [Proposal-2], see 23.247 CR0125 (S2-2205836)