3GPP TSG SA WG2 Meeting #152E 	S2-2205877

Online, August 17-26 2022	(revision of)

Source:	Ericsson

Title:	Discussion on the Action points from SA#96

Document for:	Discussion and Agreement

Agenda Item:	8.28

Work Item / Release:	NR_slice_core/Rel-17

Abstract: This discussion paper provides background on action items identified at SA#96

# 1. Introduction

At SA#96 the following CRs from SA2#150 were approved

| S2-2203620 | CR | 23.501 CR3317R5 (Rel-17, 'B'): Enabling slice priority and slice groups for RRM purposes | Ericsson | Rel-17 |
| --- | --- | --- | --- | --- |
| S2-2203618 | CR | 23.501 CR3539R3 (Rel-17, 'B'): Enabling configuration of Network Slice AS Groups | Nokia, Nokia Shanghai Bell, CMCC, NEC, Verizon UK ltd, ZTE, InterDigital, Xiaomi, LGE, Samsung | Rel-17 |
| S2-2203619 | CR | 23.502 CR3300R3 (Rel-17, 'B'): Enabling configuration of Network Slice AS Groups | Nokia, Nokia Shanghai Bell, CMCC, NEC, Verizon UK ltd.,ZTE, InterDigital, Xiaomi, LGE, Samsung | Rel-17 |

However, S#96 also acknowledged there was more work to do on these and issued the following guidance to SA2#152:

TSG SA#96 asks SA WG2 to work on clarifying NR_Slice_Core in Q3. SA WG2 is asked to work on clarifying the Random access behaviour and how the NSAG priority is formed. Additionally, the sentence describing how Cell Reselection is disabled if the NSAG priorities are not sent should be clarified. Impacts on RAN shall be avoided. Interested companies also need to ensure that the corresponding stage 3 aspects are finalized at TSG CT#97.)

Indeed, the Random Access behaviour description is missing, there are some clarifications required on some text and  it can be clarified further how an NSAG priority is determined.

For the aspects subject of this paper and the SA#96 guidance, the FS_NR_slice objective was (see RP-201612):

Study mechanisms to enable UE fast access to the cell supporting the intended slice, including [RAN2] a. Slice based cell reselection under network control b. Slice based RACH configuration or access barring

Then, in the course of the study, RAN2 discussed the concept of intended slice but concluded it is not in their scope and so we need to figure out and specify in SA2 how the intended slice is linked to the NSAG and NSAG priority. RAN2 relies on SA2 to do so.

This paper proposed the rationale for selection of the Intended Slices and how this is implemented in the updates of TS 23.501.

NOTE: 	recent CT1 CRs change the split between NAS and AS. (see C1-224295 " Reply LS on Slice list and priority information for cell reselection"). Since this impacts the existing text in TS23.501, we propose also to update the TS so we no longer refer to the split between NAS and AS inside the UE.

TSG SA#96 asks SA WG2 to work on clarifying NR_Slice_Core in Q3. SA WG2 is asked to work on clarifying the:

1.	Random access behaviour and

2.	how the NSAG priority is formed.

3.	Additionally, the sentence describing how Cell Reselection is disabled if the NSAG priorities are not sent should be clarified.

Impacts on RAN shall be avoided. Interested companies also need to ensure that the corresponding stage 3 aspects are finalized at TSG CT#97.

# 2. Discussion

The following summarizes what is needed to address the aspects requested by TSG SA#96:

Random access behaviour

The UE logic for determining NSAG to use for Random Access needs to be specified, and as different Random Access resources may be assigned to different NSAGs the selected NSAG should reflect why the UE is making the access attempt. The access attempt may be related to all the S-NSSAIs that the UE is using (i.e. all S-NSSAIs of Requested NSSAI or Allowed NSSAI), or the access attempt may be related to a specific S-NSSAI, or the access attempt may be related to a set of S-NSSAIs. When the UE is using Random Access resources related to NSAG, the selected NSAG should be the highest prioritized NSAG related to S-NSSAIs that the access attempt relates to i.e. the S-NSSAIs that are triggering the access attempt. If, for whatever reason, the UE cannot use Random Access resources assigned to an NSAG as described above, then the UE may use the common Random Access resources.

Proposal 1: When the UE is using Random Access resources related to NSAG, the selected NSAG should be the highest prioritized NSAG related to S-NSSAIs that the access attempt relates to i.e. the S-NSSAIs that are triggering the access attempt.

How the NSAG priority is formed.

The NSAG is used for Access Stratum purposes i.e. RAN functionality. The operator determines the NSAG to provide in SIB16 using input customers, knowledge of its network and other input. OAM is used to set what is to be provided in SIB16.

To make the configuration and usage consistent, the OAM can configure the NSAG priorities to be applied consistently with operator preferences for how NSAGs are configured and used in NG-RAN.

Proposal 2: OAM configures the NSAG priorities in AMF such that they can be applied consistently with operator preferences for how NSAGs are configured and used in NG-RAN.

Sentence describing how Cell Reselection is disabled if the NSAG priorities are not sent should be clarified.

The mentioned sentence was added as to avoid that the UE determines and decides on priorities by itself that would impact cell re-selection and therefore NG-RAN performance. The sentence was needed as it was optional for the AMF to provide NSAG priorities to the UE. If the AMF is mandated to provide NSAG priorities when the AMF provides NSAG information to the UE, there would be no case when the UE does not receive any NSAG priorities.

Proposal 3: Mandate AMF to provide NSAG priorities when the AMF provides NSAG information to the UE and remove the sentence that UE shall not use Network Slice based cell reselection when the UE does not receive any NSAG priorities.

Further on the cell re-selection.

Current slice based cell re-selection logic does treat the S-NSSAIs with and without any established PDU Session equally. As in connected mode the NG-RAN will typically prioritize the mobility according to what DRBs are activated it is natural to already at the cell re-selection by the UE first consider S-NSSAIs with established PDU Sessions.

Proposal 4: slice based cell re-selection should consider S-NSSAIs with established PDU Sessions before S-NSSAIs without established PDU Sessions.

On Intended slice(s).

The RAN2 discussed the concept of intended slice but concluded it is not in their scope. The network slice or network slices that the UE intends to use are normally those triggering the UE access to the network or those that the applications been set to use e.g. via URSP that then results in UE registering network slices and establishes PDU Sessions for specific network slices. With regards to NSAG functionality it is natural that the intended slices are those that triggers the procedure leading to slice based RACH or the slices used as input to slice based cell reselection. For cell based cell reselection, SA2 already in 23.501 specified that the slices to consider are those S-NSSAIs in the Requested NSSAI or the Allowed NSSAI. Further, as proposed in proposal 4, the S-NSSAIs related to established PDU Sessions are further to be regarded as an intended slice. Then, for the process of deciding which NSAG to first consider for slice based AS functionality (RACH and cell re-selection), SA2 specified that it is the highest prioritized NSAG associated to the slices to be considered for the procedure. Therefore, it can be concluded that SA2 already worked on deciding what are the intended slices for cell re-selection and similar work need to be done for slices based RACH.

Proposal 5: The intended slices are those S-NSSAIs to be considered for the related slice based procedure.

# 3. Proposal(s)

It is proposed to update the specifications according to the conclusion above, namely:

Proposal 1: When the UE is using Random Access resources related to NSAG, the selected NSAG should be the highest prioritized NSAG related to S-NSSAIs that the access attempt relates to i.e. the S-NSSAIs that are triggering the access attempt.

Proposal 2: OAM configures the NSAG priorities in AMF such that they can be applied consistently with operator preferences for how NSAGs are configured and used in NG-RAN.

Proposal 3: Mandate AMF to provide NSAG priorities when the AMF provides NSAG information to the UE and remove the sentence that UE shall not use Network Slice based cell reselection when the UE does not receive any NSAG priorities.

Proposal 4: slice based cell re-selection should consider S-NSSAIs with established PDU Sessions before S-NSSAIs without established PDU Sessions.

Proposal 5: The intended slices are those S-NSSAIs to be considered for the related slice based procedure.

CRs aligned with the above proposals are available in S2-2205878 (23.501) and S2-2205879 (23.502)