3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205889

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	China Mobile, Huawei

Title:	KI#7, Sol#19: Update to remove FFS

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract: This paper proposes to update the Sol#19 to remove some FFS.

- Introduction/Discussion

This contribution proposes to remove the following two FFS.

Editor's note:	How NWDAF gets the UE list located in the finer granularity area is FFS.

For a finer granularity area smaller than a cell, the NWDAF can determine the Cell ID for the cell based on the local configuration of the mapping between the geo area and the network area. Alternatively, during the QoS sustainability analytics subscription by the AF via NEF, the NEF can also determine the Cell ID for the cell based on its local configuration and send the Cell ID to NWDAF.

To get a UE ID list located in the finer granularity area, the NWDAF can request the UE location information for all the UEs located in the cell then determine the UE ID list by itself. Alternatively, the NWDAF can also request LMF to provide the UE ID list located in the finer granularity area. In this, the NWDAF should include both the Cell ID and the finer granularity area information into the request service operation to the LMF.

Editor's note:	Whether the network data collection from data sources should be considered except from SMF/PCF, e.g. UPF, OAM(MDT), is FFS.

In addition to the input data from SMF/PCF, the NWDAF can also collect the UPF data, MDT data per UE within the finer granularity area from OAM. Note that the UPF data has been included in Sol#50, so this solution only list the MDT data.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-81.

# * * * * First change * * * *

6.19	Solution #19: Enhanced QoS Sustainability Analytics in the finer granularity area

6.19.1	Description

QoS Sustainability analytics is an important analytics service, especially for V2X-related or safety related applications, since it helps to handle the impact of a sudden change of the QoS and thus avoid a harsh application adjustment, which could affect safety and efficiency. However, there is the need to enhance the granularity and the accuracy of the QoS Sustainability analytics as well as to extend the QoS parameters that the QoS Sustainability notification can be provided. The latter will help to increase the usefulness of the specific analytics ID as well as to support the requirements of various use cases (e.g. V2X related).

More accurate QoS Sustainability outputs needs QoS and input parameters to be associated with more accurate location information, when input information is monitored at RAN. Also, the list of input features of the QoS Sustainability analytics ID should be extended by adding information on radio resource utilization and performance measurements for transfer over the UP to improve the correctness and accuracy of QoS sustainability output.

Additional QoS parameters (delay, packet loss rate) should be considered by the QoS Sustainability analytics and additional inputs for these QoS parameters are needed as inputs to the QoS Sustainability analytics. Finally, in the case that an area of interest is provided in the QoS Sustainability request message by an analytics consumer then the necessary granularity of the area of interest should be indicated.

The outputs of the QoS Sustainability analytics (statistics and predictions), should be provided for more granular location, according to the request with an area or a path of interest.

6.19.2	Procedures

6.19.2.1	General

According to the description in clause 6.9 in TS 23.288 [5] the consumer of QoS Sustainability analytics may request the NWDAF analytics information regarding the QoS change statistics for an Analytics target period in the past in a certain area or the likelihood of a QoS change for an Analytics target period in the future in a certain area or path of interest, which is smaller than a cell.

Editor's note:	If the AF cannot provide the certain area or path of interest, which is smaller than cell, it is FFS how the NWDAF determines an area smaller than cell for the QoS sustainability analytics.

The area is described in the request message (either of a Subscribe-Notify model or of a Request-Response model) via Location information (mandatory):

-	a path of interest: The location information could reflect a list of waypoints;

-	an area: The location information could reflect expected granularity levels of the requested area.

6.19.2.2	Input data

The table with input data of the QoS Sustainability analytics (TS 23.288 [5]) should be updated by adding information on radio resource utilization and input related to other QoS parameters (e.g. delay, packet error rate).

Table 6.19.2.2-1: Data collection for "QoS Sustainability" analytics (TS 23.288 [5])

| Information | Source | Description |
| --- | --- | --- |
| RAN UE Throughput | OAM TS 28.554 [9] | Average UE bitrate in the Area or path of Interest (Payload data volume on RLC level per elapsed time unit on the air interface, for transfers restricted by the air interface), per timeslot, per cell, per 5QI and per S-NSSAI. |
| QoS flow Retainability | OAM TS 28.554 [9] | Number of abnormally released QoS flows during the time the QoS Flows were used per timeslot, per cell, per 5QI and per S-NSSAI. |
| RAN status, load and performance information | OAM | Statistics on RAN status (up/down), load (i.e. Radio Resource Utilization) and performance in the Area or path of Interest as defined in TS 28.552 [7] |
| Packet Delay | OAM | Statistics on RAN delay measurements (e.g. Average delay UL/DL air-interface), Round-trip GTP Data Packet Delay, packet delay between NG-RAN and PSA UPF in the Area or path of Interest as defined in TS 28.552 [7] |
| Packet Loss and/or Drop | OAM | packet loss (UL/DL) information in the NG-RAN in the Area or path of Interest as defined in TS 28.552 [7] |

NOTE 1:	The timeslot is the time interval split according to the time unit of the OAM statistics defined by operator.

NOTE 2:	Whether the information listed in Table 6.19.2.2-1 can be collected in requested Area or path of Interest from AF should be dependent on RAN WG3/SA WG5.

With the OAM data per finer granularity area in Table 6.19.2.2-1, the NWDAF derives the "QoS Sustainability" statistics on the finer granularity area.

Alternatively, if the information listed in Table 6.19.2.2-1 can be collected from the OAM in requested Area or path of Interest, additional input data per UE within the finer granularity area for the QoS Sustainability analytics is shown in Table 6.19.2.2-2.

Note that the NWDAF should firstly determine the Cell list or TA list where requested area or path of Interest is included before the data collection from LMF, AMF, SMF and PCF.

For a finer granularity area smaller than a cell, the NWDAF can determine the Cell ID for the cell based on the local configuration of the mapping between the geo area and the network area. Alternatively, during the QoS sustainability analytics subscription by the AF via NEF, the NEF can also determine the Cell ID for the cell based on its local configuration and send the Cell ID to NWDAF.

To get a UE ID list located in the finer granularity area, the NWDAF can request the UE location information for all the UEs located in the cell then determine the UE ID list by itself. Alternatively, the NWDAF can also request LMF to provide the UE ID list located in the finer granularity area. In this, the NWDAF should include both the Cell ID and the finer granularity area information into the request service operation to the LMF.

Table 6.19.2.2-2: Additional data collection for "QoS Sustainability" analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp |  | A time stamp associated with the collected information. |
| UE ID | LMF, AMF | SUPI |
| UE locations (1..max) | LMF, AMF | UE positions located in the Cell or TA where requested area or path of Interest is included |
| >UE location |  | Geodetic Location that the UE is located |
| >Timestamp |  | A time stamp when the LMF calculates the UE is located in this location |
| QoS Notification Control of the QoS profiles or Alternative QoS Profiles | SMF/PCF | QNC or AQP events from SMF/PCF to indicate that the GFBR, the PDB or the PER of the QoS profile cannot be fulfilled or can be fulfilled again. |
| QoS profiles | SMF/PCF | QoS profiles corresponding to the QNC or AQP |
| Average UE throughput | OAM | Average UE throughput measurement separately for DL and UL, per DRB per UE and per UE for the DL, per DRB per UE and per UE for the UL, by gNB, see TS 28.552 [xx]. |
| Packet Delay | OAM | Packet Delay measurement separately for DL and UL, per DRB per UE, TS 28.552 [xx] and TS 38.314 [yy] |
| Packet loss rate | OAM | Packet loss rate measurement separately for DL and UL, per DRB per UE, TS 28.552 [xx] and TS 38.314 [yy]. |

NOTE:	The new input about QoS profile from SMF/PCF corresponding to the QNC or AQP will not trigger new RAN measurement.

Note that based on the current QNC/AQP notification mechanism, the SMF/PCF cannot know which parameter of the PER, PDB or GFBR cannot be fulfilled from the QNC/AQP="GFBR can no longer be guaranteed" event. On one hand, because the V2X Application based on the QoS sustainability analytics is quite safety sensitive, when QNC/AQP="GFBR can no longer be guaranteed", the NWDAF can default assume that all the GFBR/PDB/PER are not fulfilled in RAN and send the analytics to V2X AF to help application adjustment. Alternatively, the NWDAF can try to figure the reason out on its own by comparing the RAN notification (i.e. a new reference for the newly fulfilled QoS profile) with the previously fulfilled QoS profile, i.e. if only one of the values has changed the reason is that parameter (if two or three have changed, the NWDAF does not know which parameter(s) caused the problem).

This solution does not provide any enhancement on the current QNC/QAP triggering mechanism i.e. NWDAF can only collect the current QNC/AQP events triggered by the AF. It means that the signalling pressure is not too high since signalling is not collected for every QoS Sustainability request, Notification control is enabled when the QoS Notification Control parameter is set in the PCC rule (received from the PCF) that is bound to the QoS Flow.

NWDAF subscribes to the network data from AMF in the Table 6.19.2.2-2 by invoking Namf_EventExposure_Subscribe (Event IDs = Location Changes, Area of Interest=Cell list or TA list).

Editor's note:	How to collect the UE location from LMF is FFS and the cooperation with FS_eLCS_Ph3 is needed.

NWDAF subscribes the network data from SMF/PCF in the Table 6.19.2.2-2 by using the services provided by SMF/PCF together with the Cell list or TA list.

The correlation information in clause 6.2.4 of TS 23.288 [5] are re-used to correlate the data from LMF, AMF and SMF/PCF.

With the CN data per UE in the finer granularity area in Table 6.19.2.2-2, the NWDAF derives the "QoS Sustainability" statistics on the finer granularity area. For example, the NWDAF can derives the analytics as follows:

-	how many percent of the UE's bit rate/packet delay/packet error rate which can be or cannot be guaranteed on the finer granularity area (below cell level); or

-	average bit rate/packet delay/packet error rate can be or cannot be guaranteed on the finer granularity area (below cell level).

6.19.2.3	Output analytics

The NWDAF outputs the QoS Sustainability analytics, which should be provided according to the analytics request area of path of interest granularity. The provided granularity depends also on collected monitoring data (e.g. that the model has been trained).

Also, the Crossed Reporting Threshold(s) should be indicated for QoS parameter: Throughput, Delay, Packet loss.

Table 6.19.2.3-1: "QoS Sustainability" statistics

| Information | Description |
| --- | --- |
| List of QoS sustainability Analytics (1..max) |  |
| >Applicable Area | the Location information that the analytics applies to, the Area or path of Interest according to the analytics request |
| >Applicable Time Period | The time period within the Analytics target period that the analytics applies to. |
| >Crossed Reporting Threshold(s) | The Reporting Threshold(s) for each QoS parameter that are met or exceeded or crossed by the statistics value or the expected value of the QoS KPI. |

Table 6.19.2.3-2: "QoS Sustainability" predictions

| Information | Description |
| --- | --- |
| List of QoS sustainability Analytics (1..max) |  |
| >Applicable Area | the Location information that the analytics applies to, the Area or path of Interest according to the analytics request |
| >Applicable Time Period | The time period within the Analytics target period that the analytics applies to. |
| >Crossed Reporting Threshold(s) | The Reporting Threshold(s) for each QoS parameter that are met or exceeded or crossed by the statistics value or the expected value of the QoS KPI. |
| >Confidence | Confidence of the prediction. |

6.19.2.4	Procedures

The procedure in Figure 6.9.4-1 of TS 23.288 [5] is re-used with additional data collection step from LMF, AMF, etc.

Editor's note:	The procedure will be added when the service operation for LMF data collection is defined.

6.19.3	Impacts on existing nodes and functionality

NWDAF:

-	Monitor more information by OAM including radio resource utilization and input related to other QoS parameters or collects QNC/AQP events from SMF/PCF, UE Location from LMF or AMF.

-	The outputs of the QoS Sustainability analytics (statistics and predictions), should be provide:

-	Crossed Reporting Threshold(s) for QoS parameter: Throughput, Delay, Packet loss.

-	more granular location, according to the request with an area or a path of interest.

Consumer NFs:

-	In the case that an area of interest is provided in the QoS Sustainability request message by an analytics consumer then the necessary granularity of the area of interest should be indicated.

# * * * * End of changes * * * *