3GPP TSG SA2 Meeting #152e 	S2-2206090

17-26 August 2022, Electronic Meeting 	(revision of S2-22xxxxx)

# Source:	Xiaomi, China Mobile, MATRIXX Software, vivo

# Title:	New WID: 5GC supporting untrusted Non-3GPP Satellite Access Network (TEI18_5GUN3SAT)

# Document for:	Approval

Agenda Item:	10.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	5GC supporting untrusted Non-3GPP Satellite Access Network

Acronym:	TEI18_5GUN3SAT

Unique identifier:

Potential target Release:	Rel-18

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 740061 | Stage 2 of 5G System - Phase 1 | Rel-15 Stage 2 work item to specify 5GS architecture supporting Untrusted Non-3GPP access. |
| 820014 | Stage 2 of Wireless and Wireline Convergence for the 5G system architecture | Rel-16 Stage 2 work item to specify 5GS architecture enhancements supporting Trusted Non-3GPP Access. |
| 800048 | Stage 1 of 5GSAT | Service requirements of satellite access in 5G |
| 860005 | (Stage 2 of) Integration of satellite components in the 5G architecture | Baseline of 5G architecture to support satellite access |

3	Justification

In the existing 3GPP defined satellite access in 5GS (before Rel-18), the UE shall support NR(Satellite) to access 3GPP network via satellite, and the satellite NG-RAN is a NG-RAN which uses NR(Satellite) in providing satellite access to UEs.

There is a service requirement in Stage1 TS22.261(v18.6.1) ‘Service requirements for the 5G system’:

“A 5G system with satellite access shall support different configurations where the radio access network is either a satellite NG-RAN or a non-3GPP satellite access network, or both.”

How to support satellite access as non-3GPP access network in 5GS needs to be specified in Rel-18 according to the Rel-18 stage1 service requirement.

3GPP has specified untrusted non-3GPP access to 5GC since Rel-15 and trusted non-3GPP access to 5GC since Rel-16, which can be the baseline for the statellite access as non-3GPP access network in 5GS.

Since the trusted non-3GPP Access case has more change requirements to the Non-3GPP Satellite Access Network, e.g. TNAP and TNGF, a TEI18 work item on 5GC supporting untrusted Non-3GPP Satellite Access Network (TEI18_5GUN3SAT) is proposed. For the trusted Non-3GPP Satellite Access Network, it can be introduced in the future release.

4	Objective

The objective is to add description and procedures related to support untrusted non-3GPP satellite access to 5GC.

For policy control (e.g. QoS) and charging purpose, the RAT Type of the untrusted satellite access to 5GC needs to be specified.

Note 1: In this release, the N3SAT UE needs to support 5G NAS.

Note 2: The following UEs are in the scope: supporting untrusted Non-3GPP Satellite access and 3GPP defined access (TN and/or NTN), supporting only untrusted Non-3GPP satellite access.

This work item will require 0.5 TU(s) to discuss and agree on the corresponding CRs.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Add description and architecture for untrusted non-3GPP satellite access to 5GC | SA#99 (Mar. 2023) |  |
| TS 23.502 | Add description and procedures for untrusted non-3GPP satellite access to 5GC | SA#99 (Mar. 2023) |  |

6	Work item Rapporteur(s)

Chunhui Zhu (Xiaomi), zhuchunhui@xiaomi.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

SA5 for charging aspects.

9	Supporting Individual Members

Supporting IM name

Xiaomi

China Mobile

MATRIXX Software

vivo