3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206094

Elbonia, August 17 – 26, 2022

Source:	Huawei, HiSilicon

Title:	Update solution#14 to remove EN

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR / Rel-18

Abstract: Updating the solution#14 to address the EN.

1. Discussion

This contribution proposes to address the following EN of Solution#14:

Editor's note: It is FFS how to handle the case where the UE and MBSR are from different PLMNs.

The issue described in the above EN is that when the HPLMN of IAB-UE and the HPLMN of UE is different, the UE-AMF cannot determine the UDM located in HPLMN of IAB-UE due to the absence of IAB-UE context in the UE-AMF.

In order to address this EN, it is proposed to store the Cell ID of IAB node, IAB-UE ID and the identifier of AMF serving IAB-UE into the PLMN level NRF, which is assumed to be accessible by any AMF within the whole Serving PLMN. The PLMN level NRF ID is pre-configured in the AMFs. Therefore, even though the UE and MBSR are from different PLMNs, the IAB-AMF stores the information of IAB node in the PLMN level NRF and the UE-AMF queries the PLMN level NRF using the Cell ID of IAB node to obtain the IAB-UE ID and IAB-AMF ID.

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-05 V0.3.0.

# * * * * First change * * * *

6.14	Solution #14: Support of location services for UEs accessing via a mobile IAB node

6.14.1	Introduction

This solution addresses the KI#5: Support of location services for UEs accessing via a mobile IAB node.

6.14.2	Functional Description

This solution is based on the following principles:

-	The IAB-UE provides the serving Cell ID of IAB node to the core network. The NRF stores the association of IAB-UE ID, the identifier of AMF serving IAB-UE and Cell ID of IAB node.

-	The UE-AMF determines the UE is accessing via a mobile IAB node based on the additional ULI related to the IAB-UE included in the NGAP message as described in the Solution#9. When the location reporting procedure for UE is needed, the UE-AMF queries the IAB-UE ID and the identifier of AMF serving IAB-UE from NRFusing Cell ID that UE is camping.

-	The UE-AMF sends Location Request to the LMF including the LCS Correlation identifier for UE, IAB-UE ID and identifier of AMF serving IAB-UE. The LMF additionally performs IAB-UE positioning procedure.

-	The LMF determines the location of UE by taking the location of the IAB-UE into account.

6.14.3	Procedures

Figure 6.14.3-1: Positioning procedure for UE accessing via mobile IAB node

1.	After the cell is configured and activated in the mobile IAB-node, the IAB-UE initiates Registration procedure and provides the Cell ID of IAB node to the 5GC. The AMF serving IAB-UE (i.e. IAB-AMF) sends the Cell ID of IAB node, IAB-UE ID (i.e. SUPI) and the identifier of AMF serving IAB-UE to the NRF .

If the Cell ID of IAB node is changed during mobility, the IAB-UE provides the new Cell ID to the IAB-AMF to trigger the cell information update in the NRF.

NOTE: It is assumed the NRF is at PLMN level and it is accessible by any AMF within the whole Serving PLMN. The ID of PLMN level NRF is pre-configured in the AMFs.

2.	The UE is camped on the cell served by the mobile IAB-node and registered to the network.

3.	The AMF serving UE (i.e. UE-AMF) initiates the location reporting procedure for UE, as defined in the 5GC-MT-LR procedure or the 5GC-MO-LR procedure of TS 23.273 [4].

The UE-AMF determines the UE is accessing via mobile IAB-node based on the additional ULI related to the IAB-UE included in the NGAP message as described in the Solution#9.

4.	The UE-AMF sends Request message to the NRF including the Cell ID that UE is camping.

5.	The NRF returns the IAB-UE ID and the identifier of AMF serving IAB-UE corresponding to the Cell ID.

6.	The UE-AMF selects LMF and invokes Nlmf_Location_DetermineLocation Request service operation towards the LMF including the LCS Correlation identifier for UE, the IAB-UE ID and the identifier of AMF serving IAB-UE.

7.	The LMF performs one or more of the positioning procedures as described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [4].

8.	The LMF triggers the AMF serving IAB-UE to initiate the location reporting procedure for IAB-UE based on the identifier of AMF serving IAB-UE received in the step 6. The UE positioning procedure in step 7 and IAB-UE positioning procedure in step 8 are performed at the same time.

During this step the LMF receives the location of the IAB-UE.

9.	The LMF determines the location of UE by taking the location of the IAB-UE into account and provides the UE location to the AMF serving the UE.

NOTE: How the LMF determines the location of UE by taking the location of the IAB-UE into account is up to LMF implementation.

6.14.4	Impacts on services, entities and interfaces

IAB-UE:

-	Supports providing current Cell ID of IAB node to the AMF.

IAB-AMF:

-	Supports sending the Cell ID of IAB node to the UDM.

PLMN level NRF:

-	Supports the storage of the mapping between a Cell ID and an IAB node.

-	Supports finding an IAB-UE ID and the identifier of AMF serving IAB-UE based on Cell ID.

UE-AMF:

-	Supports requesting the IAB-UE ID and the identifier of AMF serving IAB-UE from UDM using Cell ID of IAB node.

LMF:

-	Supports triggering the positioning procedure for IAB-UE based on the IAB-UE ID and the identifier of AMF serving IAB-UE.

-	Determines the location of UE by taking the location of the IAB-UE into account.

# * * * * End of changes * * * *