3GPP TSG-WG SA2 Meeting #152E	S2-2205983

Aug 17 – 26, 2022

Source:	Rakuten Mobile

Title:	New Solution: Reusing existing procedures for Control of UE's access to 5GS via a mobile base station relay

Document for:	Approval

Agenda Item: 	9.9

Work Item / Release:	FS_VMR

1	Discussion

For Key Issue #7: Control of UE's access to 5GS via a mobile base station relay, it is required that the support of legacy UE(s) shall be considered which means Rel-15 UEs without any specific capabilities. To achieve this the operator should apply specific tracking area for VMR and provide the access control for the UE with existing solution.

2	Proposal

It is proposed to adopt the following changes into TS23.700-05

* * * * Start of Change * * * *

6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | x |  |  |  |  |  |  |
| 2 | x |  |  |  |  |  |  |
| 3 |  | x |  |  |  |  |  |
| 4 |  | x | x |  |  |  |  |
| 5 |  | x | x |  |  |  |  |
| 6 |  |  |  | x |  |  |  |
| 7 |  |  |  |  | x |  |  |
| 8 |  |  |  |  | x |  |  |
| 9 |  |  |  |  |  | x |  |
| 10 | x |  |  | x |  |  |  |
| 11 |  | x |  |  |  |  |  |
| 12 |  |  | x |  |  |  |  |
| 13 |  |  | x |  |  |  |  |
| 14 |  |  |  |  | x |  |  |
| 15 |  |  |  |  | x |  |  |
| x |  |  |  |  |  |  | x |

* * * * Next Changes * * * *

6.X	Solution #X: Reusing existing procedures for Control of UE's access to 5GS via a mobile base station relay

6.X.1	General

This is a solution to Key Issue#7, which addresses the following:

-	Whether and how to manage access control for the UE's access to IAB-node;

-	whether and how to manage the UE's access (e.g. permitted/restricted) to IAB-node for specific geographical location(s), time window(s), or other conditions/criteria;

In this solution mainly focus on the legacy UEs without any capabilities.

6.X.2	Functional descriptions

In case the operator recognizes the specific area for the MBSR, existing Mobility Restriction mechanism specified in clause 5.3.4.1 of TS 23.501[2] can be applied.

6.X.3	Procedures

Existing procedure will be reused i.e. UE's subscription may include access restriction for MBSR(e.g. Tracking Area) and provided/updated to the UE during the Registration procedure.

6.X.4	Impacts on services, entities, and interfaces

There is no impacts identified in this solution in case existing IE(e.g. Forbidden Tracking Area) can be re used.

Editor's note:	The impacts will be coordinated with CT/RAN WGs, if any.

* * * * End of Changes * * * *