SA WG2 Meeting #152e	S2-2206100

# August 17th – 26th, 2022; Elbonia               	  			  (revision of S2-220)

Source:	Nokia, Nokia Shanghai Bell, Broadcom

Title:		new TEI18 on the Support of Qos Monitoring also in the Non 3GPP access (N3IWF/TNGF) case

Document for:	Approval

Agenda Item:	10.4

Work Item:	TEI18_SDNAEPC

Abstract:		new TEI18 on the Support of Qos Monitoring also in the Non 3GPP access (N3IWF/TNGF) case.

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Support of Qos Monitoring also in the Non 3GPP access (N3IWF/TNGF) case

## Acronym: TEI18_N3ANQOSM

## Unique identifier: to be allocated by MCC

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don't know |  |  |  |  |  |

AN impacts are not for NG RAN.

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

{Not applicable for Feature nor for a Study Item}

{For a Building Block: list here the parent Feature }

{For a Work Task: list here the parent Building Block }

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

## 3	Justification

QoS monitoring has been defined in TS 23.501 clause 5.33.3 but only for 3GPP access. This means that following features are not possible for application traffic over Untrusted/Trusted non 3GPP access to 5GC including for UE access over Wireline:

- statistics / SLA monitoring / trouble shooting for sensitive applications / users over Untrusted/Trusted non 3GPP access to 5GC

- adapting the application behaviour based on QoS monitoring results for applications over Untrusted/Trusted non 3GPP access to 5GC

- AF guiding the UE (via URSP) to use another access when the current access does not meet QoS requirements

.

## 4	Objective

The objective of this WID is to specify QoS monitoring over Untrusted/Trusted non 3GPP access to 5GC according to following principles:

- SMF is able to send a QoS Monitoring request to a N3IWF/TNGF via N2 signalling (PDU Session resource create/modify) as in the NG RAN case

- The N3IWF/TNGF based on this request exchange IKE signaling with the UE to measure the delay with the UE

- The N3IWF/TNGF reports the measured delay using the same mechanisms as defined for NG RAN case in TS 23.501 clause 5.33.3.2

- 5GC reports measurements done over N3GPP access to the AF using the same mechanisms as defined for NG RAN case in TS 23.501 clause 5.33.3.2

- (optionally) The AF may then use Application guidance for URSP determination defined in clause 4.15.6.10 to update Access Type preference for the PDU Session carrying traffic whose monitored delay does not meet applicative requirements

.5 TU(s) are foreseen to complete this normative work.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2: The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Support of Qos Monitoring also in the Non 3GPP access (N3IWF/TNGF) case | SA#99 March 2023 |  |
| 23.502 | Support of Qos Monitoring also in the Non 3GPP access (N3IWF/TNGF) case | SA#99 March 2023 |  |

## 6	Work item Rapporteur(s)

Laurent.thiebaut@nokia.com

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

SA3 for security aspects. SA5 for management and charging aspects.

## 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

Broadcom