3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206116

Elbonia, Aug17th – 26th, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	Conclusion for FS_5TRS_URLLC KI#5

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract: Providing Conclusion for KI#5.

1. Introduction/Discussion

Based on evaluations of solutions for KI#5, it is proposed to update the conclusion for KI#5.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-25.

# * * * * First change (all new) * * * *

8.x	Key Issue #5: Interworking with TSN network deployed in the transport network

The following bullet points summarize the principles for the way forward:

- 	NG-RAN/NG-TT and UPF/TNW-TT may support LLDP to report the topology to CNC.

-	SMF may allow information access with the collocated CUC to support UNI as described in IEEE P802.1Qdj [10] and determine the traffic requirements for a QoS Flow and initiate that CUC translates them to merged stream requirements which are then passed to the CNC in Transport Network.

-	NG-RAN may determine a BAT offset in UL direction at the gNB egress, based on the BAT in UL direction the NG-RAN receives from the SMF in TSCAI. The NG-RAN may provide the BAT offset value to the SMF in a response to the QoS Flow establishment or modification request.

-	UPF may assign a separate CN tunnel end point address for each QFI of the N4 Session.

-	NG-RAN may provide stream requirements to transport network CNC, and allocate separate tunnel end-point destination addresses per QoS Flow (for QoS flows with TSCAI) used for the N3 tunnels (existing functionality).

-	NG-RAN/UPF may support the functionality of Listener/Talker.

# * * * * End of changes * * * *