3GPP SA WG2 Meeting #152E	S2-2206434

E-meeting, 17-26 Aug 2022

Source:	CATT

Title:	KI #2: Updating conclusion for key issue #2

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This paper to update the conclusion for KI#2.

1. Discussion

It is proposed to update the conclusion based on update of solution #7 in pCR S2-2206433.

2. Proposal

It is proposed to agree the following texts in clause 8.2 of TR 23.700-25.

>>>> Start of Changes <<<<

8.2	Key Issue #2: Time synchronization service enhancements

The following bullet points summarize the principles for the way forward:

-	To request a Requested Coverage Area for time synchronization services, AFs within the operator's domain formulates a spatial validity condition using a list of Tracking Area or Cell identities, but AFs outside the operator's domain use a geographical area (e.g. a civic address or shapes) instead, while NEF transforms this information into a list of Tracking Area or Cell identities.

-	The TSCTSF is responsible of activate/deactivate time synchronization services (ASTI or (g)PTP based) considering the spatial validity condition provided by the AF.

-	The TSCTSF discovers the AMF(s) serving the list of TA(s)/Cell(s) that comprise the spatial validity condition using the NRF.

-	The TSCTSF subscribes to UE's presence in Area of Interest at the discovered AMF(s).

-	The TSCTSF determines (based on notifications from the discovered AMF(s)) whether the targeted UE(s) are inside or outside the AF Requested Coverage Area.

-	The TSCTSF activates time synchronization services for UE(s) that are inside the Requested Coverage Area.

-	TSCTSF notifies the AF whether the targeted UE(s) are inside or move inside or outside of the Requested Coverage Area.

-	Determines whether activate/deactivate time synchronization services based on combining conditions (Spatial Validity Condition and Temporal Validity Condition).

>>>> End of Changes <<<<