SA WG2 Meeting #S2-152E	S2-2206901

17 – 26 August , 2022, E-meeting

Source:	Xiaomi

Title:	evaluation on enhancement of network selection for key issue#4

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_Ph2/Rel-18

Abstract of the contribution: This paper proposes the evaluation for key issue#4 on enhancement of network selection

*** Start of changes ***

7	Evaluation

Editor's note:	This clause provides an evaluation of the solutions.

7.3 Evaluation for Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services

Enhancement for network selection to support discover, select and access the NPN as hosting network.

- enhance existing network selection specified in TS 23.122 [6], by defining a new list of hosting network(s) that used for UE to perform automatic network selection to access the hosting network for the localized service, including NPN ID, optional valid time, area information (solution#33)

- UE is configured with a "prioritized list of SNPNs for localized services" for SNPN hosting network for localized services, including: (solution#10)

  - Identification Mode 1: SNPN ID (PLMN ID + NID) or a Group ID for Network Selection (GIN)

  - Identification Mode 2: SNPN ID or GIN, plus an additional Subnetwork ID.

  - Validity information: time period or additional location information

- UE is configured with Validity information of the "prioritized list of SNPNs for localized services" will contain the Cell Identifiers / Tracking area(s). Serving network will broadcast the availability of hosting network (i.e. indication of availability of one or more hosting network: YES/NO) to all the UEs in that area.(solution#29)

- The UE is (pre)-configured or provisioned (signalled) by Home Network with a List of SNPN Hosting Networks with Validity Information (i.e. time and/or location) and an Indication that the UE is only allowed to select a Hosting Network upon Steering by the Home Network (solution#30)

*** End of changes ***