3GPP TSG-SA WG2#152E e-meeting	S2-2205589r01

Electronic, 17-26 August 2022	Revision of S2-220xxxx

Source:	Ericsson, AT&T, Intel, Samsung, NTT DOCOMO

Title:	DetNet conclusions

Document for:	Approval

Agenda Item:	9.6

Work Item / Release:	FS_DetNet / Rel-18

Abstract of the contribution: We discuss the main outstanding issues in the DetNet study and propose a conclusion.

# Discussion of the main DetNet outstanding questions

The solutions in the DetNet study TR 23.700-46 have a lot of overlap between them, but there are also differences and open issues. We consider the main open questions and propose a resolution for them.

Which protocol is used between 5GS and DetNet controller?

The IETF DetNet work aims to provide deterministic service on the IP layer, so that a number of underlying technologies, including the 5G system, can make use of the DetNet framework. The DetNet solution uses IETF protocols for this. To make the 5G integration of DetNet useful, we need to use IETF protocols as described for DetNet for the signalling between 5GS and the DetNet controller.

The DetNet framework uses YANG models which can be carried over Netconf or Restconf. It is proposed that the 5GS integration of DetNet follows the same approach and uses YANG models carried over Netconf or Restconf between the TSCTSF and the DetNet controller. We suggest leaving it to the needs of the actual deployments whether Netconf or Restconf is chosen according to the market demand.

Conclusion 1: YANG models over Netconf or Restconf are used between the TSCTSF and the DetNet controller.

Is an optional NEF supported between TSCTSF and DetNet controller?

In the TSN integration case, there is no NEF used in the signalling from the CNC and the TSN AF, since the TSN AF as well as the CNC are considered trusted entities, and the operator has the necessary business agreements in place. Similarly, the DetNet integration can be supported with a trusted DetNet controller without the need to insert additional entity such as the NEF into the signalling path. Note that the DetNet controller is capable of influencing the QoS for the whole network, not just for a single UE as in the case of an AF, and that is something for which a trusted relationship with the operator can be assumed. Note also that the TSCTSF can also perform a verification of the requests from the DetNet controller and can reject the requests which are deemed unacceptable as per operator policy.

Involving the NEF in the signalling path between TSCTSF and DetNet controller is not easy from the signalling protocol point of view. According to Conclusion 1, an IETF protocol (Restconf or Netconf) is used to carry YANG models, but these protocols do not have a standardized way to insert a new entity acting as a proxy into the signalling path. Further, 3GPP normally uses a 3GPP defined signalling between two 3GPP entities such as the NEF and the TSCTSF, but in this case we would need to either use an IETF protocol or perform protocol conversion which is unnecessarily complex. Overall, the involvement of the NEF into the signalling would require a significant protocol extension of the IETF signalling framework, which is beyond the scope of the current study, and the need for such complexity is not justified.

In deployments where the security functions of NEF would be desirable, it can be considered to include relevant security functions in the TSCTSF. As described in 23.501 section 6.2.5.0 concerning NEF functionality, the NEF may authenticate and authorize and assist in throttling the Application Functions. Similar to the NEF, the TSCTSF may also perform the authentication and authorization of the signalling with the DetNet controller, and if the risk of excessive signalling arises, the TSCTSF may also help to throttle the signalling. The need for such functionality can be a deployment decision.

Conclusion 2: 3GPP does not standardize any signalling mechanism to include the NEF into the signalling path between the TSCTSF and the DetNet controller. If NEF functionality is desired, the relevant functions such as the authentication, authorization and potential throttling of signalling can be achieved by including such functionality in the TSCTSF depending on the needs of the given deployment.

Which entity collects and provides exposure information to the DetNet controller?

From the point of view of the DetNet controller, the 5GS acts as a router, and the management interface to the 5GS router has to have a single termination point, which should be the TSCTSF according to the assumptions of the study. There cannot be a different termination point for exposure (KI#1) and a different termination point for the parameter mapping (KI#2). The existing Rel-17 TSC procedures can be re-used to collect information from the UPF/NW-TT and the SMF, adding information elements as needed.

Conclusion 3: The TSCTSF terminates the interface towards the DetNet controller. The TSCTSF collects and provides exposure information to the DetNet controller. The TSCTSF collects the information from the UPF/NW-TT and the SMF based on the existing procedures in Rel-17 TSC.

How do we map e2e DetNet requirements to per 5GS requirements?

The DetNet configuration provides currently only e2e requirements (such as for the delay and the packet loss rate) and not per node level requirements. In other technologies, such as with fixed routers, the delay can be sufficiently controlled by existing configuration e.g., for bandwidth management, but in the case of 5GS, the required delay for the 5GS is needed as an input to the QoS setup.

As described in Solution #3, there can be two ways to perform this mapping. In simpler network deployments, the TSCTSF can be pre-configured to map the e2e requirements to per 5GS node requirements using a simple translation that is determined based on the knowledge of the given deployment. The requirements determined this way are sufficiently strict to guarantee the fulfillment of e2e requirements. In more complex network deployments, the 5GS can extend the IETF defined YANG model with a per 5GS specific delay requirement in addition to the e2e maximum delay, and the DetNet controller can make use of this additionally provided optional parameter. This applies to scenarios where both the 5GS and the DetNet controller support such an extension. In practice, YANG models are often extended according to the needs of the deployments; in our case, we can make use of the extensibility of the YANG framework by defining optional parameters that can extend the existing e2e requirements based on the needs of the 5GS. Note that if later on, IETF also defines a similar parameter extension, that can also be easily adopted; however there is currently no such work ongoing in the IETF due to the fact that typically fixed routers do not have an independent delay parameter to configure. Therefore, we propose to define such a 5GS specific parameter extension in 3GPP which can be specified without affecting the IETF YANG specification.

Conclusion 4: The TSCTSF may use the e2e traffic requirements in the YANG configuration, and based on a pre-configured mapping, derive sufficiently strict 5GS requirements from them. Alternatively, 3GPP may define a new YANG module (e.g. 3gpp-yang) that extends the IETF DetNet YANG modulewhich allows the DetNet controller to explicitly provide the 5GS traffic requirements.

How is routing information handled?

In Solution #8, the uplink routing information in the UPF/NW-TT is exposed to the DetNet controller. But this may be difficult in practice, since the routing functionality on N6 is up to implementation outside of 3GPP scope, hence defining 3GPP signalling for this purpose may be unnecessarily complex to realize and not in line with the 3GPP concept that keeps N6 routing outside the 3GPP scope.

Note also that DetNet as a technology does not require the collection of routing information from the routers; it is sufficient for the DetNet controller to collect topology information. Best effort traffic can use the existing routing mechanisms; while for DetNet flows in general, the controller may provide explicit routes. In the case of 5GS, however, it is outside the study scope to influence the routing in the 5GS. That is not necessary either, since in the 5GS deployments, we have IP endhosts in the terminal devices associated with the PDU Sessions, and the routing to the PDU Sessions is defined by the assigned IP addresses corresponding to the PDU Sessions. For the routing functionality on N6, that is proposed to be handled outside the scope of 3GPP. (In more complex deployments where there are multiple N6 interfaces and the controller needs to influence the uplink interface, the DetNet controller may explicitly interface and control the N6 router. In simpler deployments, such control may not be needed if there is only a single route possible through the 5GS for DetNet flows.)

Conclusion 5a: It can be possible for the 5GS to verify in the TSCTSF whether the explicit routing information provided by the DetNet controller is in line with the 5GS routing. Apart from the verification, the 5GS routing is not modified by the DetNet controller in line with the agreed scope of the work. There is no need to standardize the exposure of N6 routing information.

Alternative view on this question

In centrally configured DetNet networks it is in general enough that the Controller just learns the network topology, not necessarily the current routing state of the network. This is because the Controller is assumed to be able to enforce the path for the DetNet traffic flows by installing explicit routes for them.

However, in 3GPP Rel-18 it is assumed the Controller cannot install explicit routes on the 5GS DetNet Node, also all DetNet traffic flows will be forwarded according to the “normal” routing state in the 5GS. The Controller is still required to know and provide the ingress (source) and egress (destination) interfaces of the traffic flows to the 5GS DetNet Node.

This can only work under special circumstances: First, there could not be any alternative paths via alternative egress interfaces from the 5GS DetNet Node to any IP destination prefix, as the DetNet controller would not be able to know or influence which interface is used. This would mean the 5GS DetNet Node could practically only have a single UPF N6-side interface. Also, it would not be possible to support topologies where the 5GS DetNet Node forwards any IP prefixes other than the one allocated to the UE itself via the UE side interfaces, as the Controller would not know via which UE side interfaces packets to (or even from) them are forwarded.

These limitations should be resolved by these methods:

- Allowing multiple UPF N6-side interfaces: If 5GS DetNet Node exposes its UPF N6-interface specific IP forwarding/routing tables, the Controller can see which N6-side egress interface is used for which destination prefix. The information would be sent from UPF/NW-TT to TSCTSF with PMIC or UMIC.

- Allowing additional IP prefixes behind the UE-side interfaces: If 5GS DetNet Node exposes the UE-interface specific Framed Route and IPv6 prefix delegation information, the Controller can see which UE-side egress interface is used for which destination prefix. This information would be exposed to TSCTSF by SMF or UDM.

Note that the UPF N6-side interface routing would not really be impacted by this as such. Its state would just be exposed to the TSCTSF and from there to the DetNet controller. Note also that without the Controller being able to install explicit routes, it seems only static routes would be feasible for DetNet traffic flow related IP prefixes in order to prevent the traffic path suddenly changing. Framed routing and IPv6 prefix delegation are already supported by the 5GS and it would be just required to expose this information to the TSCTSF.

TSCTSF would expose the information to the Controller using (e.g.) IETF RFC 8345 YANG data model.

In the longer term 5GS DetNet Node should support Controller provided explicit routes but this is out of scope of Rel-18 according to the Study Item Description.

Conclusion 5b: It should be possible for UPF/NW-TT to provide UPF N6-interface specific IP routing/forwarding table entries to TSCTSF, and SMF/UDM to provide UE-interface specific IP framed route and IPv6 prefix delegation information to TSCTSF. This would allow TSCTSF to expose this information to the DetNet Controller using (e.g.) RFC 8345 YANG data model, and the Controller to use this information in all cases to determine the correct ingress and egress interfaces for the DetNet traffic flows.

Should there be an NW-TT?

Regardless of whether only interface and topology (neighbor) or also routing/forwarding related information is provided from the UPF N6-side interfaces to the Controller, the information has to be first provided from the UPF to the TSCTSF. In many solutions this is assumed to be provided using similar PMIC or UMIC containers as in Rel-16 TSN and Rel-17 TSC support. In some solutions it is proposed that the entity providing the information logically separate from UPF and called NW-TT as in Rel-16 and Rel-17, while in other solutions it is proposed that no NW-TT is necessary for Rel-18 DetNet support and the UPF can provide information directly.

It is true that Rel-18 DetNet will not require any similar User Plane packet processing or manipulation as done in Rel-16 TSN for the Time Aware Shaper or in Rel-16 and Rel-17 for (g)PTP. Only exposure of information is required, there is no configuration either. It seems however most consistent with the TSC work done in earlier releases that also in DetNet the entity providing the information and thus interfacing with UPF’s N6 IP and Layer 2 interface information and generating the Management Information Containers is still kept logically separate from the UPF and called NW-TT.

Conclusion 6: It is proposed that the entity exposing 5GS DetNet Node and its N6 interface related information via PMIC or UMIC is still kept logically separate from the UPF and called NW-TT in a consistent manner with the Rel-16 TSN and Rel-17 TSC related definitions.

What type of network topologies should be supported behind the UE-side interfaces and what information 5GS should expose to the Controller about the UE-side interfaces?

Solution #1 states the following:

The interfaces of the 5GS acting as a DetNet node can be reported based on the YANG model in RFC 8344 [6]. That model is in turn based on RFC 8343 [7]. The model includes for each interface

-	if-Index and name to identify the interface (see below for more details);

-	type of the interface, which can identify whether it is a 3GPP interface on the device side or a fixed interface on the network side;

-	IP address and subnet;

-	when available in the case of network side interfaces, list of neighbour IP address and link layer address (which could be based on ARP or IPv6 neighbour discovery). In the case of device side interfaces the solution does not assume neighbours; only the hosts that are reachable with the assigned IP address(es) on the given PDU Session are reachable.

It seems to be a major limitation if 5GS DetNet integration can only support network topologies where the Application Endpoint runs in the same device as the UE, and it is not possible to support topologies where the UE is integrated with a Router and has additional IP subnets/prefixes connected to it via Ethernet or other non-3GPP interfaces.

Rel-16 5GS Bridge type of solution where the UE and DS-TT together form an Ethernet-based port for the 5GS Bridge is not feasible to follow as UE (and DS-TT) impacts are out of scope for Rel-18.

Exposing framed route and IPv6 prefix delegation information about the UE-side interface to the TSCTSF and from TSCTSF to the Controller via RFC 8345 YANG data model as proposed in Conclusion 5b seems to address this issue partially. The Controller can with that information, e.g., map DetNet DL traffic flow destination IP addresses to correct 5GS DetNet Node UE-side egress interfaces even if the IP addresses are from IP prefixes located behind the UE in the network topology (for example corresponding to Framed Routes).

It would however be also useful to support topology discovery for the case where 5G UE device implements IP router functionality and exposes its interface and topology information to the DetNet controller separately from the 5GS DetNet Node. There special care should be paid to how each node reports its 3GPP interface information so that the Controller can link them as neighbours. It should be noted that in that model the UE logically belongs to the 5GS DetNet Node and the link between the 5GS DetNet Node and the separate 5GS UE Device Router Node resides logically inside the physical device as depicted below.

While it is out of scope of 3GPP Rel-18 what the “5G Mobile Device Logical DetNet Node” will report about its 3GPP interface, the goal should be that both sides report consistently using RFC 8343 and 8344 YANG data models.

On the IP layer each side basically only has the UE IP address or UE IPv6 prefix as known identifiers for the interface. There are many approaches to use and interpret them. For instance:

5GS DetNet Node reports:

- Interface-type=”3GPP”, Role=”Network”, IP-address=”[UE IP address]”, Prefix=”[UE Prefix]”

The Mobile Device Router reports:

- Interface-type=”3GPP”, Role=”UE”, IP-address=”[UE IP address]”, Prefix=”[UE Prefix]”

where Role would be new 3GPP-defined YANG model extension for “3GPP” interface type. The controller would be able to explicitly link the interfaces with this information.

NOTE: This mechanism does not require UE updates.

There are also other approaches how to use or extend the RFC 8343 and/or 8344 data models for this purpose. It seems also in this case it might be best to explain the issue to IETF and ask if there is already a best practice solution for it or whether 3GPP should work on an extension related to the “3GPP” interface type.

Conclusion 7: It should be possible to support DetNet network topologies where there are external IP hosts or IP routers connected to the 5GS DetNet Node UE-side interfaces, and where 5GS forwards multiple IP prefixes to the UE side interface, not just the IP address/prefix allocated to the UE. One step to address this is already included in Conclusion 5b, namely exposing the UE-related framed route and IPv6 prefix delegation information to the TSCTSF and the DetNet controller. The other step needed is to agree how best using RFC 8343 and 8344 YANG data models 5GS DetNet Node and a 5G Mobile Router should report their “3GPP” interface information so that the Controller can link the interfaces in its topology view. It is proposed to ask guidance on this from the IETF and based on that potentially define a 3GPP-specific YANG data model for the “3GPP” interface type according to RFC 8343.

Architecture

The following figure illustrates the DetNet architecture in line with the conclusions above.

Which information is exposed from 5GS to the DetNet controller?

The 5GS is exposed by the TSCTSF to the DetNet controller as a router on a per UPF granularity. Based on the discussions in Solution #1 and Solution #8, the following information is proposed to be exposed from TSCTSF to DetNet controller. The node may be identified by a Node ID. The interfaces correspond to the PDU Sessions and to the network side interfaces. Each interface is identified by an interface identifier.

- The following information may be reported from TSCTSF to DetNet controller for each interface. Type of interface

- IP address,

- subnet (prefix length)

- Neighbour address (in case of network side interfaces)

- MAC address (in case of network side interfaces)

- MTU size

The TSCTSF collects the information from the UPF/NW-TT and the SMF.

An alternative with additional information exposed related to 5GS Node IP routing/forwarding state is provided in the Conclusions.

How does the TSCTSF map the DetNet configuration to 5GS parameters?

Based on solutions #3, #4, #6, #7, the following mapping of the parameters are proposed in the TSCTSF.

- Max-latency to Required delay

- Min-bandwidth to GFBR

- Max-loss to Required PER (new in Rel-18)

- Max-consecutive-loss-tolerance to Survival time – when such mapping is possible, such as when there is only a single packet per interval.

- Interval to Periodicity (in TSC info)

- max-pkts-per-interval * max-payload-size to Max burst size

- max-pkts-per-interval * max-payload-size / Interval to MFBR

- DetNet flow specification to 3GPP flow description (also including the DSCP value and optionally IPv6 flow label and IPsec SPI)

The TSCTSF uses the identity of the incoming and outgoing interfaces to determine the affected PDU Session(s) and whether the flow is uplink or downlink. The TSCTSF also determines if the flow is UE to UE in which case two PDU Sessions will be affected for the flow; in that case the TSCTSF breaks up the requirements to individual requirements for the PDU Sessions. The TSCTSF provides the parameters to the PCF.

# Proposal

We propose to update 23.700-46 and add conclusions as follows.

***** START CHANGE ******

# 7	Conclusions

## 7.1	General

The following bullet points summarize the principles for the way forward:

- YANG models over Netconf or Restconf are used between the TSCTSF and the DetNet controller.

- 3GPP does not standardize any signalling mechanism to include the NEF into the signalling path between the TSCTSF and the DetNet controller. If NEF functionality is desired, the relevant functions such as the authentication, authorization and potential throttling of signalling can be achieved by including such functionality in the TSCTSF depending on the needs of the given deployment.

- The TSCTSF terminates the interface towards the DetNet controller. The TSCTSF collects and provides exposure information to the DetNet controller. The TSCTSF collects the information from the UPF/NW-TT and the SMF based on the existing procedures in Rel-17 TSC with extensions to the 5GC data models or information elements as required. The entity reporting UPF N6 interface related information to the TSCTSF is NW-TT as in Rel-17 TSC.

- The TSCTSF may use the E2E traffic requirements in the YANG configuration, and based on a pre-configured local mapping, derive sufficiently strict 5GS requirements from them. This local mapping may correspond to a “do nothing” mapping (take the incoming information from the Dt Net controller in IETF YANG model as node level information), when the Controller provides the TSCTSF with 5GS DetNet Node specific requirements using the existing IETF YANG model and a local deployment specific interpretation

Alternatively, 3GPP may define a new YANG module (e.g. 3gpp-yang) that extends the IETF DetNet YANG module which allows the DetNet controller to explicitly provide the 5GS traffic requirements.

NOTE: YANG extensions that allow the DetNet controller to explicitly provide (5GS) DetNet Node traffic requirements may be used if IETF endorses this approach early enough (if IETF answers in Q4 2022 to a LS discussing the topic)..

- It should be possible for the 5GS DetNet Node to have multiple UPF N6 side interfaces connected to a DetNet network. With such topology the same IP address or IP prefix is potentially reachable via multiple 5GS DetNet Node interfaces. To facilitate this, the UPF N6 side interfaces’ IP routing/forwarding tables are exposed from UPF (NW-TT) to the TSCTSF and from the TSCTSF to the DetNet controller.

- It should be possible for the 5GS DetNet Node to have UE side interfaces that are not only connected to a UE device acting as an IP host or DetNet end system, but also connected to a UE device acting as an IP Router or DetNet node that connects to further IP hosts or routers. This means that 5GS DetNet Node will have to be able to forward via its UE side interface IP packets destined not only to the UE’s IP address or prefix but also to other IP prefixes according to 3GPP framed routes or prefixes delegated to the UE by IPv6 prefix delegation. To facilitate this, the framed route and IPv6 prefix delegation information is exposed by the SMF to the TSCTSF and by TSCTSF to the DetNet controller. The 5GS DetNet Node has also to report its UE side interface information in such a manner that when the UE device acting as an IP router reports about its 3GPP interface, the DetNet controller can link the two interfaces as interconnected. For this IETF should be contacted to check if the issue is 3GPP specific and there would be no similar work done in IETF for the general purpose.

- As the DetNet controller learns the 5GS DetNet Node IP routing/forwarding state, it is always able to determine the 5GS DetNet Node ingress and egress interfaces for any DetNet traffic flow. In addition, as the TSCTSF has the same information it is possible to verify in the TSCTSF whether the explicit routing information provided by the DetNet controller is in line with the 5GS routing. Apart from the verification, the 5GS routing is not modified by the DetNet controller in line with the agreed scope of the work. There is no need to standardize the exposure of N6 routing information.

The following figure illustrates the DetNet architecture. The node connected to the 5GS logical DetNet Node UE side interface does not need to be a DetNet End System but could also be another DetNet Node with IP routing capabilities.

Figure 7.1-1 DetNet Architecture

## 7.2	Key Issue #1: 5GS DetNet node reporting

The 5GS is exposed by the TSCTSF to the DetNet controller as a router on a per UPF granularity. The node may be identified by a Node ID. The interfaces correspond to the PDU Sessions and to the network side interfaces. Each interface is identified by an interface identifier.

The following information may be reported from TSCTSF to DetNet controller for each UPF N6 side interface.

- Type of interface

- MAC address

- IP address,

- subnet (prefix length)

- Neighbour IP and MAC address

- MTU size

- IP forwarding table entries where this interface or one of its directly connected neighbors is the next hop.

The information is exposed to the Controller using RFC 8343, 8344 and 8345 YANG data models.

The following information may be reported from TSCTSF to DetNet controller for each UE side interface:

- Type of interface = “3GPP” (or any new type identifier allocated for this purpose)

  - Information specific to “3GPP” interface type, i.e., Role=”Network”

- For IPv4: UE IPv4 address

- For IPv6: UE interface id and IPv6 Prefix

- MTU size

- IP forwarding table entries where this interface is the next hop based on framed route and IPv6 prefix delegation information.

The information is exposed to the Controller using RFC 8343, 8344 and 8345 YANG data models with possible 3GPP defined extensions for the new “3GPP” interface type.

The TSCTSF collects the information from the UPF/NW-TT and the SMF re-using the existing procedures in Rel-17 TSC, with the addition of new parameters as needed.

## 7.3	Key Issue #2: Provisioning DetNet configuration from the DetNet controller to 5GS

The parameters are mapped in the TSCTSF as follows.

- Max-latency to Required delay

- Min-bandwidth to GFBR

- Max-loss to Required PER (new in Rel-18)

- Max-consecutive-loss-tolerance to Survival time – when such mapping is possible, such as when there is only a single packet per interval.

- Interval to Periodicity (in TSC info)

- max-pkts-per-interval * max-payload-size to Max burst size

- max-pkts-per-interval * max-payload-size / Interval to MFBR

- DetNet flow specification to 3GPP flow description (also including the DSCP value and optionally IPv6 flow label and IPsec SPI)

The TSCTSF uses the identity of the incoming and outgoing interfaces to determine the affected PDU Session(s) and whether the flow is uplink or downlink. The TSCTSF also determines if the flow is UE to UE in which case two PDU Sessions will be affected for the flow; in that case the TSCTSF breaks up the requirements to individual requirements for the PDU Sessions. The TSCTSF provides the parameters to the PCF re-using the existing procedures in Rel-17 TSC, with the addition of new parameters as needed.

The TSCTSF provides a response to the DetNet controller regarding the success of the configuration setup. Optionally, if 3GPP may defines a new YANG module that extends the IETF DetNet YANG module, the 5GS may provide 3GPP specific status codes for additional information if the requested configuration could not be set up.

If the status of the flow changes later on for any reason, the TSCTSF notifies the DetNet controller. Upon release of a PDU Session that is part of the existing DetNet configuration, the PCF notifies the TSCTSF for the PDU Session release, and TSCTSF notifies the DetNet controller on status of the flow.

***** END CHANGE ******