3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205717r06

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluation and conclusion for KI#2

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract: It is proposed to have evaluation and conclusion for KI#2.

1. Introduction/Discussion

There are 13 solutions for KI#2. This KI relates to:

- Whether and how the 5GC can be made aware whether or when the UE enforces a URSP rule to route an application traffic to a PDU Session based on the URSP rule provisioned by 5GC.

- Whether there are any actions the 5GS can take after 5GC is aware whether the UE enforces a URSP rule for specific application traffic or not. If any, what action 5GC should take?

- User privacy needs to be considered when defining solution (e.g. 5GC will know if a user started/stopped "any" application).

The details are shown in Table 1-1.

| Solutions | Title | Packet detection | Authorization by CP | URSP enforcement feedback |
| --- | --- | --- | --- | --- |
| 7 | URSP Compliance Verification |  | X |  |
| 8 | URSP Rule Precedence reporting for awareness of URSP enforcement | X |  |  |
| 9 | Per-PDU session awareness of URSP enforcement | X |  |  |
| 10 | Network based URSP rules enforcement feedback | X |  |  |
| 11 | 5GC awareness of URSP recognition by UE |  |  | X |
| 12 | URSP rule enforcement validation via gating control | X |  |  |
| 13 | URSP enforcement via PDU Session authorization/authentication when UE associating with application |  | X |  |
| 14 | 5GC awareness of URSP rule evaluation and verification of network slice usage |  | X |  |
| 15 | UE URSP enforcement validation by the network |  | X |  |
| 30 | Leveraging NWDAF to determine UEs that enforce URSP rules incorrectly | X |  |  |
| 31 | New URSP Notification Component |  |  | X |
| 32 | Application Detection using Domain Descriptor | X |  |  |

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-85.

# * * * * First change * * * *

7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

7.x	Evaluation on Key Issue #2: 5GC awareness of URSP enforcement

The solutions can be divided into three categories.

- Packet detection (#8, #9, #19, #12, #30, and #32)

Most of the solutions suggests this approach. SMF/UPF will detect the exact packets in the UP. The network will ensure the traffic of the wrong application will not get through. The UE can provide URSP rule related info to present the which URSP during PDU Session Establishment/Modification, otherwise the network will not know the related URSP rule for this PDU Session and may need to investigate all the URSP rules. Most of the solutions mention the PCF for a UE will update the URSP based on the URSP investigation result as the UE may not update the URSP in time.

- Authorization by CP (Solution #7, #13, #14 and #15)

This kind of solution let SMF authorize the application related information received in PDU Session Establishment/Modificationto make sure the application sent in NAS signalling should be correct. Solution #13, and #14 mentions that the application should be also authorized by the third party (AF or AAA server).

URSP enforcement feedback (Solution #12 and #31)

This kind of solutions assume UE cannot support some of the URSP rules. PCF will update URSP based on he UE notification  on non-supporting URSP rules..

# * * * * End of changes * * * *