3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205887r02

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	China Mobile, CATT, vivo

Title:	FS_5MBS_Ph2 KI#3, Sol#11 Update for making clarification

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract of the contribution: This paper proposes clarification to TR 23.700-47 Solution #11.

# Proposal

It is proposed to apply the changes below to solution#11 in TR 23.700-47.

*** Start of changes***

### 6.11.3	Procedures

#### 6.11.3.1	on-demand multicast MBS session management

Figure 6.11.3.1-1: On-demand multicast MBS session establishment

The key steps of the procedure for this solution are as follows:

1.	MBS Session Creation with PCC procedure is performed as specified in clause 7.1.1.3 of TS 23.247 [4], where the AF may perform a Service Announcement towards the UEs.

2.	The AF decides to set or update multicast related service parameters, including the multicast indication, for specific services/applications. The AF may, based on local configuration or triggered by e.g. event report from the NEF, subscribe to or requests network analytics information (e.g. Observed Service Experience analytics) from the NWDAF as specified in TS 23.288 [10], and decide to update the multicast related service parameters based on the analytics information.

The AF provides updated service parameters, which include the multicast indication and a MBS Session ID for the multicast MBS session to be established, to the PCF via the NEF by using Nnef_AFsessionWithQoS_Update request (as defined in clause 4.15.6.6a of TS 23.502 [3])). The "AF session with required QoS update" procedure continues as defined in TS 23.502 [3] and the PCF receives the updated service parameters.

Editor´s Note: Those procedures are meant for individual UEs and are not meant for an MBS session Are UE IDs or groups indicated? How is the PCF selected? What are the QoS Updates that justify the use of the Nnef_AFsessionWithQoS_Update procedure? Are those updates required by the MB-SMF? Why does the PCF need to be involved if there are no QoS updates? What policy would the PCF provide?

Editor’s note: PCF identifying the UE is based on UE’s private IP address. How to handle the case that only public IP address is available is to be clarified.

3.	The PCF initiates SM Policy Association Modification procedure as defined in clause 4.16.5.2 of TS 23.502 [3] to notify the SMF, with the multicast MBS session ID.

4.	Steps 2-19 specified in clause 7.2.1.3 of TS 23.247 [4] are reused to complete on-demand multicast MBS session establishment, for the UEs subscribed to the multicast MBS Session and with an associated PDU Session established for the multicast MBS Session, and Namf_Communication_N1N2MessageTransfer is used in step 5 instead of Nsmf_PDUSession_UpdateSMContext response message. The UE is informed of successfully joining the multicast MBS session upon receiving the PDU Session Modification Command which contains the MBS Session ID.

Editor´s Note: How can a UE subscribe to an MBS session? How can it be determined which UEs subscribed to the MBS session and where is that information stored? How can user consent be checked? It is ffs if and how the UE can reject the join request.

Editor’s note: The impact is to be further clarified, e.g. what information is provided by the AF to the SMF (via PCF), how does the UE know this is a request to join an MBS Session? Does the UE NAS layer need to inform the MBS client to receive multicast data? How to handle Rel-17 UEs?

### 6.11.4	Impacts on services, entities and interfaces.

AF:

-	The AF needs to support triggering the on-demand multicast MBS Session establishment, based on e.g. the network analytics information provided by the NWDAF.

PCF:

-	The PCF is enhanced to provide to the SMF(s) the multicast service related policy, i.e. MBS session ID that the AF request UE to join, for triggering the SMF(s) to complete the on-demand multicast MBS session establishment.

*** End of changes ***