3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206095r02

Elbonia, August 17 – 26, 2022

Source:	Huawei, HiSilicon

Title:	Evaluation for KI#4

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR / Rel-18

Abstract: Evaluation for the candidate solutions mapping for KI#4captured in the TR 23.700-05 V0.3.0.

# 1. Discussion

This contribution proposes the evaluation for the candidate solutions mapping for KI#4captured in the TR 23.700-05 V0.3.0.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-05 V0.3.0.

# * * * * First change * * * *

# 7	Evaluation

Editor's note:	This clause provides the evaluations of the solutions of clause 6.

# 7.x Evaluation for KI#4

There are two solutions address KI#4 Support of roaming of mobile base station relays, i.e. Soltuion#6 and Solution#10.

High level aspects of the solutions for KI#4:

- Solution#6 allows the roaming of MBSR and provides means by which the IAB-UE is configured with PLMN ID list, geographic areas and OAM server FQDN. The roaming operation of IAB-UE is authorized by the 5GC based on the Access and Mobility Subscription data. When MBSR moves into VPLMN, it makes use of the information to either perform an IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to continue provide service via VPLMN NG-RAN. The feasibility to support the roaming of MBSR as an IAB-DU using IAB node integration procedure and inter-IAB-donor gNB mobility procedure in the VPLMN needs RAN WGs evaluation.

- Solution#10 assumes the existence of a roaming agreement for the MBSR operation in VPLMN. The VPLMN 5GC authorizes the MBSR during mobility registration procedures and provides  the OAM server address and the security information required to access the OAM during the registration. However, the solution does not specify how the VPLMN 5GC determines the OAM server address and the security information. In addition, the security aspects need SA3 evaluation.

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:	This clause provides the conclusions for the study.

# * * * * End of changes * * * *