SA WG2 Meeting #S2-152E	S2-2206348r01

17 Aug. – 26 Aug. 2022, Electronic, Elbonia	(revision of S2-220xxxx)

Source:	MediaTek Inc.

Title:	KI#5, New solution: Access Localized Services via URSP rules

Document for:	Approval

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2/Rel-18

Abstract of the contribution: This paper proposes a new solution for KI#5

1	Proposal

It is proposed to adopt the following proposal into TS23.700-08.

* * * * 1st Change * * * *

6.X	Accesses Localized Services via URSP rules

6.X.1	Introduction

In this solution, it addresses how the UE access the Localized Services in the registered Hosting Network. To support the UE accesses the Localized Services using URSP, the following preconditions should be fulfilled:

-	the Route Selection Validation Criteria (i.e., Time Window and Location Criteria) of the matched URSP rules should be fulfilled

To use the PDU Session for access the Localized Services in the Hosting Network, the UE should evaluate the URSP rules. When the UE finds the Traffic Descriptor (TD) of a URSP rule is matched, the UE starts to evaluate the Route Selection Descriptor (RSD). Only when the field(s) of the Route Selection Validation Criteria in the RSD is satisfied, the UE can access the Localized Services via

-	using the existing PDU Session; or

-	establishing the PDU Session based on the matched RSD

6.X.2	Functional Description

After the UE successfully registers to the selected Hosting Network, the UE may start to access the Localized Services. Before the UE initiates the PDU Session Establishment procedure, the UE should evaluate the URSP rules.

The URSP rules could be

-	provisioned (signalled) or pre-configured by the Home Network; or

-	provisioned (signalled) by the Hosting Network

Furthermore, if the URSP rules are updated by Home Network or the Hosting Network at the time when the UE is registered to the Hosting Network, the UE can re-evaluate the URSP rules for the proper handlings for the corresponding PDU Session for the Localized Services as specified in TS 23.503 [5].

General principle is used as specified in the clause 6.6.2.2.1 of TS 23.503 [5]:

-	Only the URSP rules provisioned (signalled) by the PCF are used by the UE, if both URSP rules provisioned (signalled) by the PCF and pre-configured URSP rules are present.

Furthermore, the following principles are applied in the UE when the UE evaluates the URSP rules for the Localized Services:

-	If the Home Network is PLMN and the Hosting Network is also PLMN, the UE can only use the URSP rules provisioned (signalled) by the HPLMN of the UE.

-	If the Home Network is PLMN or SNPN and the Hosting Network is SNPN, the UE uses the URSP rules as specified in clause 6.6.2.2.2 of TS 23.503 [5] if the UE access the Hosting Network using the credentials from the Home Network as a Credentials Holder (CH).

To Support the Localized Services, the Time Window and the Location Criteria should be configured according to the Validity Information of the Localized Services.

-	The Location Criteria can contain one or more types of location area as specified in Table 5.2.2 of TS 24.526 [xx]. For the Localized Services, the Location Criteria could be TAI list which consists of the Hosting Network information. Furthermore, it needs to include NID information if the Hosting Network is an SNPN Hosting Network.

-	The Time Window can contain "Start Time" and "Stop Time" for the Localized Services as specified in Table 5.2.1 of TS 24.526 [xx].

Only when the Route Selection Validation Criteria of the matched URSP rule are satisfied, the UE can establish the new PDU Session or re-use the existing PDU Session for the Localized Services.

6.X.3	Procedures

The solution reuses existing procedures

6.X.4	Impacts on services, entities, and interfaces

H-PCF/V-PCF:

-	Support to update the URSP rules in roaming scenario as proposed in FS_eUEPO [xx]

* * * * End of Changes * * * *